CMOS SETUP 
UTILITY

AwardBIOS
Version 4.51PG

User’s Guide

Intel 82430TX PCIset

Proprietary Notice and Disclaimer

Unless otherwise noted, this document and the information herein disclosed are proprietary to Award Software International, Inc. (Award Software). Any person or entity to whom this document is furnished or who otherwise has possession thereof, by acceptance agrees that it will not be copied or reproduced in whole or in part, nor used in any manner except to meet the purposes for which it was delivered.

The information in this document is subject to change without notice, and should not be considered as a commitment by Award Software. Although Award Software will make every effort to inform users of substantive errors, Award Software disclaims all liability for any loss or damage resulting from the use of this document or any hardware or software described herein, including without limitation contingent, special, or incidental liability.

Copyright © 1998 by Award Software International Inc.  All rights reserved.  
Document date 27 March, 1998

Master document revision 1.3; August 1996

Award Software International and the Award logo are registered trademarks of Award Software International Inc.

All other products and brand names are trademarks and registered trademarks of their respective companies.

| Award Software International Inc. 777 East Middlefield Road Mountain View, CA 94043-4023 | Tel: (650) 237-6800 Fax: (650) 968-0274 www: http://www.award.com |
| --- | --- |

Table of Contents

Introduction to Setup	1

Starting Setup	1

Setup Keys	2

Getting Help	3

In Case of Problems	3

Setup Variations	3

Main Setup Menu	5

Standard CMOS Setup	7

BIOS Features Setup	11

Chipset Setup	15

Power Management	19

PM Timers	20

Reload Global Timer Events	21

PnP/PCI Configuration	23

Integrated Peripherals	27

Password Setting	31

POST Messages	33

POST Codes	37

Index of Setup Fields	41

Introduction to Setup

This manual describes the AwardBIOS Setup program.  The Setup program lets you modify basic system configuration settings.  The settings are then stored in a dedicated battery-backed memory, called CMOS RAM, that retains the information when the power is turned off.

The AwardBIOS in your computer is a customized version of an industry-standard BIOS for IBM PC AT–compatible personal computers.  It supports Intel x86 and compatible processors.  The BIOS provides critical low-level support for the system central processing, memory, and I/O subsystems.

The AwardBIOS has been customized by adding important, but nonstandard, features such as virus and password protection and power management.

The rest of this manual is intended to guide you through the process of configuring your system using Setup.

# Starting Setup

The AwardBIOS is immediately activated when you first turn on the computer.  The BIOS reads system configuration information in CMOS RAM and begins the process of checking out the system and configuring it through the power-on self test (POST).

When these preliminaries are finished, the BIOS seeks an operating system on one of the data storage devices (hard drive, floppy drive, etc.).  The BIOS launches the operating system and hands control of system operations to it.

During POST, you can start the Setup program in one of two ways:

- By pressing <Del> immediately after switching the system on, or

- By pressing the <Del> key or by simultaneously pressing <Ctrl>, <Alt>, and <Esc> keys when the following message appears briefly at the bottom of the screen during POST:

TO ENTER SETUP BEFORE BOOT PRESS CTRL-ALT-ESC OR DEL KEY

If the message disappears before you respond and you still wish to enter Setup, restart the system to try again by turning it OFF then ON or pressing the RESET button on the system case.  You may also restart by simultaneously pressing <Ctrl>, <Alt>, and <Delete> keys.  If you do not press the keys at the correct time and the system does not boot, an error message appears and you are again asked to

PRESS F1 TO CONTINUE, CTRL-ALT-ESC OR DEL TO ENTER SETUP

# Setup Keys

The following table shows to navigate in Setup using the keyboard.

| Up arrow | Move to previous item |
| --- | --- |
| Down arrow | Move to next item |
| Left arrow | Move to the item in the left hand |
| Right arrow | Move to the item in the right hand |
| Esc key | Main Menu: Quit and not save changes into CMOS RAM. Status Page Setup Menu and Option Page Setup Menu: Exit current page and return to Main Menu |
| PgUp key | Increase the numeric value or make changes |
| PgDn key | Decrease the numeric value or make changes |
| + key | Increase the numeric value or make changes |
| - key | Decrease the numeric value or make changes |
| F1 key | General help, only for Status Page Setup Menu and Option Page Setup Menu |
| F2 key Shift-F2 | Change color from total 16 colors. F2 to select color forward, Shift-F2 to select color backward |
| F3 key | Calendar, only for Status Page Setup Menu |
| F4 key | Reserved |
| F5 key | Restore the previous CMOS value from CMOS, only for Option Page Setup Menu |
| F6 key | Load the default CMOS RAM value from BIOS default table, only for Option Page Setup Menu |
| F7 key | Load the default |
| F8 key | Reserved |
| F9 key | Reserved |
| F10 key | Save all the CMOS changes, only for Main Menu |

# Getting Help

Press F1 to pop up a small help window that describes the appropriate keys to use and the possible selections for the highlighted item.  To exit the Help Window press Esc or the F1 key again.

# In Case of Problems

If, after making and saving system changes with Setup, you discover that your computer no longer is able to boot, the AwardBIOS supports an override to the CMOS settings that resets your system to its default configuration.

You can invoke this override by immediately pressing <Insert> when you restart your computer.  You can restart by either using the ON/OFF switch, the RESET button or by pressing <Ctrl>, <Alt> and <Delete> at the same time.

The best advice is to alter only settings that you thoroughly understand.

# Setup Variations

Not all systems have the same Setup.  While the basic look and function of the Setup program remains the same for all systems, the appearance of your Setup screens may differ from the screens shown here.  Each system design and chipset combination require custom configurations.  In addition, the final appearance of the Setup program depends on your system designer.  Your system designer can decide that certain items should not be available for user configuration and remove them from the Setup program.

This page intentionally blank.

Main Setup Menu

When you enter the AwardBIOS CMOS Setup Utility, a Main Menu, similar to the one shown below, appears on the screen.  The Main Menu allows you to select from several Setup functions and two exit choices.  Use the arrow keys to select among the items and press ¿ to accept and enter the sub-menu.

A brief description of each highlighted selection appears at the bottom of the screen.  Following is a brief summary of each Setup category.

Options in the original PC AT-compatible BIOS.

## Standard CMOS Setup

Award enhanced BIOS options.

## BIOS Features Setup

Options specific to your system chipset.

## Chipset Features Setup

Advanced Power Management (APM) options.

## Power Management Setup

Plug and Play standard and PCI Local Bus configuration options.

## PnP/PCI Configuration

I/O subsystems that depend on the integrated peripherals controller in your system.

## Integrated Peripherals

Change, set, or disable a password.  Only the supervisor password permits access to Setup.  The user password generally allows only power-on access.

## Supervisor/User Password

Automatically detect and configure IDE hard disk parameters.

## IDE HDD Auto Detection

Many BIOS versions omit this option.  Most manufacturers of IDE hard drives strongly recommend that you do not run a low-level format on their drives, because of the danger that the bad-track table may be over-written.  Award supplies this utility for service personnel only.  If you feel that you need to run a low-level format on your hard drive, contact your drive manufacturer for instructions!

## HDD Low Level Format

BIOS defaults are factory settings for the most stable, minimal-performance system operations.

## Load BIOS Defaults

Setup defaults are factory settings for optimal-performance system operations.

## Load Setup Defaults

Save settings in nonvolatile CMOS RAM and exit Setup.

## Save & Exit Setup

Abandon all changes and exit Setup.

## Exit Without Save

Standard CMOS Setup

In the Standard CMOS menu you can set the system clock and calendar, record disk drive parameters and the video subsystem type, and select the type of errors that stop the BIOS POST.

The BIOS determines the day of the week from the other date information.  This field is for information only.

## Date

Press the ® or ¬ key to move to the desired field (date, month, year).  Press the PgUp or PgDn key to increment the setting, or type the desired value into the field.

The time format is based on the 24-hour military-time clock.  For example, 1 p.m. is 13:00:00. Press the ® or ¬ key to move to the desired field .  Press the PgUp or PgDn key to increment the setting, or type the desired value into the field.

## Time

This category may not be present in your Setup program.  When enabled, it adds one hour to the clock when daylight-saving time begins.  It also subtracts one hour when standard time returns.

## Daylight Saving

The BIOS supports up to four IDE drives.  This section does not show information about other IDE devices, such as a CD-ROM drive, or about other hard drive types, such as SCSI drives.

## HARD DISKS

NOTE:	We recommend that you select type auto for all drives.

The BIOS can automatically detect the specifications and optimal operating mode of almost all IDE hard drives.  When you select type auto for a hard drive, the BIOS detects its specifications during POST, every time the system boots.

If you do not want to select drive type auto, other methods of selecting the drive type are available:

- Match the specifications of your installed IDE hard drive(s) with the preprogrammed values for drive types 1 through 45.

- Select user and enter values into each drive parameter field.

- Use the ide hdd auto dectection function in Setup.

Here is a brief explanation of drive specifications:

- Type:  The BIOS contains a table of pre-defined drive types.  Each defined drive type has a specified number of cylinders, number of heads, write precompensation factor, landing zone, and number of sectors.  Drives whose specifications do not accommodate any pre-defined type are classified as type user.

- Size:  Disk drive capacity (approximate).  Note that this size is usually slightly greater than the size of a formatted disk given by a disk-checking program.

- Cyls:  Number of cylinders

- Head:  Number of heads

- Precomp:  Write precompensation cylinder

- Landz:  Landing zone

- Sector:  Number of sectors

- Mode:  Auto, Normal, Large, or LBA

- Auto:  The BIOS automatically determines the optimal mode.

- Normal:  Maximum number of cylinders, heads, and sectors supported are 1024, 16, and 63.

- Large:  For drives that do not support LBA and have more than 1024 cylinders.

- LBA (Logical Block Addressing):  During drive accesses, the IDE controller transforms the data address described by sector, head, and cylinder number into a physical block address, significantly improving data transfer rates.  For drives with greater than 1024 cylinders.

Select the correct specifications for the diskette drive(s) installed in the computer.

## Drive A 
Drive B

| None | No diskette drive installed |
| --- | --- |
| 360K, 5.25 in | 5-1/4 inch PC-type standard drive; 360 kilobyte capacity |
| 1.2M, 5.25 in | 5-1/4 inch AT-type high-density drive; 1.2 megabyte capacity |
| 720K, 3.5 in | 3-1/2 inch double-sided drive; 720 kilobyte capacity |
| 1.44M, 3.5 in | 3-1/2 inch double-sided drive; 1.44 megabyte capacity |
| 2.88M, 3.5 in | 3-1/2 inch double-sided drive; 2.88 megabyte capacity |

Select the type of primary video subsystem in your computer.  The BIOS usually detects the correct video type automatically.  The BIOS supports a secondary video subsystem, but you do not select it in Setup.

## Video

| EGA/VGA | Enhanced Graphics Adapter/Video Graphics Array. For EGA, VGA, SEGA, SVGA or PGA monitor adapters. |
| --- | --- |
| CGA 40 | Color Graphics Adapter, power up in 40 column mode |
| CGA 80 | Color Graphics Adapter, power up in 80 column mode |
| MONO | Monochrome adapter, includes high resolution monochrome adapters |

During the power-on self-test (POST), the computer stops if the BIOS detects a hardware error.  You can tell the BIOS to ignore certain errors during POST and continue the boot-up process.  These are the selections:

## Halt On

| No Errors | POST does not stop for any errors. |
| --- | --- |
| All Errors | If the BIOS detects any non-fatal error, POST stops and prompts you to take corrective action. |
| All, But Keyboard | POST does not stop for a keyboard error, but stops for all other errors. |
| All, But Diskette | POST does not stop for diskette drive errors, but stops for all other errors. |
| All, But Disk/Key | POST does not stop for a keyboard or disk error, but stops for all other errors. |

You cannot change any values in the Memory fields; they are only for your information.  The fields show the total installed random access memory (RAM) and amounts allocated to base memory, extended memory, and other (high) memory.  RAM is counted in kilobytes (KB:  approximately one thousand bytes) and megabytes (MB:  approximately one million bytes).

## Memory

RAM is the computer's working memory, where the computer stores programs and data currently being used, so they are accessible to the CPU.  Modern personal computers may contain up to 64 MB, 128 MB, or more.

- Base Memory

Typically 640 KB.  Also called conventional memory.  The DOS operating system and conventional applications use this area.

- Extended Memory

Above the 1-MB boundary.  Early IBM personal computers could not use memory above 1 MB, but current PCs and their software can use extended memory.

- Other Memory

Between 640  KB and 1 MB; often called High memory.  DOS may load terminate-and-stay-resident (TSR) programs, such as device drivers, in this area, to free as much conventional memory as possible for applications.  Lines in your config.sys file that start with loadhigh load programs into high memory.

BIOS Features Setup

This screen contains industry-standard options additional to the core PC AT BIOS.  This section describes all fields offered by Award Software in this screen.  The example screen below may vary from the one in your Setup program.  Your system board designer may omit or modify some fields.

When Enabled, you receive a warning message if a program (specifically, a virus) attempts to write to the boot sector or the partition table of the hard disk drive.  You should then run an anti-virus program.  Keep in mind that this feature protects only the boot sector, not the entire hard drive.

## Virus Warning

NOTE:	Many disk diagnostic programs that access the boot sector table can trigger the virus warning message.  If you plan to run such a program, we recommend that you first disable the virus warning.

Cache memory is additional memory that is much faster than conventional DRAM (system memory).  CPUs from 486-type on up contain internal cache memory, and most, but not all, modern PCs have additional (external) cache memory.  When the CPU requests data, the system transfers the requested data from the main DRAM into cache memory, for even faster access by the CPU.

## CPU Internal Cache /External Cache

The External Cache field may not appear if your system does not have external cache memory.

Select Enabled to reduce the amount of time required to run the power-on self-test (POST).  A quick POST skips certain steps.  We recommend that you normally disable quick POST.  Better to find a problem during POST than lose data during your work.

## Quick Power On Self Test

The original IBM PCs loaded the operating system from drive A (floppy disk), so IBM PC-compatible systems are designed to search for an operating system first on drive A, and then on drive C (hard disk).  However, the BIOS now offers different 10 boot sequence options of three drives each.  In addition to the traditional drives A and C, options include IDE hard drives D, E, and F; plus a SCSI hard drive and a CD-ROM drive.

## Boot Sequence

This field is effective only in systems with two floppy drives.  Selecting Enabled assigns physical drive B to logical drive A, and physical drive A to logical drive B.

## Swap Floppy Drive

When Enabled, the BIOS tests (seeks) floppy drives to determine whether they have 40 or 80 tracks.  Only 360-KB floppy drives have 40 tracks; drives with 720 KB, 1.2 MB, and 1.44 MB capacity all have 80 tracks.  Because very few modern PCs have 40-track floppy drives, we recommend that you set this field to Disabled to save time.

## Boot Up Floppy Seek

Toggle between On or Off to control the state of the NumLock key when the system boots.  When toggled On, the numeric keypad generates numbers instead of controlling cursor operations.

## Boot Up NumLock Status

Select High to boot at the default CPU speed; select Low to boot at the speed of the AT bus.  Some add-in peripherals or old software (such as old games) may require a slow CPU speed.  The default setting is High.

## Boot Up System Speed

When Disabled, the following two items (Typematic Rate and Typematic Delay) are irrelevant.  Keystrokes repeat at a rate determined by the keyboard controller in your system.

## Typematic Rate Setting

When Enabled, you can select a typematic rate and typematic delay.

When the typematic rate setting is enabled, you can select a typematic rate (the rate at which character repeats when you hold down a key) of 6, 8, 10,12, 15, 20, 24 or 30 characters per second.

## Typematic Rate (Chars/Sec)

When the typematic rate setting is enabled, you can select a typematic delay (the delay before key strokes begin to repeat) of 250, 500, 750 or 1000 milliseconds.

## Typematic Delay (Msec)

If you have set a password, select whether the password is required every time the System boots, or only when you enter Setup.

## Security Option

Your BIOS Setup many not contain this field.  If the field is present, leave at Disabled.

## PCI/VGA Palette Snoop

Select OS2 only if you are running OS/2 operating system with greater than 64 MB of RAM on your system.

## OS Select for DRAM > 64MB

Software that resides in a read-only memory (ROM) chip on a device is called firmware.  The AwardBIOS permits shadowing of firmware such as the system BIOS, video BIOS, and similar operating instructions that come with some expansion peripherals, such as, for example, a SCSI adaptor.

## Shadow

Shadowing copies firmware from ROM into system RAM, where the CPU can read it through the 16-bit or 32-bit DRAM bus.  Firmware not shadowed must be read by the system through the 8-bit X-bus.  Shadowing improves the performance of the system BIOS and similar ROM firmware for expansion peripherals, but it also reduces the amount of high memory (640 KB to 1 MB) available for loading device drivers, etc.

Enable shadowing into each section of memory separately.  Many system designers hardwire shadowing of the system BIOS and eliminate a System BIOS Shadow option.

Video BIOS shadows into memory area C0000-C7FFF.  The remaining areas shown on the BIOS Features Setup screen may be occupied by other expansion card firmware.  If an expansion peripheral in your system contains ROM-based firmware, you need to know the address range the ROM occupies to shadow it into the correct area of RAM.

Chipset Features Setup

This section describes features of the Intel 82430TX PCIset.  If your system contains a different chipset, this section will bear little resemblance to what you see on your screen.

advanced options.  The parameters in this screen are for system designers, service personnel, and technically competent users only.  Do not reset these values unless you understand the consequences of your changes.

NOTE:	This chapter describes all fields offered by Award Software in this screen.  Your system board designer may omit or modify some fields.

Auto Configuration selects predetermined optimal values of chipset parameters.  When Disabled, chipset parameters revert to setup information stored in CMOS.  Many fields in this screen are not available when Auto Configuration is Enabled.

## Auto Configuration

The value in this field depends on performance parameters of the installed memory chips (DRAM).  Do not change the value from the factory setting unless you install new memory that has a different performance rating than the original DRAMs.

## DRAM Timing

Select the combination of CPU clocks the DRAM on your board requires before each read from or write to the memory.  Changing the value from the setting determined by the board designer for the installed DRAM may cause memory errors.

## DRAM Leadoff Timing

Sets the timing for reads from EDO (Extended Data Output) or FP[M] (Fast Page Mode) memory.  The lower the timing numbers, the faster the system addresses memory.  Selecting timing numbers lower than the installed DRAM is able to support can result in memory errors.

## DRAM Read Burst (EDO/FP)

Sets the timing for writes to memory.  The lower the timing numbers, the faster the system addresses memory.  Selecting timing numbers lower than the installed DRAM is able to support can result in memory errors.

## DRAM Write Burst Timing

Select Enabled only for EDO DRAMs in either a synchronous cache or a cacheless system.  It causes a 1-HCLK pull-in for all read leadoff latencies for EDO DRAMs (i.e., page hits, page misses, and row misses).  Select Disabled if any of the DRAM rows are populated with FPM DRAMs.

## Fast EDO Leadoff

Select the number of clock cycles in which RAS# is asserted for refresh cycles.

## Refresh RAS# Assertion

This field lets you insert a timing delay between the CAS and RAS strobe signals, used when DRAM is written to, read from, or refreshed.

## Fast RAS# to CAS# Delay

Select the amount of time in HCLKs that the DRAM controller waits to close a DRAM page after the CPU becomes idle.

## DRAM Page Idle Timer

When Enabled, the chipset keeps the page open until a page/row miss.  When Disabled, the chipset uses additional information to keep the DRAM page open when the host may be “right back.”

## DRAM Enhanced Paging

The values in this field are set by the system board designer, depending on the DRAM installed.  Do not change the values in this field unless you change specifications of the installed DRAM or the installed CPU.

## Fast MA to RAS# Delay

You can select a combination of CAS latency and RAS-to-CAS delay in HCLKs of 2/2 or 3/3.  The system board designer should set the values in this field, depending on the DRAM installed.  Do not change the values in this field unless you change specifications of the installed DRAM or the installed CPU.

## SDRAM (CAS Lat/RAS-to-CAS)

The chipset can "speculate" on a DRAM read address, thus reducing read latencies.  The CPU issues a read request containing the data memory address. The DRAM controller receives the request.  When this field is Enabled, the controller issues the read command slightly before it has finished decoding the data address.

## SDRAM Speculative Read

Selecting Enabled allows caching of the system BIOS ROM at F0000h-FFFFFh, resulting in better system performance.  However, if any program writes to this memory area, a system error may result.

## System BIOS Cacheable

Selecting Enabled allows caching of the video BIOS ROM at C0000h to C7FFFh, resulting in better video performance.  However, if any program writes to this memory area, a system error may result.

## Video BIOS Cacheable

The I/O recovery mechanism adds bus clock cycles between PCI-originated I/O cycles to the ISA bus.  This delay takes place because the PCI bus is so much faster than the ISA bus.

## 8/16 Bit I/O Recovery Time

These two fields let you add recovery time (in bus clock cycles) for 16-bit and 8-bit I/O.

You can reserve this area of system memory for ISA adapter ROM.  When this area is reserved, it cannot be cached. The user information of peripherals that need to use this area of system memory usually discusses their memory requirements.

## Memory Hole at 15M-16M

When Enabled, CPU to PCI bus accesses are allowed during passive release.  Otherwise, the arbiter only accepts another PCI master access to local DRAM.

## PCI Passive Release

The chipset has an embedded 32-bit posted write buffer to support delay transactions cycles.  Select Enabled to support compliance with PCI specification version 2.1.

## PCI Delayed Transaction

Select Faster if your system contains one bank of pipelined burst SRAM.  Select Fastest if your system contains two banks of pipelined burst SRAM.

## Pipeline Cache Timing

When Disabled, NA# assertion is dependent on the cache type and size.  When Enabled, the NA# pin never asserted.  Selecting Enabled permits pipelining, in which the chipset signals the CPU for a new memory address before all data transfers for the current cycle are complete, resulting in faster performance.

## Chipset NA# Asserted

(Memory Address Drive Strength)  This field controls the strength of the output buffers driving the MA and BA1 pins (first value) and SRASx#, SCASx#, MWEx#, and CKEx pins (second value).

## Mem. Drive Str. (MA/RAS)

Select the period required to refresh the DRAMs, according to DRAM specifications.

## DRAM Refresh Rate

Power Management

NOTE:	This chapter describes all fields offered by Award Software in this screen.  Your system board designer may omit or modify some fields.

This option allows you to select the type (or degree) of power saving for Doze, Standby, and Suspend modes.  See the section PM Timers for a brief description of each mode.

## Power Management

This table describes each power management mode:

| Max Saving | Maximum power savings. Only Available for SL CPUs. Inactivity period is 1 minute in each mode. |
| --- | --- |
| User Define | Set each mode individually. Select time-out periods in the PM Timers section, following. |
| Min Saving | Minimum power savings. Inactivity period is 1 hour in each mode (except the hard drive). |

If Advanced Power Management (APM) is installed on your system, selecting Yes gives better power savings.

## PM Control by APM

Determines the manner in which the monitor is blanked.

## Video Off Method

| V/H SYNC+Blank | System turns off vertical and horizontal synchronization ports and writes blanks to the video buffer. |
| --- | --- |
| DPMS Support | Select this option if your monitor supports the Display Power Management Signaling (DPMS) standard of the Video Electronics Standards Association (VESA). Use the software supplied for your video subsystem to select video power management values. |
| Blank Screen | System only writes blanks to the video buffer. |

As the system moves from lesser to greater power-saving modes, select the mode in which you want the monitor to blank.

## Video Off After

| # PM Timers | # The following modes are Green PC power saving functions. They are user-configurable only during User Defined Power Management mode. |
| --- | --- |

After the selected period of system inactivity (1 minute to 1 hour), the CPU clock runs at slower speed while all other devices still operate at full speed.

## Doze Mode

After the selected period of system inactivity (1 minute to 1 hour), the fixed disk drive and the video shut off while all other devices still operate at full speed.

## Standby Mode

After the selected period of system inactivity (1 minute to 1 hour), all devices except the CPU shut off.

## Suspend Mode

After the selected period of drive inactivity (1 to 15 minutes), the hard disk drive powers down while all other devices remain active.

## HDD Power Down

When the system enters Doze mode, the CPU clock runs only part of the time.  You may select the percent of time that the clock runs.

## Throttle Duty Cycle

When Enabled, the ZZ signal is active during Suspend mode.

## ZZ Active in Suspend

When Enabled, any video activity restarts the global timer for Standby mode.

## VGA Active Monitor

When Enabled, the CPU fan turns off during Suspend mode.

## CPUFAN Off in Suspend

You can turn On or Off monitoring of IRQ8 (the Real Time Clock) so it does not awaken the system from Suspend mode.

## IRQ8 Clock Event..

| # Reload Global Timer Events |  | # When Enabled, an event occurring on each device listed below restarts the global timer for Standby mode. |  |  |
| --- | --- | --- | --- | --- |
|  | IRQ3-7, 9-15, NMI Primary IDE 0 Primary IDE 1 Secondary IDE 0 Secondary IDE 1 |  | Floppy Disk Serial Port Parallel Port |  |

This page intentionally blank.

PnP/PCI Configuration

NOTE:	This chapter describes all fields offered by Award Software in this screen.  Your system board designer may omit or modify some fields.

Select Yes if the system operating environment is Plug-and-Play aware (e.g., Windows 95).

## PNP OS Installed

The Award Plug and Play BIOS can automatically configure all the boot and Plug and Play-compatible devices.  If you select Auto, all the interrupt request (IRQ) and DMA assignment and memory base address fields disappear, because the BIOS automatically assigns them.

## Resources Controlled By

Normally, you leave this field Disabled.  Select Enabled to reset Extended System Configuration Data (ESCD) when you exit Setup if you have installed a new add-on and the system reconfiguration has caused such a serious conflict that the operating system cannot boot.

## Reset Configuration Data

When resources are controlled manually, assign each system interrupt as one of the following types, depending on the type of device using the interrupt:

## IRQ n Assigned to

Legacy ISA 	Devices compliant with the original PC AT bus specification, requiring a specific interrupt (such as IRQ4 for serial port 1).

PCI/ISA PnP	Devices compliant with the Plug and Play standard, whether designed for PCI or ISA bus architecture.

When resources are controlled manually, assign each system DMA channel as one of the following types, depending on the type of device using the interrupt:

## DMA n Assigned to

Legacy ISA 	Devices compliant with the original PC AT bus specification, requiring a specific DMA channel

PCI/ISA PnP	Devices compliant with the Plug and Play standard, whether designed for PCI or ISA bus architecture.

Leave the IRQ trigger set at Level unless the PCI device assigned to the interrupt specifies Edge-triggered interrupts.

## PCI IRQ Activated by

This field lets you select PCI IDE IRQ mapping or PC AT (ISA) interrupts.  If your system does not have one or two PCI IDE connectors on the system board, select values according to the type of IDE interface(s) installed in your system (PCI or ISA).  Standard ISA interrupts for IDE channels are IRQ14 for primary and IRQ15 for secondary.

## PCI IDE IRQ Map to

Each PCI peripheral connection is capable of activating up to four interrupts: INT# A, INT# B, INT# C and INT# D.  By default, a PCI connection is assigned INT# A.  Assigning INT# B has no meaning unless the peripheral device requires two interrupt services rather than just one.  Because the PCI IDE interface in the chipset has two channels, it requires two interrupt services.  The primary and secondary IDE INT# fields default to values appropriate for two PCI IDE channels, with the primary PCI IDE channel having a lower interrupt than the secondary.

## Primary/Secondary IDE INT#

Select a base address for the memory area used by any peripheral that requires high memory.

## Used Mem base addr

Select a length for the memory area specified in the previous field.  This field does not appear if no base address is specified.

## Used Mem Length

This page intentionally blank.

Integrated Peripherals

NOTE:	This chapter describes all fields offered by Award Software in this screen.  Your system board designer may omit or modify some fields.

Block mode is also called block transfer, multiple commands, or multiple sector read/write.  If your IDE hard drive supports block mode (most new drives do), select Enabled for automatic detection of the optimal number of block read/writes per sector the drive can support.

## IDE HDD Block Mode

The four IDE PIO (Programmed Input/Output) fields let you set a PIO mode (0-4) for each of the four IDE devices that the onboard IDE interface supports. Modes 0 through 4 provide successively increased performance.  In Auto mode, the system automatically determines the best mode for each device.

## IDE Primary/Secondary Master/Slave PIO

Ultra DMA/33 implementation is possible only if your IDE hard drive supports it and the operating environment includes a DMA driver (Windows 95 OSR2 or a third-party IDE bus master driver).  If your hard drive and your system software both support Ultra DMA/33, select Auto to enable BIOS support.

## IDE Primary/Secondary Master/Slave UDMA

The integrated peripheral controller contains an IDE interface with support for two IDE channels.  Select Enabled to activate each channel separately.

## On-Chip Primary/Secondary PCI IDE

Select Enabled if your system contains a Universal Serial Bus (USB) controller and you have a USB keyboard.

## USB Keyboard Support

Select the minimum amount of time, in PCI clock cycles, that the USB controller can retain ownership of the PCI bus.

## USB Latency Time (PCI CLK)

Select Enabled if your system has a floppy disk controller (FDC) installed on the system board and you wish to use it.  If you install an add-in FDC or the system has no floppy drive, select Disabled in this field.

## Onboard FDC Controller

Select an address and corresponding interrupt for the first and second serial ports.

## Onboard UART 1/2

The second serial port offers these InfraRed interface modes:

## Onboard UART 2 Mode

HPSIR	IrDA-compliant serial infrared port
ASKIR	Amplitude shift keyed infrared port

Select the value required by the IR device connected to the IR port. Full-duplex mode permits simultaneous two-direction transmission.  Half-duplex mode permits transmission in one direction only at a time.  If no infrared port is present in the system, select Disabled.

## IR Duplex Mode

Consult your IR peripheral documentation to select the correct setting of the TxD and RxD signals.

## Use IR Pins

Select a logical LPT port name and matching address for the physical parallel (printer) port.

## Onboard Parallel Port

Select an operating mode for the onboard parallel (printer) port.  Select Normal unless your hardware and software require one of the other modes offered in this field.

## Parallel Port Mode

Select a DMA channel for the port.

## ECP Mode Use DMA

Select EPP port type 1.7 or 1.9.

## Parallel Port EPP Type

This page intentionally blank.

Password Setting

When you select

this function, a message appears at the center of the screen:

enter password:

Type the password, up to eight characters, and press ¿.  Typing a password clears any previously entered password from CMOS memory.

Now the message changes:

confirm password:

Again, type the password and press ¿.

To abort the process at any time, press Esc.

In the Security Option item in the BIOS Features Setup screen, select System or Setup:

| System | Enter a password each time the system boots and when ever you enter Setup. |
| --- | --- |
| Setup | Enter a password when ever you enter Setup. |

NOTE:	To clear the password, simply press ¿ when asked to enter a password.  Then the password function is disabled.

This page intentionally blank.

POST Messages

During the power-on self test (POST), the BIOS either sounds a beep code or displays a message when it detects a correctable error.

Following is a list of POST messages for the ISA BIOS kernel.  Specific chipset ports and BIOS extensions may include additional messages.  An error message may be followed by a prompt to press F1 to continue or press DEL to enter Setup.

### Beep

Currently the only beep code indicates that a video error has occurred and the BIOS cannot initialize the video screen to display any additional information.  This beep code consists of a single long beep followed by two short beeps.

### BIOS ROM checksum error - System halted

The checksum of the BIOS code in the BIOS chip is incorrect, indicating the BIOS code may have become corrupt.  Contact your system dealer to replace the BIOS.

### CMOS battery failed

CMOS battery is no longer functional.  Contact your system dealer for a replacement battery.

### CMOS checksum error - Defaults loaded

Checksum of CMOS is incorrect, so the system loads the default equipment configuration.  A checksum error may indicate that CMOS has become corrupt.  This error may have been caused by a weak battery.   Check the battery and replace if necessary.

### CPU at nnnn

Displays the running speed of the CPU.

### Display switch is set incorrectly.

The display switch on the motherboard can be set to either monochrome or color.  This message indicates the switch  is set to a different setting than indicated in Setup.  Determine which setting is correct, and then either turn off the system and change the jumper, or enter Setup and change the VIDEO selection.

### Press ESC to skip memory test

The user may press Esc to skip the full memory test.

### Floppy disk(s) fail

Cannot find or initialize the floppy drive controller or the drive.  Make sure the controller is installed correctly.  If no floppy drives are installed, be sure the Diskette Drive selection in Setup is set to NONE or AUTO.

### HARD DISK initializing  
Please wait a moment...

Some hard drives require extra time to initialize.

### HARD DISK INSTALL FAILURE

Cannot find or initialize the hard drive controller or the drive.  Make sure the controller is installed correctly.  If no hard drives are installed, be sure the Hard Drive selection in Setup is set to NONE.

### Hard disk(s) diagnosis fail

The system may run specific disk diagnostic routines.  This message appears if one or more hard disks return an error when the diagnostics run.

### Keyboard error or no keyboard present

Cannot initialize the keyboard.  Make sure the keyboard is attached correctly and no keys are pressed during POST.  To purposely configure the system without a keyboard, set the error halt condition in Setup to HALT ON ALL, BUT KEYBOARD.  The BIOS then ignores the missing keyboard during POST.

### Keyboard is locked out - Unlock the key

This message usually indicates that one or more keys have been pressed during the keyboard tests.  Be sure no objects are resting on the keyboard.

### Memory Test :

This message displays during a full memory test, counting down the memory areas being tested.

### Memory test fail

If POST detects an error during memory testing, additional information appears giving specifics about the type and location of the memory error.

### Override enabled - Defaults loaded

If the system cannot boot using the current CMOS configuration, the BIOS can override the current configuration is a set of BIOS defaults designed for the most stable, minimal-performance system operations.

### Press TAB to show POST screen

System OEMs may replace the AwardBIOS POST display with their own proprietary display.  Including this message in the OEM display permits the operator to switch between the OEM display and the default POST display.

### Primary master hard disk fail

POST detects an error in the primary master IDE hard drive.

### Primary slave hard disk fail

POST detects an error in the secondary master IDE hard drive.

### Resuming from disk, Press TAB to show POST screen

Award offers a save-to-disk feature for notebook computers.  This message may appear when the operator re-starts the system after a save-to-disk shut-down.  See the Press TAB ... message above for a description of this feature.

### Secondary master hard disk fail

POST detects an error in the primary slave IDE hard drive.

### Secondary slave hard disk fail

POST detects an error in the secondary slave IDE hard drive.

This page intentionally blank.

POST Codes

NOTE:	EISA POST codes are typically output to port address 300h.  
ISA POST codes are output to port address 80h.

| Code (hex) | Name | Description |
| --- | --- | --- |
| C0 | Turn Off Chipset Cache | OEM Specific-Cache control |
| 1 | Processor Test 1 | Processor Status (1FLAGS) Verification. Tests the following processor status flags: carry, zero, sign, overflow, The BIOS sets each flag, verifies they are set, then turns each flag off and verifies it is off. |
| 2 | Processor Test 2 | Read/Write/Verify all CPU registers except SS, SP, and BP with data pattern FF and 00. |
| 3 | Initialize Chips | Disable NMI, PIE, AIE, UEI, SQWV Disable video, parity checking, DMA Reset math coprocessor Clear all page registers, CMOS shutdown byte Initialize timer 0, 1, and 2, including set EISA timer to a known state Initialize DMA controllers 0 and 1 Initialize interrupt controllers 0 and 1 Initialize EISA extended registers. |
| 4 | Test Memory Refresh Toggle | RAM must be periodically refreshed to keep the memory from decaying. This function assures that the memory refresh function is working properly. |
| 5 | Blank video, Initialize keyboard | Keyboard controller initialization. |
| 6 | Reserved |  |
| 7 | Test CMOS Interface and Battery Status | Verifies CMOS is working correctly, detects bad battery. |
| BE | Chipset Default Initialization | Program chipset registers with power on BIOS defaults. |
| C1 | Memory presence test | OEM Specific-Test to size on-board memory |
| C5 | Early Shadow | OEM Specific-Early Shadow enable for fast boot. |
| C6 | Cache presence test | External cache size detection |
| 8 | Setup low memory | Early chip set initialization Memory presence test OEM chip set routines Clear low 64K of memory Test first 64K memory. |
| 9 | Early Cache Initialization | Cyrix CPU initialization Cache initialization |
| A | Setup Interrupt Vector Table | Initialize first 120 interrupt vectors with SPURIOUS_INT_HDLR and initialize INT 00h-1Fh according to INT_TBL |
| B | Test CMOS RAM Checksum | Test CMOS RAM Checksum, if bad, or insert key pressed, load defaults. |
| C | Initialize keyboard | Detect type of keyboard controller (optional) Set NUM_LOCK status. |
| D | Initialize Video Interface | Detect CPU clock. Read CMOS location 14h to find out type of video in use. Detect and Initialize Video Adapter. |
| E | Test Video Memory | Test video memory, write sign-on message to screen. Setup shadow RAM - Enable shadow according to Setup. |
| F | Test DMA Controller 0 | BIOS checksum test. Keyboard detect and initialization |
| 10 | Test DMA Controller 1 |  |
| 11 | Test DMA Page Registers | Test DMA Page Registers. |
| 12-13 | Reserved |  |
| 14 | Test Timer Counter 2 | Test 8254 Timer 0 Counter 2. |
| 15 | Test 8259-1 Mask Bits | Verify 8259 Channel 1 masked interrupts by alternately turning off and on the interrupt lines. |
| 16 | Test 8259-2 Mask Bits | Verify 8259 Channel 2 masked interrupts by alternately turning off and on the interrupt lines. |
| 17 | Test Stuck 8259's Interrupt Bits | Turn off interrupts then verify no interrupt mask register is on. |
| 18 | Test 8259 Interrupt Functionality | Force an interrupt and verify the interrupt occurred. |
| 19 | Test Stuck NMI Bits (Parity/IO Check) | Verify NMI can be cleared. |
| 1A |  | Display CPU clock |
| 1B-1E | Reserved |  |
| 1F | Set EISA Mode | If EISA non-volatile memory checksum is good, execute EISA initialization. If not, execute ISA tests an clear EISA mode flag. Test EISA Configuration Memory Integrity (checksum & communication interface). |
| 20 | Enable Slot 0 | Initialize slot 0 (System Board). |
| 21-2F | Enable Slots 1-15 | Initialize slots 1 through 15. |
| 30 | Size Base and Extended Memory | Size base memory from 256K to 640K and extended memory above 1MB. |
| 31 | Test Base and Extended Memory | Test base memory from 256K to 640K and extended memory above 1MB using various patterns. NOTE: This test is skipped in EISA mode and can be skipped with ESC key in ISA mode. |
| 32 | Test EISA Extended Memory | If EISA Mode flag is set then test EISA memory found in slots initialization. NOTE: This test is skipped in ISA mode and can be skipped with ESC key in EISA mode. |
| 33-3B | Reserved |  |
| 3C | Setup Enabled |  |
| 3D | Initialize & Install Mouse | Detect if mouse is present, initialize mouse, install interrupt vectors. |
| 3E | Setup Cache Controller | Initialize cache controller. |
| 3F | Reserved |  |
| BF | Chipset Initialization | Program chipset registers with Setup values |
| 40 |  | Display virus protect disable or enable |
| 41 | Initialize Floppy Drive & Controller | Initialize floppy disk drive controller and any drives. |
| 42 | Initialize Hard Drive & Controller | Initialize hard drive controller and any drives. |
| 43 | Detect & Initialize Serial/Parallel Ports | Initialize any serial and parallel ports (also game port). |
| 44 | Reserved |  |
| 45 | Detect & Initialize Math Coprocessor | Initialize math coprocessor. |
| 46 | Reserved |  |
| 47 | Reserved |  |
| 48-4D | Reserved |  |
| 4E | Manufacturing POST Loop or Display Messages | Reboot if Manufacturing POST Loop pin is set. Otherwise display any messages (i.e., any non-fatal errors that were detected during POST) and enter Setup. |
| 4F | Security Check | Ask password security (optional). |
| 50 | Write CMOS | Write all CMOS values back to RAM and clear screen. |
| 51 | Pre-boot Enable | Enable parity checker Enable NMI, Enable cache before boot. |
| 52 | Initialize Option ROMs | Initialize any option ROMs present from C8000h to EFFFFh. NOTE: When FSCAN option is enabled, ROMs initialize from C8000h to F7FFFh. |
| 53 | Initialize Time Value | Initialize time value in 40h: BIOS area. |
| 60 | Setup Virus Protect | Setup virus protect according to Setup |
| 61 | Set Boot Speed | Set system speed for boot |
| 62 | Setup NumLock | Setup NumLock status according to Setup |
| 63 | Boot Attempt | Set low stack Boot via INT 19h. |
| B0 | Spurious | If interrupt occurs in protected mode. |
| B1 | Unclaimed NMI | If unmasked NMI occurs, display Press F1 to disable NMI, F2 reboot. |
| E1-EF | Setup Pages | E1- Page 1, E2 - Page 2, etc. |
| FF | Boot |  |

Index of Setup Fields

8/16 Bit I/O Recovery Time	17

Auto Configuration	16

Boot Sequence	12

Boot Up Floppy Seek	12

Boot Up NumLock Status	12

Boot Up System Speed	13

Chipset NA# Asserted	18

CPU Internal Cache /External Cache	12

CPUFAN Off in Suspend	21

Date	7

Daylight Saving	8

DMA n Assigned to	24

Doze Mode	20

DRAM Enhanced Paging	17

DRAM Leadoff Timing	16

DRAM Page Idle Timer	17

DRAM Read Burst (EDO/FP)	16

DRAM Refresh Rate	18

DRAM Timing	16

DRAM Write Burst Timing	16

Drive A/Drive B	9

ECP Mode Use DMA	29

Fast EDO Leadoff	16

Fast MA to RAS# Delay	17

Fast RAS# to CAS# Delay	16

Halt On	10

HARD DISKS	8

HDD Power Down	21

IDE HDD Block Mode	27

IDE Primary/Secondary Master/Slave PIO	28

IDE Primary/Secondary Master/Slave UDMA	28

IR Duplex Mode	28

IRQ n Assigned to	24

IRQ8 Clock Event..	21

Mem. Drive Str. (MA/RAS)	18

Memory Hole at 15M-16M	18

Memory	10

Onboard FDC Controller	28

Onboard Parallel Port	29

Onboard UART 1/2	28

Onboard UART 2 Mode	28

On-Chip Primary/Secondary PCI IDE	28

OS Select for DRAM > 64MB	13

Parallel Port EPP Type	29

Parallel Port Mode	29

PCI Delayed Transaction	18

PCI IDE IRQ Map to	24

PCI IRQ Activated by	24

PCI Passive Release	18

PCI/VGA Palette Snoop	13

Pipeline Cache Timing	18

PM Control by APM	20

PNP OS Installed	23

Power Management	19

Primary/Secondary IDE INT#	24

Quick Power On Self Test	12

Refresh RAS# Assertion	16

Reload Global Timer Events	21

Reset Configuration Data	24

Resources Controlled By	23

SDRAM (CAS Lat/RAS-to-CAS)	17

SDRAM Speculative Read	17

Security Option	13

Shadow	13

Standby Mode	20

Suspend Mode	20

Swap Floppy Drive	12

System BIOS Cacheable	17

Throttle Duty Cycle	21

Time	8

Typematic Delay (Msec)	13

Typematic Rate (Chars/Sec)	13

Typematic Rate Setting	13

USB Keyboard Support	28

USB Latency Time (PCK CLK)	28

Use IR Pins	29

Used Mem base addr	25

Used Mem Length	25

VGA Active Monitor	21

Video BIOS Cacheable	17

Video Off After	20

Video Off Method	20

Video	9

Virus Warning	11

ZZ Active in Suspend	21

For information about parallel port modes, see http://www.fapo.com/1284.htm