CMOS SETUP 
UTILITY

EliteBIOS
Version 4.51PG

User’s Guide

Intel 82430VX PCIset

for
AST Research Inc.
bcm Advanced Research

Proprietary Notice and Disclaimer

Unless otherwise noted, this document and the information herein disclosed are proprietary to AWARD Software International, Inc. (Award). Any person or entity to whom this document is furnished or who otherwise has possession thereof, by acceptance agrees that it will not be copied or reproduced in whole or in part, nor used in any manner except to meet the purposes for which it was delivered.

The information in this document is subject to change without notice, and should not be considered as a commitment by Award. Although Award will make every effort to inform users of substantive errors, Award disclaims all liability for any loss or damage resulting from the use of this document or any hardware or software described herein, including without limitation contingent, special, or incidental liability.

Copyright © 1997 by Award Software International Inc.  All rights reserved.  
Document date 10 January, 1997

Master document revision 1.3; August 1996

EliteBIOSÔ is a trademark of Award Software International Inc.

Award Software International and the Award logo are registered trademarks of Award Software International Inc.

All other products and brand names are trademarks and registered trademarks of their respective companies.

| Award Software International Inc. 777 East Middlefield Road Mountain View, CA 94043-4023 | Tel: (415) 968-4433 Fax: (415) 968-0274 www: http://www.award.com |
| --- | --- |

Table of Contents

Introduction to Setup	1

Starting Setup	1

Setup Keys	2

Getting Help	3

In Case of Problems	3

Setup Variations	3

Main Setup Menu	5

Standard CMOS Setup	7

BIOS Features Setup	11

Power Management	13

PM Timers	14

Wake Up Events ... 	15

Power Down and Resume Events	15

Integrated Peripherals	17

Password Setting	21

POST Messages	23

POST Codes	27

Index of Setup Fields	31

Introduction to Setup

This manual describes the Award BIOS Setup program.  The Setup program lets you modify basic system configuration settings.  The settings are then stored in a dedicated battery-backed memory, called CMOS RAM, that retains the information when the power is turned off.

The Award BIOS in your computer is a customized version of an industry-standard BIOS for IBM PC AT–compatible personal computers.  It supports Intel x86 and compatible processors.  The BIOS provides critical low-level support for the system central processing, memory, and I/O subsystems.

The Award BIOS has been customized by adding important, but nonstandard, features such as virus and password protection and power management.

The rest of this manual is intended to guide you through the process of configuring your system using Setup.

# Starting Setup

The Award BIOS is immediately activated when you first turn on the computer.  The BIOS reads system configuration information in CMOS RAM and begins the process of checking out the system and configuring it through the power-on self test (POST).

When these preliminaries are finished, the BIOS seeks an operating system on one of the data storage devices (hard drive, floppy drive, etc.).  The BIOS launches the operating system and hands control of system operations to it.

During POST, you can start the Setup program in one of two ways:

- By pressing <Del> immediately after switching the system on, or

- By pressing the <Del> key or by simultaneously pressing <Ctrl>, <Alt>, and <Esc> keys when the following message appears briefly at the bottom of the screen during POST:

TO ENTER SETUP BEFORE BOOT PRESS CTRL-ALT-ESC OR DEL KEY

If the message disappears before you respond and you still wish to enter Setup, restart the system to try again by turning it OFF then ON or pressing the RESET button on the system case.  You may also restart by simultaneously pressing <Ctrl>, <Alt>, and <Delete> keys.  If you do not press the keys at the correct time and the system does not boot, an error message appears and you are again asked to

PRESS F1 TO CONTINUE, CTRL-ALT-ESC OR DEL TO ENTER SETUP

# Setup Keys

The following table shows to navigate in Setup using the keyboard.

| Up arrow | Move to previous item |
| --- | --- |
| Down arrow | Move to next item |
| Left arrow | Move to the item in the left hand |
| Right arrow | Move to the item in the right hand |
| Esc key | Main Menu: Quit and not save changes into CMOS RAM. Status Page Setup Menu and Option Page Setup Menu: Exit current page and return to Main Menu |
| PgUp key | Increase the numeric value or make changes |
| PgDn key | Decrease the numeric value or make changes |
| + key | Increase the numeric value or make changes |
| - key | Decrease the numeric value or make changes |
| F1 key | General help, only for Status Page Setup Menu and Option Page Setup Menu |
| F2 key Shift-F2 | Change color from total 16 colors. F2 to select color forward, Shift-F2 to select color backward |
| F3 key | Calendar, only for Status Page Setup Menu |
| F4 key | Reserved |
| F5 key | Restore the previous CMOS value from CMOS, only for Option Page Setup Menu |
| F6 key | Load the default CMOS RAM value from BIOS default table, only for Option Page Setup Menu |
| F7 key | Load the default |
| F8 key | Reserved |
| F9 key | Reserved |
| F10 key | Save all the CMOS changes, only for Main Menu |

# Getting Help

Press F1 to pop up a small help window that describes the appropriate keys to use and the possible selections for the highlighted item.  To exit the Help Window press <Esc> or the F1 key again.

# In Case of Problems

If, after making and saving system changes with Setup, you discover that your computer no longer is able to boot, the Award BIOS supports an override to the CMOS settings that resets your system to its default configuration.

You can invoke this override by immediately pressing <Insert> when you restart your computer.  You can restart by either using the ON/OFF switch, the RESET button or by pressing <Ctrl>, <Alt> and <Delete> at the same time.

The best advice is to alter only settings that you thoroughly understand.

# Setup Variations

Not all systems have the same Setup.  While the basic look and function of the Setup program remains the same for all systems, the appearance of your Setup screens may differ from the screens shown here.  Each system design and chipset combination require custom configurations.  In addition, the final appearance of the Setup program depends on your system designer.  Your system designer can decide that certain items should not be available for user configuration and remove them from the Setup program.

This page intentionally blank.

Main Setup Menu

When you enter the Award BIOS CMOS Setup Utility, a Main Menu, similar to the one shown below, appears on the screen.  The Main Menu allows you to select from several Setup functions and two exit choices.  Use the arrow keys to select among the items and press ¿ to accept and enter the sub-menu.

A brief description of each highlighted selection appears at the bottom of the screen.  Following is a brief summary of each Setup category.

Options in the original PC AT-compatible BIOS.

## Standard CMOS Setup

Award enhanced BIOS options.

## BIOS Features Setup

Advanced Power Management (APM) options.

## Power Management Setup

I/O subsystems that depend on the integrated peripherals controller in your system.

## Integrated Peripherals

Change, set, or disable a password.  In BIOS versions that allow separate user and supervisor passwords, only the supervisor password permits access to Setup.  The user password generally allows only power-on access.

## Supervisor/User Password

BIOS defaults are factory settings for the most stable, minimal-performance system operations.

## Load BIOS Defaults

Setup defaults are factory settings for optimal-performance system operations.

## Load Setup Defaults

Save settings in nonvolatile CMOS RAM and exit Setup.

## Save & Exit Setup

Abandon all changes and exit Setup.

## Exit Without Save

Standard CMOS Setup

In the Standard CMOS menu you can set the system clock and calendar, record disk drive parameters and the video subsystem type, and select the type of errors that stop the BIOS POST.

The BIOS determines the day of the week from the other date information.  This field is for information only.

## Date

Press the ® or ¬ key to move to the desired field (date, month, year).  Press the PgUp or PgDn key to increment the setting, or type the desired value into the field.

The time format is based on the 24-hour military-time clock.  For example, 1 p.m. is 13:00:00. Press the ® or ¬ key to move to the desired field .  Press the PgUp or PgDn key to increment the setting, or type the desired value into the field.

## Time

This category may not be present in your Setup program.  When enabled, it adds one hour to the clock when daylight-saving time begins.  It also subtracts one hour when standard time returns.

## Daylight Saving

The BIOS supports up to four IDE drives.  This section does not show information about other IDE devices, such as a CD-ROM drive, or about other hard drive types, such as SCSI drives.

## HARD DISKS

NOTE:	We recommend that you select type auto for all drives.

The BIOS can automatically detect the specifications and optimal operating mode of almost all IDE hard drives.  When you select type auto for a hard drive, the BIOS detects its specifications during POST, every time the system boots.

If you do not want to select drive type auto, other methods of selecting the drive type are available:

- Match the specifications of your installed IDE hard drive(s) with the preprogrammed values for drive types 1 through 45.

- Select user and enter values into each drive parameter field.

- Use the ide hdd auto dectection function in Setup.

Here is a brief explanation of drive specifications:

- Type:  The BIOS contains a table of pre-defined drive types.  Each defined drive type has a specified number of cylinders, number of heads, write precompensation factor, landing zone, and number of sectors.  Drives whose specifications do not accommodate any pre-defined type are classified as type user.

- Size:  Disk drive capacity (approximate).  Note that this size is usually slightly greater than the size of a formatted disk given by a disk-checking program.

- Cyls:  Number of cylinders

- Head:  Number of heads

- Precomp:  Write precompensation cylinder

- Landz:  Landing zone

- Sector:  Number of sectors

- Mode:  Auto, Normal, large, or LBA

- Auto:  The BIOS automatically determines the optimal mode.

- Normal:  Maximum number of cylinders, heads, and sectors supported are 1024, 16, and 63.

- Large:  For drives that do not support LBA and have more than 1024 cylinders.

- LBA (Logical Block Addressing):  During drive accesses, the IDE controller transforms the data address described by sector, head, and cylinder number into a physical block address, significantly improving data transfer rates.  For drives with greater than 1024 cylinders.

Select the correct specifications for the diskette drive(s) installed in the computer.

## Drive A 
Drive B

| None | No diskette drive installed |
| --- | --- |
| 360K, 5.25 in | 5-1/4 inch PC-type standard drive; 360 kilobyte capacity |
| 1.2M, 5.25 in | 5-1/4 inch AT-type high-density drive; 1.2 megabyte capacity |
| 720K, 3.5 in | 3-1/2 inch double-sided drive; 720 kilobyte capacity |
| 1.44M, 3.5 in | 3-1/2 inch double-sided drive; 1.44 megabyte capacity |
| 2.88M, 3.5 in | 3-1/2 inch double-sided drive; 2.88 megabyte capacity |

Select the type of primary video subsystem in your computer.  The BIOS usually detects the correct video type automatically.  The BIOS supports a secondary video subsystem, but you do not select it in Setup.

## Video

| EGA/VGA | Enhanced Graphics Adapter/Video Graphics Array. For EGA, VGA, SEGA, SVGA or PGA monitor adapters. |
| --- | --- |
| CGA 40 | Color Graphics Adapter, power up in 40 column mode |
| CGA 80 | Color Graphics Adapter, power up in 80 column mode |
| MONO | Monochrome adapter, includes high resolution monochrome adapters |

During the power-on self-test (POST), the computer stops if the BIOS detects a hardware error.  You can tell the BIOS to ignore certain errors during POST and continue the boot-up process.  These are the selections:

## Halt On

| No errors | POST does not stop for any errors. |
| --- | --- |
| All errors | If the BIOS detects any non-fatal error, POST stops and prompts you to take corrective action. |
| All, But Keyboard | POST does not stop for a keyboard error, but stops for all other errors. |
| All, But Diskette | POST does not stop for diskette drive errors, but stops for all other errors. |
| All, But Disk/Key | POST does not stop for a keyboard or disk error, but stops for all other errors. |

You cannot change any values in the Memory fields; they are only for your information.  The fields show the total installed random access memory (RAM) and amounts allocated to base memory, extended memory, and other (high) memory.  RAM is counted in kilobytes (KB:  approximately one thousand bytes) and megabytes (MB:  approximately one million bytes).

## Memory

RAM is the computer's working memory, where the computer stores programs and data currently being used, so they are accessible to the CPU.  Modern personal computers may contain up to 64 MB, 128 MB, or more.

- Base Memory

Typically 640 KB.  Also called conventional memory.  The DOS operating system and conventional applications use this area.

- Extended Memory

Above the 1-MB boundary.  Early IBM personal computers could not use memory above 1 MB, but current PCs and their software can use extended memory.

- Other Memory

Between 640  KB and 1 MB; often called High memory.  DOS may load terminate-and-stay-resident (TSR) programs, such as device drivers, in this area, to free as much conventional memory as possible for applications.  Lines in your config.sys file that start with loadhigh load programs into high memory.

BIOS Features Setup

This screen contains industry-standard options additional to the core PC AT BIOS.  This section describes all fields offered by Award Software in this screen.  The example screen below may vary from the one in your Setup program.  Your system board designer may omit or modify some fields.

When enabled, you receive a warning message if a program (specifically, a virus) attempts to write to the boot sector or the partition table of the hard disk drive.  You should then run an anti-virus program.  Keep in mind that this feature protects only the boot sector, not the entire hard drive.

## Virus Warning

NOTE:	Many disk diagnostic programs that access the boot sector table can trigger the virus warning message.  If you plan to run such a program, we recommend that you first disable the virus warning.

Select Enabled to reduce the amount of time required to run the power-on self-test (POST).  A quick POST skips certain steps.  We recommend that you normally disable quick POST.  Better to find a problem during POST than lose data during your work.

## Quick Power On Self Test

The original IBM PCs loaded the DOS operating system from drive A (floppy disk), so IBM PC-compatible systems are designed to search for an operating system first on drive A, and then on drive C (hard disk).  However, modern computers usually load the operating system from the hard drive, and may even load it from a CD-ROM drive.

## Boot Sequence

If you have set a password, select whether the password is required every time the System boots, or only when you enter Setup.

## Security Option

If your system has a PS/2 mouse port and you install a serial pointing device, select Disabled to avoid the possibility of conflicts and confusion.

## PS/2 mouse function control

Power Management

NOTE:	This chapter describes all fields offered by Award Software in this screen.  Your system board designer may omit or modify some fields.

This option allows you to select the type (or degree) of power saving for Doze, Standby, and Suspend modes.  See the section PM Timers for a brief description of each mode.

## Power Management

This table describes each power management mode:

| Max Saving | Maximum power savings. Only Available for SL CPUs. Inactivity period is 1 minute in each mode. |
| --- | --- |
| User Define | Set each mode individually. Select time-out periods in the PM Timers section, following. |
| Min Saving | Minimum power savings. Inactivity period is 1 hour in each mode (except the hard drive). |

If Advanced Power Management (APM) is installed on your system, selecting Yes gives better power savings.

## PM Control by APM

Determines the manner in which the monitor is blanked.

## Video Off Method

| V/H SYNC+Blank | System turns off vertical and horizontal synchronization ports and writes blanks to the video buffer. |
| --- | --- |
| DPMS Support | Select this option if your monitor supports the Display Power Management Signaling (DPMS) standard of the Video Electronics Standards Association (VESA). Use the software supplied for your video subsystem to select video power management values. |
| Blank Screen | System only writes blanks to the video buffer. |

Name the interrupt request (IRQ) line assigned to the modem (if any) on your system.  Activity of the selected IRQ always awakens the system.

## MODEM Use IRQ

| # PM Timers | # The following modes are Green PC power saving functions. They are user-configurable only during User Defined Power Management mode. |
| --- | --- |

After the selected period of system inactivity (1 minute to 1 hour), the CPU clock runs at slower speed while all other devices still operate at full speed.

## Doze Mode

After the selected period of system inactivity (1 minute to 1 hour), the fixed disk drive and the video shut off while all other devices still operate at full speed.

## Standby Mode

After the selected period of system inactivity (1 minute to 1 hour), all devices except the CPU shut off.

## Suspend Mode

After the selected period of drive inactivity (1 to 15 minutes), the hard disk drive powers down while all other devices remain active.

## HDD Power Down

| # Wake Up Events ... | # You can turn On or Off monitoring of four commonly used interrupt requests so they do not awaken the system from, or reset activity timers for, Doze and Standby modes. |
| --- | --- |

For example, if you have a modem on IRQ3, you can turn On IRQ3 as a wake-up event, so an interrupt from the modem can wake up the system. Or you may wish to turn Off IRQ12 (the PS/2) mouse as a wake-up event, so accidentally brushing the mouse does not awaken the system.

The default wake-up event is keyboard activity.

| # Power Down and Resume Events |  | # You can disable monitoring of common interrupt requests so they do not awaken the system from, or reset activity timers for, Suspend mode. |  |  |
| --- | --- | --- | --- | --- |
|  | IRQ3 (COM 2 ) IRQ4 (COM 1) IRQ5 (LPT 2) IRQ6 (Floppy Disk) IRQ7 (LPT 1) IRQ8 (RTC Alarm) IRQ9 (IRQ2 Redir) |  | IRQ10 (Reserved) IRQ11 (Reserved) IRQ12 (PS/2 Mouse) IRQ13 (Coprocessor) IRQ14 (Hard Disk) IRQ15 (Reserved) |  |

This page intentionally blank.

Integrated Peripherals

NOTE:	This chapter describes all fields offered by Award Software in this screen.  Your system board designer may omit or modify some fields.

The integrated peripheral controller contains an IDE interface with support for two IDE channels.  Select Enabled to activate each channel separately.

## On-Chip Primary/Secondary PCI IDE

Select Enabled if your system contains a Universal Serial Bus (USB) controller and you have USB peripherals.

## USB Controller

Select Enabled if your system contains a Universal Serial Bus (USB) controller and you have a USB keyboard.

## USB Keyboard Support

Select Enabled if your system has a floppy disk controller (FDC) installed on the system board and you wish to use it.  If you install an add-in FDC or the system has no floppy drive, select Disabled in this field.

## Onboard FDC Controller

Select a logical COM port name and matching address for the first and second serial ports.

## Onboard Serial Port 1/2

Select an optional infrared mode for the second serial port:

## UART2 Mode

IrDA	Compliant with Infrared Data Association (IrDA) standards 
	for serial infrared port
ASK-IR	Amplitude shift keyed infrared port

In an infrared port mode, this field appears.  Full-duplex mode permits simultaneous two-direction transmission.  Half-duplex mode permits transmission in one direction only at a time.  Select the value required by the IR device connected to the IR port.

## Duplex Select

Consult your IR peripheral documentation to select the correct setting of the TxD and RxD signals.

## TxD, RxD Active

Select a logical LPT port name and matching address for the physical parallel (printer) port.

## Onboard Parallel Port

Select an operating mode for the onboard parallel port.  Select an operating mode for the onboard parallel (printer) port.  Select Normal unless your hardware and software require one of the other modes offered in this field.

## Parallel Port Mode

Select a DMA channel for the port in ECP mode.

## ECP Mode Use DMA

Select the value required by the IR device connected to the IR port. Full-duplex mode permits simultaneous two-direction transmission.  Half-duplex mode permits transmission in one direction only at a time.  If no infrared port is present in the system, select Disabled.

## InfraRed Duplex Type

Select Enabled to use the audio capabilities of your system.

## Oboard Audio Chip

This page intentionally blank.

Password Setting

When you select this function, a message appears at the center of the screen:

enter password:

Type the password, up to eight characters, and press ¿.  Typing a password clears any previously entered password from CMOS memory.

Now the message changes:

confirm password:

Again, type the password and press ¿.

To abort the process at any time, press Esc.

In the Security Option item in the BIOS Features Setup screen, select System or Setup:

| System | Enter a password each time the system boots and when ever you enter Setup. |
| --- | --- |
| Setup | Enter a password when ever you enter Setup. |

NOTE:	To clear the password, simply press ¿ when asked to enter a password.  Then the password function is disabled.

This page intentionally blank.

POST Messages

During the power-on self test (POST), the BIOS either sounds a beep code or displays a message when it detects a correctable error.

Following is a list of POST messages for the ISA BIOS kernel.  Specific chipset ports and BIOS extensions may include additional messages.  An error message may be followed by a prompt to press F1 to continue or press DEL to enter Setup.

### Beep

Currently the only beep code indicates that a video error has occurred and the BIOS cannot initialize the video screen to display any additional information.  This beep code consists of a single long beep followed by two short beeps.

### BIOS ROM checksum error - System halted

The checksum of the BIOS code in the BIOS chip is incorrect, indicating the BIOS code may have become corrupt.  Contact your system dealer to replace the BIOS.

### CMOS battery failed

CMOS battery is no longer functional.  Contact your system dealer for a replacement battery.

### CMOS checksum error - Defaults loaded

Checksum of CMOS is incorrect, so the system loads the default equipment configuration.  A checksum error may indicate that CMOS has become corrupt.  This error may have been caused by a weak battery.   Check the battery and replace if necessary.

### CPU at nnnn

Displays the running speed of the CPU.

### Display switch is set incorrectly.

The display switch on the motherboard can be set to either monochrome or color.  This message indicates the switch  is set to a different setting than indicated in Setup.  Determine which setting is correct, and then either turn off the system and change the jumper, or enter Setup and change the VIDEO selection.

### Press ESC to skip memory test

The user may press Esc to skip the full memory test.

### Floppy disk(s) fail

Cannot find or initialize the floppy drive controller or the drive.  Make sure the controller is installed correctly.  If no floppy drives are installed, be sure the Diskette Drive selection in Setup is set to NONE or AUTO.

### HARD DISK initializing  
Please wait a moment...

Some hard drives require extra time to initialize.

### HARD DISK INSTALL FAILURE

Cannot find or initialize the hard drive controller or the drive.  Make sure the controller is installed correctly.  If no hard drives are installed, be sure the Hard Drive selection in Setup is set to NONE.

### Hard disk(s) diagnosis fail

The system may run specific disk diagnostic routines.  This message appears if one or more hard disks return an error when the diagnostics run.

### Keyboard error or no keyboard present

Cannot initialize the keyboard.  Make sure the keyboard is attached correctly and no keys are pressed during POST.  To purposely configure the system without a keyboard, set the error halt condition in Setup to HALT ON ALL, BUT KEYBOARD.  The BIOS then ignores the missing keyboard during POST.

### Keyboard is locked out - Unlock the key

This message usually indicates that one or more keys have been pressed during the keyboard tests.  Be sure no objects are resting on the keyboard.

### Memory Test :

This message displays during a full memory test, counting down the memory areas being tested.

### Memory test fail

If POST detects an error during memory testing, additional information appears giving specifics about the type and location of the memory error.

### Override enabled - Defaults loaded

If the system cannot boot using the current CMOS configuration, the BIOS can override the current configuration is a set of BIOS defaults designed for the most stable, minimal-performance system operations.

### Press TAB to show POST screen

System OEMs may replace the Award BIOS POST display with their own proprietary display.  Including this message in the OEM display permits the operator to switch between the OEM display and the default POST display.

### Primary master hard disk fail

POST detects an error in the primary master IDE hard drive.

### Primary slave hard disk fail

POST detects an error in the secondary master IDE hard drive.

### Resuming from disk, Press TAB to show POST screen

Award offers a save-to-disk feature for notebook computers.  This message may appear when the operator re-starts the system after a save-to-disk shut-down.  See the Press TAB ... message above for a description of this feature.

### Secondary master hard disk fail

POST detects an error in the primary slave IDE hard drive.

### Secondary slave hard disk fail

POST detects an error in the secondary slave IDE hard drive.

This page intentionally blank.

POST Codes

NOTE:	EISA POST codes are typically output to port address 300h.  
ISA POST codes are output to port address 80h.

| Code (hex) | Name | Description |
| --- | --- | --- |
| C0 | Turn Off Chipset Cache | OEM Specific-Cache control |
| 1 | Processor Test 1 | Processor Status (1FLAGS) Verification. Tests the following processor status flags: carry, zero, sign, overflow, The BIOS sets each flag, verifies they are set, then turns each flag off and verifies it is off. |
| 2 | Processor Test 2 | Read/Write/Verify all CPU registers except SS, SP, and BP with data pattern FF and 00. |
| 3 | Initialize Chips | Disable NMI, PIE, AIE, UEI, SQWV Disable video, parity checking, DMA Reset math coprocessor Clear all page registers, CMOS shutdown byte Initialize timer 0, 1, and 2, including set EISA timer to a known state Initialize DMA controllers 0 and 1 Initialize interrupt controllers 0 and 1 Initialize EISA extended registers. |
| 4 | Test Memory Refresh Toggle | RAM must be periodically refreshed to keep the memory from decaying. This function assures that the memory refresh function is working properly. |
| 5 | Blank video, Initialize keyboard | Keyboard controller initialization. |
| 6 | Reserved |  |
| 7 | Test CMOS Interface and Battery Status | Verifies CMOS is working correctly, detects bad battery. |
| BE | Chipset Default Initialization | Program chipset registers with power on BIOS defaults. |
| C1 | Memory presence test | OEM Specific-Test to size on-board memory |
| C5 | Early Shadow | OEM Specific-Early Shadow enable for fast boot. |
| C6 | Cache presence test | External cache size detection |
| 8 | Setup low memory | Early chip set initialization Memory presence test OEM chip set routines Clear low 64K of memory Test first 64K memory. |
| 9 | Early Cache Initialization | Cyrix CPU initialization Cache initialization |
| A | Setup Interrupt Vector Table | Initialize first 120 interrupt vectors with SPURIOUS_INT_HDLR and initialize INT 00h-1Fh according to INT_TBL |
| B | Test CMOS RAM Checksum | Test CMOS RAM Checksum, if bad, or insert key pressed, load defaults. |
| C | Initialize keyboard | Detect type of keyboard controller (optional) Set NUM_LOCK status. |
| D | Initialize Video Interface | Detect CPU clock. Read CMOS location 14h to find out type of video in use. Detect and Initialize Video Adapter. |
| E | Test Video Memory | Test video memory, write sign-on message to screen. Setup shadow RAM - Enable shadow according to Setup. |
| F | Test DMA Controller 0 | BIOS checksum test. Keyboard detect and initialization |
| 10 | Test DMA Controller 1 |  |
| 11 | Test DMA Page Registers | Test DMA Page Registers. |
| 12-13 | Reserved |  |
| 14 | Test Timer Counter 2 | Test 8254 Timer 0 Counter 2. |
| 15 | Test 8259-1 Mask Bits | Verify 8259 Channel 1 masked interrupts by alternately turning off and on the interrupt lines. |
| 16 | Test 8259-2 Mask Bits | Verify 8259 Channel 2 masked interrupts by alternately turning off and on the interrupt lines. |
| 17 | Test Stuck 8259's Interrupt Bits | Turn off interrupts then verify no interrupt mask register is on. |
| 18 | Test 8259 Interrupt Functionality | Force an interrupt and verify the interrupt occurred. |
| 19 | Test Stuck NMI Bits (Parity/IO Check) | Verify NMI can be cleared. |
| 1A |  | Display CPU clock |
| 1B-1E | Reserved |  |
| 1F | Set EISA Mode | If EISA non-volatile memory checksum is good, execute EISA initialization. If not, execute ISA tests an clear EISA mode flag. Test EISA Configuration Memory Integrity (checksum & communication interface). |
| 20 | Enable Slot 0 | Initialize slot 0 (System Board). |
| 21-2F | Enable Slots 1-15 | Initialize slots 1 through 15. |
| 30 | Size Base and Extended Memory | Size base memory from 256K to 640K and extended memory above 1MB. |
| 31 | Test Base and Extended Memory | Test base memory from 256K to 640K and extended memory above 1MB using various patterns. NOTE: This test is skipped in EISA mode and can be skipped with ESC key in ISA mode. |
| 32 | Test EISA Extended Memory | If EISA Mode flag is set then test EISA memory found in slots initialization. NOTE: This test is skipped in ISA mode and can be skipped with ESC key in EISA mode. |
| 33-3B | Reserved |  |
| 3C | Setup Enabled |  |
| 3D | Initialize & Install Mouse | Detect if mouse is present, initialize mouse, install interrupt vectors. |
| 3E | Setup Cache Controller | Initialize cache controller. |
| 3F | Reserved |  |
| BF | Chipset Initialization | Program chipset registers with Setup values |
| 40 |  | Display virus protect disable or enable |
| 41 | Initialize Floppy Drive & Controller | Initialize floppy disk drive controller and any drives. |
| 42 | Initialize Hard Drive & Controller | Initialize hard drive controller and any drives. |
| 43 | Detect & Initialize Serial/Parallel Ports | Initialize any serial and parallel ports (also game port). |
| 44 | Reserved |  |
| 45 | Detect & Initialize Math Coprocessor | Initialize math coprocessor. |
| 46 | Reserved |  |
| 47 | Reserved |  |
| 48-4D | Reserved |  |
| 4E | Manufacturing POST Loop or Display Messages | Reboot if Manufacturing POST Loop pin is set. Otherwise display any messages (i.e., any non-fatal errors that were detected during POST) and enter Setup. |
| 4F | Security Check | Ask password security (optional). |
| 50 | Write CMOS | Write all CMOS values back to RAM and clear screen. |
| 51 | Pre-boot Enable | Enable parity checker Enable NMI, Enable cache before boot. |
| 52 | Initialize Option ROMs | Initialize any option ROMs present from C8000h to EFFFFh. NOTE: When FSCAN option is enabled, ROMs initialize from C8000h to F7FFFh. |
| 53 | Initialize Time Value | Initialize time value in 40h: BIOS area. |
| 60 | Setup Virus Protect | Setup virus protect according to Setup |
| 61 | Set Boot Speed | Set system speed for boot |
| 62 | Setup NumLock | Setup NumLock status according to Setup |
| 63 | Boot Attempt | Set low stack Boot via INT 19h. |
| B0 | Spurious | If interrupt occurs in protected mode. |
| B1 | Unclaimed NMI | If unmasked NMI occurs, display Press F1 to disable NMI, F2 reboot. |
| E1-EF | Setup Pages | E1- Page 1, E2 - Page 2, etc. |
| FF | Boot |  |

Index of Setup Fields

BIOS Features Setup	6

Boot Sequence	12

Date	7

Daylight Saving	8

Doze Mode	14

Drive A  Drive B	9

Duplex Select	18

ECP Mode Use DMA	18

Exit Without Save	6

Halt On	10

HARD DISKS	8

HDD Power Down	15

InfraRed Duplex Type	19

Integrated Peripherals	6

Load BIOS Defaults	6

Load Setup Defaults	6

Memory	10

MODEM Use IRQ	14

Oboard Audio Chip	19

Onboard FDC Controller	18

Onboard Parallel Port	18

Onboard Serial Port 1/2	18

On-Chip Primary/Secondary PCI IDE	17

Parallel Port Mode	18

PM Control by APM	14

Power Down and Resume Events	15

Power Management Setup	6

Power Management	13

PS/2 mouse function control	12

Quick Power On Self Test	12

Save & Exit Setup	6

Security Option	12

Standard CMOS Setup	5

Standby Mode	14

Supervisor/User Password	6

Suspend Mode	15

Time	8

TxD, RxD Active	18

UART2 Mode	18

USB Controller	17

USB Keyboard Support	18

Video Off Method	14

Video	9

Virus Warning	11

Wake Up Events ...	15

For information about parallel port modes, see http://www.fapo.com/1284.htm