Project “Napalm”

Product Specification -- Version 1.07

Revision History

| Revision | Description | By | Date |
| --- | --- | --- | --- |
| .01 | Initial Draft | GK | 3/15/99 |
| .02 | Included input from Bill Milford; 1Mx32 SGRAM proposed PCB deleted; expanded description of how multi monitor support is to work (RGB/RGB and RGB/DVI; RGB/TV dual screen support was deleted. | GK | 3/19/99 |
| .03 | Included input from Peter Wicher. | GK | 3/22/99 |
| .04 | Modified for dual tape out of napalm and napalm SLI, additional work on MRD | GK | 3/25/99 |
| .99 | Interdepartmental Input | GK | 3/29/99 |
| 1.00 | First Release for Cross Functional review | GK | 4/1/99 |
| 1.01 | First General release | GK | 4/19/99 |
| 1.02 | Added ROI | GK | 4/20/99 |
| 1.03 | Updates and General Release | GK | 4/21/99 |
| 1.05 | Revision to reflect new board configurations | PW | 6/17/99 |
| 1.06 | Remove mini-gl; remove TV option on retail boards; add TV option to OEM2; corrected RAMDAC clk to 350MHz; modified sample layout to include DVI and remove LCDfx; removed audio subsystem comments; added TV support in 320xN and 720xN modes to table 4.5.2.1; updated 12.2.2 to match Velocity family languages & documentation; 12.2.4 legacy OS requirement noted as English only; added tables 12.2.4.1, 12.2.4.2, 12.2.4.3 to include mode support for OS/2, Win3.11, NT3.51; removed QuickInstall requirements from p. 19 by referencing sec. 12.2.2; added 3DWB00 requirements to table 13.1; removed table 17.1 be referencing sec. 12.2.2 for all localization and documentation requirements; updated Table 18.1 to reflect a higher level of packaging details. | PW | 6/24/99 |
| 1.07 | Added support for 1024x512 mode; ADD VIDEO SIDE PORT TO OEM 3; ADD 3D MODES TO 2048X1536; video side port added to PCB6; changed board priorities to emphasize 64MB for OEMs by making PCB3 “OEM 1” a 64MB config.; changed all OEM configs to ATX form factor to accommodate both TV and DVI pop options (later additional board def’s beyond PCB6 will be optimized for cost by removing one or both pop options); changed OEM1 to be 6 layers to minimize risk and optimize time to market; moved table 4.3 up to section 1 to become table 1.1a to show memory types, speeds, and configurations next to the board definitions; re-ordered boards in table 1.1a to match new OEM board priorities; |  |  |

Product Specification	4

1. PCB Configurations	4

2. BOM Configurations	4

3. Functional Block Diagrams	4

4. Graphics Subsystem	5

5. Bus Interface	8

6. Video Subsystem	8

7. Audio Subsystem	9

8. Power Budget	9

9. I/O Connectors	10

10. Operating Parameters	12

11. System Support	12

12. Software Support	12

Table 12.2.3   Feature Requirements	14

Direct Draw	16

Direct3D Requirements Table	16

VGA Mode Support Requirements Table	17

Video Players Requirements Tables	17

Dynamic EDID / GTF – Timings	17

12.4.1 Interpreting EDID	17

ACPI Requirements Table	18

13. Performance	18

14. Quality / Reliability / Compatibility	19

15. Certifications / Compliance	20

16. Manufacturing Considerations	20

17. Documentation	20

18. Packaging	20

# Product Specification

# 1. PCB Configurations

  - Table of 210 Level Physical PCB Requirements used in Napalm AIC Platform:

| `P/N 210-level | Code Name | OEM / Retail | Form Factor | MEM Capacity | MEM Type | SLI | AGP Bridge | Bus | Max. PCB Layers | RGB DB15 | NTSC / PAL Out | DVI | Video Side Port |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PCB-1 | Retail 1 | Retail | ATX | 64MB | 2MX32 | YES (2-way) | NO | AGP | 6 | YES | No | No | YES |
| PCB-2 | Retail 2 | Retail | ATX | 32MB | 1Mx32** SGRAM | YES (2-way) | NO | PCI | 6 | YES | No | No | YES |
| PCB-3 | OEM 1 | OEM | ATX | 64MB | 4MX16 SDRAM | NO | NO | AGP | 6 | YES | Option | Option | YES |
| PCB-4 | Retail 3 | Retail | ATX | 128MB | 2MX32 SDRAM | YES (4-way) | YES | AGP | 8 | YES | No | No | YES |
| PCB-5 | OEM 2 | OEM | ATX | 32MB | 2MX32 SDRAM | NO | NO | AGP | 4 | YES | Option | Option | YES |
| PCB-6 | OEM 3 | OEM | ATX | 16MB | 1Mx32 ** SGRAM | NO | NO | AGP | 4 | YES | Option | Option | YES |
| TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |

** 1Mx32 SGRAM supply will need to be firmly committed from at least two major DRAM manufacturers.

Max. PCB layers are planned according to cost targets

1.1a Table of memory types and speeds

| 210-level P/N | Memory Type | Max. FB Size | Max. Speed | Max. # Devices |
| --- | --- | --- | --- | --- |
| PCB-1 | 2MX32 SDRAM | 64MB | 183MHz | 4 |
| PCB-2 | 1Mx32 SGRAM | 32MB | 166MHZ | 8 |
| PCB-3 | 4MX16 SDRAM | 64MB | 166MHz | 4 |
| PCB-4 | 2MX32 SDRAM | 128MB | 166MHz | 16 |
| PCB-6 | 2MX32 SDRAM | 32MB | 166MHz | 4 |
| PCB-5 | 1Mx32 SGRAM | 16MB | 166MHz | 4 |

  - Mechanical Dimensions

The Napalm AIC Base PCB(s) should meet the minimum and maximum dimensions for an AGP/ATX chassis specification.   Multiple layers may be employed for routing to ensure highest quality, reliability, and to make the product manufacture-able.  All components should be placed on the component side of the PCB (if possible) for maximum assembly efficiency.  There should be dedicated Power and Ground planes with no or minimal traces on these planes.

# 2. BOM Configurations

Table 2.1  140-level BOM Configurations and Part Numbers

| P/N 140-Level | Base PCB 210-Level | Memory SD or SG | Memory Array | AGP or PCI | DFP or TV | Form factor | RGB D15 | Name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBD |  |  |  |  |  |  |  |  |

2.2. 110-level BOM configurations

(110 BOM configurations 3dfx Internal Data)

# 3. Functional Block Diagrams

3.1 CONFIGURATION EXAMPLE

TBD

RGB

OUT

ITU-601 (26-pin)

FLASH

BIOS

SGRAM or SDRAM

SGRAM or SDRAM

SGRAM or SDRAM

SGRAM or SDRAM

DVI OUT

3dfx

Napalm

SGRAM or SDRAM

SGRAM or SDRAM

Silicon Image

TMDS  Transimitter

SGRAM or SDRAM

TV OUT

SGRAM or SDRAM

Break-away

tab for SFF NLX

AGP2X/4X or PCI BUS

# 4. Graphics Subsystem

4.1 Graphics Controller

The graphics subsystem of Napalm AIC will be based on 3dfx’s Napalm  2D/3D graphics accelerator.  Napalm  will be configured in an AGP1X/2X/4X signaling environment capable of supporting all functional modes of these specifications. Memory interface will consist of SDRAM and SGRAM capable of supporting speeds of 166-200MHz (see 140 BOM configurations above).

4.2 Bus Interface

4.2.1 Bus Type

The single-Napalm boards attach to the AGP bus using a universal slot and the AGP2X/4X transfer specification within a 3.3v/1.5v-signaling environment. All aspects of the AGP 2X/4X specifications must be met including signal loading and trace route considerations.

The SLI configurations attach to the AGP bus using a universal slot and the PCI 66 protocol  in a 3.3V signaling environment.  The 2-way SLI boards present a dual load to the AGP port.  The 4-way SLI board uses a bridge chip and presents a single load.

4.3 Local Frame Buffer Type, Configuration, and Speed

See table 1.1a for frame buffer type, configuration, and speed.

4.4 Clock Generation

All necessary clocks are generated internally to the Napalm  controller.  The only necessary components are an external crystal oscillator circuit.  Care must be taken in the design and layout of this circuit to prevent clock jitter or instability including potential clock start-up problems on power-up.

4.5. RAMDAC

The RAMDAC is internal to the Napalm graphics accelerator.

4.5.1 Clock Rate

The RAMDAC is specified to operate properly at 350MHz

4.5.2  Desired Resolutions and Refresh Rates

4.5.2.1 Table of  Driver Modes for Win95, Win98, WinNT, and Win2000

| Resolution | Aspect Ratio | Refresh Rates (Hz) Vertical Frequency | Color Depth (bpp) | Comments |
| --- | --- | --- | --- | --- |
| 320x200 |  | 70,85 | 8,16 | Ddraw, SW D3D only, No Desktop, TV OUT support: NTSC and PAL; WHQL Required |
| 320x240 |  | 60,72,75,85 | 8,16 | Ddraw, SW D3D only, No Desktop, TV OUT support: NTSC and PAL; WHQL Required |
| 400x300 |  | 60,72,75,85 | 8,16 | Ddraw, SW D3D only, No Desktop, WHQL Required |
| 512x384 |  | 60,70,75,85 | 8,16 | Ddraw, SW D3D, D3D, Glide, OGL No Desktop, WHQL Recommended |
| 640x400 |  | 70,85 | 8,16 | Ddraw, SW D3D, D3D, Glide, OGL No Desktop, WHQL Requirement |
| 640x480 | 4:3 | 60,72,75,85,100, 120,140,160 | 8,16,32 | Desktop, Ddraw, (D3D, Glide, OGL) * TV OUT support: NTSC and PAL |
| 720x480 |  | 60,72,85 | 8,16,32 | VIDEO ONLY Ddraw only: Video, No Desktop; TV OUT support: NTSC and PAL; WHQL Requirement |
| 720x576 |  | 72,100 | 8,16,32 | VIDEO ONLY: Ddraw only: Video, No Desktop; TV OUT support: NTSC and PAL; WHQL Requirement MPEG2: PAL |
| 800x600 | 4:3 | 60,72,75,85,100, 120,140,160 | 8,16,32 | Desktop, Ddraw, (D3D, Glide, OGL) * TV OUT support: NTSC and PAL |
| 960x720 |  | 60,75,85 | 8,16 | Desktop, Ddraw, (D3D, Glide, OGL) * |
| 1024x512 | 16:9 | 60, others TBD | 8,16,32 | Desktop, Ddraw, (D3D, Glide, OGL) * |
| 1024x768 | 4:3 | 60,70,75,85,100,120 | 8,16,32 | Desktop, Ddraw, (D3D, Glide, OGL) * |
| 1152x864 | 4:3 | 60,70,75,85,100,120 | 8,16,32 | Desktop, Ddraw, (D3D, Glide, OGL) * |
| 1280x800 | 16:10 | 60, 75, 85, 100 | 8, 16, 32 | Desktop, Ddraw |
| 1280x960 |  | 60,75,85 | 8,16 | Desktop, Ddraw, (D3D, Glide, OGL) * |
| 1280x1024 | 5:4 | 60,75,85,100 | 8,16,32 | Desktop, Ddraw, (D3D, Glide, OGL) * |
| 1600x1024 | 16:10 | 60,76,85 | 8,16,32 | Desktop, Ddraw, (D3D, Glide, OGL) * |
| 1600x1200 | 4:3 | 60,65,70,75,80,85,100 | 8,16,32 | Desktop, Ddraw, (D3D, Glide, OGL) * |
| 1792x1344 | 4:3 | 60,75 | 8,16,32 | Desktop, Ddraw |
| 1800x1440 |  | 75 | 8,16,32 | Desktop, Ddraw |
| 1856x1392 | 4:3 | 60,75 | 8,16,32 | Desktop, Ddraw |
| 1920x1080 | 16:9 | 60,72,75,85 | 8,16,32 | Desktop, Ddraw |
| 1920x1200 | 16:10 | 60,76,85 | 8,16,32 | Desktop, Ddraw |
| 1920x1440 | 4:3 | 60,75 | 8,16,32 | Desktop, Ddraw |
| 2046x1536 | 4:3 | 60,75 | 8,16,24 | Desktop (60Hz only for Napalm 2000) |

- Note: All refresh rates supported by Napalm  unless otherwise noted.

- Note: Desktop and Ddraw supported at all color depths, HW accelerated D3D, Glide and OGL supported only at 8 and 16-bit color depths.

- Note: All refresh rates in BOLD will use pre-calculated GTF timings.

4.5.2.2 Table of VGA Modes in BIOS

| Resolution | Aspect Ratio | Vertical Frequency (Hz) | Color Depth | Comments |
| --- | --- | --- | --- | --- |
| 320x200 | - | 70 | 2,4,8 | Text (8x8 characters) |
| 360x400 | - | 70 | 4 | Text 40x25 (8x8 characters) |
| 640x200 | - | 70 | 1,4 | Text (8x8 characters) |
| 640x350 | - | 70 | 4 | Text (8x16 characters) |
| 640x480 | 4:3 | 60 | 1,4 | Text (8x16 characters) |
| 720x400 | - | 70 | 4 | Text 80x25 (8x8 characters) |

4.5.2.3 Table of Extended TEXT Modes in BIOS

| Resolution | Aspect Ratio | Vertical Frequency (Hz) | Color Depth | Comments |
| --- | --- | --- | --- | --- |
| 720x480 | - | 60 | 4 | Text 80x60 (9x8 characters) |
| 1188x344 | - | 70 | 4 | Text 132x43 (9x8 characters) |
| 1188x400 | - | 70 | 4 | Text 132x25 (9x16 characters) |
| 1188x400 | - | 70 | 4 | Text 132x50 (9x8 characters) |
| 1188x480 | - | 60 | 4 | Text 132x60 (9x8 characters) |

4.5.2.4 Table of Extended VGA Modes in BIOS

| Resolution | Aspect Ratio | Vertical Frequency (Hz) | Color Depth | Frame Buffer Alignment |
| --- | --- | --- | --- | --- |
| 320x200 | - | 70 | 8,16,24 | Banked (Windowed) + Linear |
| 320x240 | 4:3 | 70 | 8,16,24 | Banked + Linear |
| 400x300 | 4:3 | 70 | 8,16,24 | Banked + Linear |
| 512x384 | 4:3 | 70 | 8,16,24 | Banked + Linear |
| 640x400 | - | 70 | 8,16,24 | Banked + Linear |
| 640x480 | 4:3 | 70 | 8,16,24 | Banked + Linear |
| 800x600 | 4:3 | 70 | 4,8,16,24 | Banked + Linear (4bpp banked only) |
| 1024x768 | 4:3 | 70 | 8,16,24 | Banked + Linear |
| 1280x1024 | 5:4 | 70 | 8,16,24 | Banked + Linear |

- This will require a VBE 3.0 BIOS.  Unlimited Refresh Rates are Possible.  The BIOS has the listed refresh as the default mode set.  See section 12.1 for BIOS details.

4.6 Video Side Port

4.6.1 Implementation

The 8-bit Video Input (CCIR 601 and CCIR 656) protocol and connector must be supported on the Napalm AIC product.

4.7 Power

The 3dfx Napalm  is a 0.25 device requiring +2.5v for the VDD-core supply. This voltage must be generated by an external voltage regulator, which meets the maximum current requirements of Napalm .

Consideration for varying the VDD-core voltage must be made as Napalm  may require some +/- modification to this voltage for optimal performance.

4.8. DVI Interface

Napalm AIC will provide support for digital flat panels via TMDS signaling. Napalm  will interface to a Silicon Image 154 and 164 transmitter through the 12-bit digital interface port.  The TMDS transmitter output will connect to the digital-only 24-pin DVI connector (See DVI 1.0 spec.).

# 5. Bus Interface

5.1 System Bus Type

Napalm AIC will interface to the host CPU via the AGP and PCI bus. This signaling environment is 3.3v for AGP2X and 1.5V for AGP4X.  It is assumed that the dual load presented by the 2-way SLI boards, while outside of the AGP2X spec., will not present a problem.

5.2 Power

The single-chip Napalm AIC must not exceed the power budget on any signal or power group specified by AGP 1.0.  The 2-way SLI boards will exceed the AGP 1.0 spec and require over 30W of power.  The 4-way SLI boards will exceed the AGP 1.0 spec and require over 60W of power.  A simple means of supplying additional power to the SLI boards must be implemented so that end users, with no technical support, can easily install the product in any Pentium 200MMX and later PC system.

# 6. Video Subsystem

6.1 Table of Video Subsystem Feature Support

| TV Encode NTSC / PAL | TV Tuner NTSC / PAL | Video Decoder | DVD S/W – H/W | MPEG-2 Encoder |
| --- | --- | --- | --- | --- |
|  | Supports 3dfx Desktop TV | Supports 3dfx Desktop TV | S/W | - |

6.2 Tuner Interface

Although Napalm AIC does not contain a tuner subsystem, support for the 3dfx Desktop TV card must be insured.

6.3 Video Decoder

Napalm AIC has no on-board video decoder, but is expected to interface with video decoder boards through the CCIR-601 / CCIR 656 interface.

6.4 Video Encoder

TV-out is not an important feature in the North America market.  The TV-out functionality will be driven by EC needs TBD.

Table of NTSC and PAL requirements for TV Out

| Country | Color System | Priority | Lines | Field Rate (Hz) | Color Burst Frequency (Hz) | Blanking Setup ? |
| --- | --- | --- | --- | --- | --- | --- |
| North America, South America, Taiwan, & Japan | NTSC - M | A | 525 | 59.94 | 3.579545 | Yes |
| Europe & Asia | PAL – BDGHI | A | 625 | 50.00 | 4.43361875 | No |
| Brazil | PAL - M | B | 525 | 59.94 | 3.57561149 | Yes |
| Paraguay & Uraguay | PAL - N | B | 625 | 50.00 | 4.43361875 | Yes |
| Argentina | PAL - Nc | B | 625 | 50.00 | 3.58205625 | No |

The NTSC/PAL data can be stored in an EEPROM for boot to TV to function properly.  Napalm may provide no support for SECAM though most SECAM Countries support PAL in their televisions for everything except broadcast signal.

6.5 MPEG-2 Encoder / Decoder

Napalm AIC should be capable of supporting video-in through the CCIR-601 / CCIR 656 interface or AGP bus from the following sources:

- DVD playback card

- DVD playback from SoftDVD players:

| Player | Priority |
| --- | --- |
| Intervideo | A |
| QI | A |
| Medimatics | B |
| Zoran | A |

- Video from stand alone TV tuner card via PCI-AGP bus transfer

- Video from stand alone TV tuner card via video port

6.6 DTV / HDTV / DBS Support

Napalm AIC will support PCI DTV streams decoded via an AFD software system on Pentium III CPUs or compatible data rates across PCI / AGP or through the CCIR -601 interface.

# 7. Audio Subsystem

N/A

# 8. Power Budget

The power budget for Napalm AIC is governed by the maximum power specifications set forth in the AGP4X specification for the single-chip boards, is 35W for the 2-way SLI boards, and is 65W for the 4-way SLI board.

Power consumption should be minimized through careful parts selection 

8.1. Power Distribution

Power distribution should be as planar as possible to maximize distribution effectiveness and minimize noise coupling.

# 9. I/O Connectors

Napalm AIC will support the following I/O connectors:

- HD15 				VGA output and monitor detection I/O

- DVI digital only 		Provides Digital Flat Panel interface

- Video				S-Video out (Composite video available through dongle)

- CCIR –601/656		Video Input 2x13 header

9.1. VGA Port  HD15 Connector

Figure 9.1  VGA Port Layout

| Pin | Signal | Description | Pin | Signal | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | RED | Red | 9 | +5V | DDC Power |
| 2 | GREEN | Green | 10 | SGND | Sync Ground |
| 3 | BLUE | Blue | 11 | ID0 | Monitor ID Bit 0 (opt) |
| 4 | RES | Reserved | 12 | SDA | Serial Data (DDC2B) |
| 5 | GND | DDC Return | 13 | HSYNC | Horizontal Sync |
| 6 | GND | Red Ground | 14 | VSYNC | Vertical Sync |
| 7 | GND | Green Ground | 15 | SCL | Serial Clock (DDC2B) |
| 8 | GND | Blue Ground |  |  |  |

Table 9.1  VGA Port Signals

9.2  DVI

See section 5.2.3.1 of DVI 1.0

Figure 9.2 Digital Flat Panel Out

| Pin | Signal | Description | Pin | Signal | Description |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  | 13 |  |  |
| 2 |  | see DVI 1.0 sec. 5.2.1.1 | 14 |  |  |
| 3 |  |  | 15 |  |  |
| 4 |  |  | 16 |  |  |
| 5 |  |  | 17 |  |  |
| 6 |  |  | 18 |  |  |
| 7 |  |  | 19 |  |  |
| 8 |  |  | 20 |  |  |
| 9 |  |  | 21 |  |  |
| 11 |  |  | 22 |  |  |
| 12 |  |  | 23 |  |  |

Table 9.2  Digital Flat Panel Out Signals

9.3  Video:

1

4

2

3

TV out will be provided via an S-Video connector.   Composite Video Out (RCA Jack) will still be available through a dongle as required.

Figure 9.3.1 S-Video connector

| Pin | Signal | Description | Pin | Signal | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | GND | Ground | 3 | Luma | Brightness |
| 2 | GND | Ground | 4 | Chroma | Color |

Table 9.3.1  S-Video Signals

Figure 9.3.2  Composite Video-Out (RCA Jack)

| Pin | Signal | Description | Pin | Signal | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | CVBS | Composite Video Out | 2 | GND | Ground |

Table 9.3.2  Composite Video-Out Jack Signals

9.4  CCIR -601 Header

Figure 9.4  Video Input Connector Layout

| Pin | Signal | Description | Pin | Signal | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | GND | Ground | 2 | P0 | VID (0) |
| 3 | GND | Ground | 4 | P1 | VID (1) |
| 5 | GND | Ground | 6 | P2 | VID (2) |
| 7 | EVIDEO* | VACTIVE | 8 | P3 | VID (3) |
| 9 | ESYNC# | User-Defined | 10 | P4 | VID (4) |
| 11 | EDCLK# | VREF | 12 | P5 | VID (5) |
| 13 | N/C | 12CCLK | 14 | P6 | VID (6) |
| 15 | GND | Ground | 16 | P7 | VID (7) |
| 17 | GND | Ground | 18 | DCLK | PIXCLK |
| 19 | GND | Ground | 20 | BLANK# | HREF |
| 21 | GND | Ground | 22 | HSYNC | N/C |
| 23 | N/C | User Defined | 24 | VSYNC | N/C |
| 25 | N/C | 12CDAT | 26 | GND | Ground |

Table 9.4  CCIR -601 Connector Signals

# 10. Operating Parameters

Napalm AIC must perform to full operating specification within the following operating parameters:

10.1 Voltage Tolerances

Voltage tolerances will range +/- 10% on all supply voltages utilized.

10.2 Ripple

Voltage ripple will range +/- 10% on all supply voltages utilized from 0Hz – 500MHz.

10.3 Temperature

Operating:		0 – 70 degrees C

Non-operating:	-25 – 125 degrees C

10.4 Humidity

Condensing:		20% - 80%

Non-condensing:	10% - 90%

# 11. System Support

11.1 Target Platform Compatibility and Support Technology Matrix

Napalm AIC must operate correctly configured within a broad variety of processor and core logic technologies. The following table provides an overview of these matrices:

| Core Logic | Processor |
| --- | --- |
| Intel 440BX | Pentium II (1), Celeron (3), Pentium III (2) |
| Intel 440ZX | Pentium II (2), Celeron (3), Pentium III (1) |
| Intel 440LX | Pentium II (2), Celeron (3), Pentium III (1) |
| Intel 440EX | Pentium II (2), Celeron (3), Pentium III (1) |
| Intel i820 | Pentium II (2), Celeron (3), Pentium III (1) |
| Intel i810 Whitney | Celeron |
| Via MVP3 | Pentium MMX, AMD K6, K6-2, K6-III, K7, Cyrix M2 |
| ALi Alladin V | Pentium MMX, AMD K6, K6-2, K6-III, K7, Cyrix M2 |
| ALi Alladin Pro II | Pentium MMX, AMD K6, K6-2, K6-III, K7, Cyrix M2 |
| SiS 5601 | Pentium MMX, AMD K6, K6-2, K6-III, K7, Cyrix M2 |
| SiS 5591/5592 | Pentium MMX, AMD K6, K6-2, K6-III, K7, Cyrix M2 |

It should also be noted that Via, ALi, and SiS are developing core logic devices to support Slot 1 based processors.

# 12. Software Support

12.1 BIOS

- Note: For all Windows 2000 requirements, 3dfx will provide support when Windows 2000 becomes available to the market

- A Flash utility for the BIOS must support flashing BIOS ROMs in multi-monitor configurations.  It needs to support flashing multiple Banshees, multiple Napalms, or even the possibility of multiple Napalms and Banshees.  The flash utility should be able to know the board type, and stop/prevent the user from flashing one board type with another board types ROM.  An over ride option should also be provided just in case.  Future versions of the Flash utility must be cross platform.

- The BIOS will be required to read the ROM to make sure the correct TV standard will boot up.

- All software will denote the company name as 3dfx with the lower case “d.”

12.1.1 VESA Support

|  | Windows 98 | Win2000* | Windows NT |
| --- | --- | --- | --- |
| Napalm AIC provides full compliance for VESA VBE 3.0 |  |  |  |
| Napalm AIC must provide full compliance for VESA DDC |  |  |  |
| Napalm AIC must provide full compliance for VESA DPMS |  |  |  |
| Napalm AIC must provide compliance for EDID as defined in Product MRD |  |  |  |
| Napalm AIC must provide compliance for supported ACPI States |  |  | N/A |
| Napalm AIC includes compliance for VESA GTF |  |  |  |

* Note: when available to the market

12.1.2 Additional Features

BIOS supports an 8 x14 font for graphics modes, or a TSR must be provided

132 Column mode support in BIOS (25, 43, 50 & 60)

BIOS must support Vital Product Data including Serial Number per WFM 2.0 Requirements

BIOS must be Flashable

BIOS must default to VGA mode 03h when no monitor is attached to the 15pin Video out D-Connector

12.2 Operating System Support Matrix

Support for the following operating systems is required:

Table 12.2  OS Support Matrix

| OS Support | Win98 | Win2000* | Win95 | WinNT 4.0 | BeOS |
| --- | --- | --- | --- | --- | --- |
| Full optimization | 1 2D/D3D | 4 2D/D3D | 2 2D/D3D | 2 2D | TBD |
| WHQL | 1 | 4 | 3 | 2 | TBD |

12.2.2 Languages (drivers & documentation)

| Language Requirements Table: Localized Drivers, Utilities and Tested w/ Localized version of OS | Priority Group for Driver localization | Win 2000* | NT 4.0 | Win 98 | Win 95 | CD Installer | On-line Manual | Print Manual | Web Release | CD Booklet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| English (US) | A | 3 | 2 | 1 | 4 | ML, Asia | ML, Asia | OEM, ML , & Asia | ML |  |
| English (UK) | A | 3 | 2 | 1 | 4 | ML, Asia | ML, Asia | ML, Asia | ML | - |
| French (FR) | A | 3 | 2 | 1 | 4 | ML | ML | ML | ML | - |
| Italian (IT) | A | 3 | 2 | 1 | 4 | ML | ML | ML | ML | - |
| German (GR) | A | 3 | 2 | 1 | 4 | ML | ML | ML | ML | - |
| Spanish (SP) | A | 3 | 2 | 1 | 4 | ML | ML | ML | ML | - |
| Dutch (NL) | A | 3 | 2 | 1 | 4 | ML | ML | ML | ML | - |
| Swedish (SE) | A | 3 | 2 | 1 | 4 | ML | ML | ML | ML | - |
| Brazilian Portuguese (BR) | A | 3 | 2 | 1 | 4 | TBD | TBD | ML | - | - |
| Traditional Chinese (TW) | A | 3 | 2 | 1 | 4 | Asia | Asia | Asia | - | - |
| Simplified Chinese (CH) | A | 3 | 2 | 1 | 4 | Asia | Asia | Asia | - | - |
| Korean (KR) | A | 3 | 2 | 1 | 4 | Asia | Asia | Asia | - | - |
| Japanese (JP) | A | 3 | 2 | 1 | 4 | Asia | Asia | Asia | - | - |
| Norwegian (NO) | A | 3 | 2 | 1 | 4 | - | - | - | - | - |
| Danish (DK) | A | 3 | 2 | 1 | 4 | - | - | - | - | - |
| Finnish (FI) | A | 3 | 2 | 1 | 4 | - | - | - | - | - |
| Arabic (AR) | A | 3 | 2 | 1 | 4 | - | - | - | - | - |
| Portugal Portuguese (PT) | A | 3 | 2 | 1 | 4 | - | - | - | - | - |
| Thai (TH) | B | 3 | 2 | 1 | 4 | - | - | - | - | - |
| Russian (RU) | B | 3 | 2 | 1 | 4 | - | - | - | - | - |
| Polish (PL) | B | 3 | 2 | 1 | 4 | - | - | - | - | - |
| Hebrew (IL) | B | 3 | 2 | 1 | 4 | - | - | - | - | - |
| Czech (CS) | B | 3 | 2 | 1 | 4 | - | - | - | - | - |
| Hungarian (HU) | C | 3 | 2 | 1 | 4 | - | - | - | - | - |
| Slovenian (SL) | C | 3 | 2 | 1 | 4 | - | - | - | - | - |
| Greek (GK) | C | 3 | 2 | 1 | 4 | - | - | - | - | - |
| Turkish (TK) | C | 3 | 2 | 1 | 4 | - | - | - | - | - |

- Priority A: Required for initial launch (Retail & OEMs)

- Priority B: Per additional specific OEM requirements

- Priority C: Will not be localized

- CD Installer = installation program (for drivers, see sect 12.4) on Retail CD shipped with drivers

- On-line Manual  = translated user’s guide shipped on Retail CD.

- Print Manual = printed manual included in retail and OEM boxes.

- CD Booklet = 12-pages, printed, color, saddle-stitched.

- OEM manual = 8-pages per language, printed, black-&-white, saddle-stitched.

- ML manual = Multi-lingual manual for the regions of Europe, Canada, Australia, S. America, etc.;  defined as (8 pages language specific + 4 pages for cover), printed, B&W, saddle-stitched.

- Asia Manual = language-specific (8 pages language specific + 4 pages for cover), printed, B&W, saddle-stitched.

- 3dfx Web Releases:  Specific INF installation of all drivers for Win9x, WinNT (2D, Glide, DirectX, OGL ICD) should be in kits that will be WHQL certified.  The web releases should be wrapped up in a self-extracting “EXE” executable.  Branding, artwork and text strings are TBD.

- Porting to WinNT will be dependent on availability of localized versions of WinNT.  (i.e. Chinese WinNT, Traditional & Simplified is currently not available).

Gold Master CD references:

- ML (Multilingual) CD: English, French, Italian, German, Spanish, Swedish, Dutch, Brazilian Portuguese

- Asia CD: English, Japanese, Korean, Simplified Chinese, and Traditional Chinese

- All CD requirements will be specified in the appropriate SRDs (Software Requirement Document) and EP-001 (Engineering Process for Gold Master CDs), before actually being released for production (example, RT1.0 SRD, and EP-001 RT1.0)

12.2.3 Features

### Table 12.2.3   Feature Requirements

| Support Requirements Table | Win2000* | NT 4.0 | Win98 | Win95 |
| --- | --- | --- | --- | --- |
| Multiple Display Support (two cards per system) | 2 | 1 | 2 | N/A |
| Dynamic Color Depth | 1 | 2 | 1 | 4 |
| Resolution Changes | 1 | 2 | 1 | 4 |
| Refresh Rate Changes | 1 | 2 | 1 | 4 |
| DirectX 6.x | 1 | N/A | 1 | 2 |
| Previous Direct X (5.x) | N/A | N/A | 1 | 2 |
| Future Direct X (7.x) | 1 | N/A | 1 | 3 |
| Support for PC 98/99 Requirements – Including ACPI | 1 | 2 | 1 | 4 |
| GAMMA control | 1 | 3 | 1 | 2 |
| Intel Streaming SIMD (Vendor Supplied) | 1(D3D/OGL) | 2 (OGL) | 1 (D3D/OGL) | N/A |
| AMD 3D NOW (Vendor Supplied) | 1(D3D/OGL) | 2 (OGL) | 1 (D3D/OGL) | N/A |
| Service/OEM Release | 1 | 2 | 1 | 4 |

12.2.4 Legacy

| OS Support | OS/2 4.X | 0S/2 3.X | Win 3.11 | WinNT 3.51 |
| --- | --- | --- | --- | --- |
| Legacy (Contractor Supplied, English only) | 4 | 3 | 1 | 2 |

- OS/2 modes and refresh rates default to the BIOS adapter settings.

12.2.4.1 Table of Driver Modes for OS/2 4.x and OS/2 3.x

| Resolution | Aspect Ratio | Refresh Rates (Hz) Vertical Frequency | Color Depth (bpp) | Comments |
| --- | --- | --- | --- | --- |
| 640x480 | 4:3 | - | 8,16,24 |  |
| 800x600 | 4:3 | - | 8,16,24 |  |
| 1024x768 | 4:3 | - | 8,16,24 |  |
| 1280x1024 | 5:4 | - | 8,16,24 |  |

- The OS/2 Gradd Driver does not support refresh rate selection, refresh rates default to the BIOS adapter settings.

12.2.4.2 Table of Driver Modes for Windows 3.11

| Resolution | Aspect Ratio | Refresh Rates (Hz) Vertical Frequency | Color Depth (bpp) | Comments |
| --- | --- | --- | --- | --- |
| 640x480 | 4:3 | 60,72,75 | 8,16 |  |
| 800x600 | 4:3 | 60,72,75 | 8,16 |  |
| 1024x768 | 4:3 | 60,66,72,75,76 | 8 |  |
| 1024x768 | 4:3 | 60,70,72 | 16 |  |
| 1280x1024 | 5:4 | 60,68, 75,76 | 8 |  |

12.2.4.3 Table of Driver Modes for Windows NT 3.51

| Resolution | Aspect Ratio | Refresh Rates (Hz) Vertical Frequency | Color Depth (bpp) | Comments |
| --- | --- | --- | --- | --- |
| 320x200 |  | 70,85 | 8,16 |  |
| 320x240 |  | 60,72,85 | 8,16 |  |
| 400x300 |  | 60,72,85 | 8,16 |  |
| 512x384 |  | 60,72,85 | 8,16 |  |
| 640x400 |  | 70,85 | 8,16 |  |
| 640x480 | 4:3 | 60,72,75,85,100,120 | 8,16,24 |  |
| 800x600 | 4:3 | 60,72,75,85,100,120 | 8,16,24 |  |
| 1024x768 | 4:3 | 60,70,75,85,100,120 | 8,16,24 |  |
| 1280x1024 | 5:4 | 60,75,85,100 | 8,16,24 |  |
| 1600x1200 | 4:3 | 60,65,70,75,85 | 8,16,24 |  |

- Note: All refresh rates in BOLD will use pre-calculated GTF timings

12.3 Feature Control Applications:

Table 12.3  Feature Control

| 3dfx/Edge Controls Phase I = OS focus Win98/95 Phase II = OS focus Win98/95, plus WinNT Phase III = OS focus Win98/95, plus Win2000 | Phase | Win 2000* | NT 4.0 | Win98 | Win95 |
| --- | --- | --- | --- | --- | --- |
| Master Control Module | I | 1 | 2 | 1 | N/A |
| Video Control Module | II | 1 | 2 | 1 | 4 |
| TV Out Module | I | 1 | 2 | 1 | 4 |
| Digital Out Module | I | 1 | 2 | 1 | 4 |
| Information / Test Module | I | 1 | 2 | 1 | 4 |
| Smart Installer Module (Basic) | II | 1 | 2 | 1 | 4 |
| Smart Installer Module (Full) | III | 1 | 2 | 1 | N/A |
| Gamers Edge Module | II | 1 | N/A | 1 | 2 |
| Advanced Tools Module | II | 1 | 2 | 1 | N/A |

- Edge Controls (3dfx Tools) Phase I will be completed for 11/99 Retail Launch, WinNT Feature Controls will be completed in Phase II.

- Porting to WinNT will be dependent on availability of Windows Chinese NT (Traditional & Simplified).

| Visual Reality | Win2000* | NT 4.0 | Win98 | Win95 |
| --- | --- | --- | --- | --- |
| Napalm AIC must be compatible with supported Multimedia Product Platforms via Visual Reality | 1 | - | 1 | 2 |

| Over-clocking Notification Utility | Win2000* | NT 4.0 | Win98 | Win95 |
| --- | --- | --- | --- | --- |
| Napalm AIC must be provide notification to users who over-clock the Napalm beyond the recommended operating frequency per appropriate SRD | 1 | - | 1 | 2 |

| Un-Installer Utility | Win2000* | NT 4.0 | Win98 | Win95 |
| --- | --- | --- | --- | --- |
| Napalm AIC must be able to detect and clean out prior board driver files per appropriate SRD | 1 | - | 1 | 2 |

12.4 Compatibility

| GUI Requirements Table | Win2000* | NT 4.0 | Win98 | Win95 |
| --- | --- | --- | --- | --- |
| A 2D driver with full hardware accelerations | 1 | 2 | 1 | 4 |

| ### Direct Draw | Win2000* | NT 4.0 | Win98 | Win95 |
| --- | --- | --- | --- | --- |
| A HAL (Hardware Abstraction Layer) driver must provide full hardware acceleration of DirectX 2D & Video applications | 1 | N/A | 1 | 2 |

| ### Direct3D Requirements Table | Win2000* | NT 4.0 | Win98 | Win95 |
| --- | --- | --- | --- | --- |
| Direct3D driver must provide hardware acceleration of DirectX 3D | 1 | N/A | 1 | 2 |
| Graphics controllers that support full AGP must support the non-local graphics memory capabilities of DirectX | 1 | N/A | 1 | 2 |
| Driver must support Intel Streaming SIMD (Vendor Supplied) | 1 | N/A | 1 | 2 |
| Driver must support AMD 3D Now (Vendor Supplied) | 1 | N/A | 1 | 2 |
| Driver must support Cyrix 3D Now (Vendor Supplied) | 1 | N/A | 1 | 2 |
| Driver must support WAIT FOR VSYNS DISABLE | 1 | N/A | 1 | 2 |

| OpenGL Requirements Table | Win 2000* | NT 4.0 | Win98 | Win95 |
| --- | --- | --- | --- | --- |
| An OpenGL ICD/driver must provide full hardware acceleration of OpenGL applications | 1 | 2 | 1 | 3 |
| The OpenGL driver must pass the OpenGL compliance tests available from the Open GL committee | 3 | 1 | 3 | 4 |
| Driver must support Intel Streaming SIMD (Vendor Supplied) | 1 | 2 | 1 | N/A |
| Driver must support AMD 3D Now (Vendor Supplied) | 1 | 2 | 1 | N/A |
| Driver must support Cyrix 3D Now (Vendor Supplied) | 1 | 2 | 1 | N/A |
| Driver must support WAIT FOR VSYNC DISABLE | 1 | 3 | 1 | 2 |

| ### VGA Mode Support Requirements Table | Win2000* | NT 4.0 | Win98 | Win95 |
| --- | --- | --- | --- | --- |
| The driver must support graphics modes 0Dh-13h in a windowed DOS session | 1 | 2 | 1 | 4 |
| The driver must support all VGA modes in a full screen DOS box | 1 | 2 | 1 | 4 |

| ### Video Players Requirements Tables | Win2000* | NT 4.0 | Win98 | Win95 |
| --- | --- | --- | --- | --- |
| Driver must support hardware acceleration of all AVI & MPG files and provide frame rates that meet or exceed PC 98 specs | 1 | 2 | 1 | 4 |
| Driver must support hardware acceleration for Video for Windows Media Player and ActiveX Media Player | 1 | 2 | 1 | 4 |
| Compatibility with MPEG-2 SW DVD players (section 6.5) | 1 | N/A | 1 | 2 |
| Kernel Mode VPE support will be provided | 1 | N/A | 1 | 2 |
| An MPEG-2 Video CODEC (SW Player) will be provided to Interface to Visual Reality Interface | 1 | N/A | 1 | 2 |

| Installation Requirements Table | Win2000* | NT 4.0 | Win98 | Win95 |
| --- | --- | --- | --- | --- |
| All files must be able to be installed using an INF file | 1 | 2 | 1 | 4 |
| All files will adhere to the 8.3 DOS naming convention. | 1 | 2 | 1 | 4 |
| Driver will provide a mechanism for obtaining driver version information | 1 | 2 | 1 | 4 |
| A method of installation (de-installation) must be provided that requires no user interaction (SILENT INSTALL) | 1 | 2 | 1 | 4 |
| Drivers and controls must be remotely installable per Edge Tools MRD | 1 | 2 | 1 | 4 |
| Installation must recognize and discard other vendor drivers and controls per Edge Tools MRD | 1 | 2 | 1 | 4 |
| An unattended installation of drivers must be possible during the installation of the OS (MSBatch-Win95 or UnAttend.Txt-WinNT) | 1 | 2 | 1 | N/A |

- See localized table (sect 12.2.2) for language requirements for CD Installer.

| Configuration Requirements Table | Win2000* | NT4.0 | Win98 | Win95 |
| --- | --- | --- | --- | --- |
| Context sensitive help must be provided for all configurations, windows, dialogs and property sheets | 1 | 2 | 1 | 3 |
| Driver must support the ability to select refresh rates, resolutions and color depths using the mechanisms provided by the OS. If the OS has no mechanism, an additional application must be provided | 1 | 2 | 1 | 3 |
| A configuration application or registry setting must allow the ability to over-ride accelerations for 2D, 3D & video in case of driver, app or other problems. | 1 | 2 | 1 | 3 |

| ### Dynamic EDID / GTF – Timings | Win2000* | NT4.0 | Win98 |
| --- | --- | --- | --- |
| Driver should implement a Optimal Mode method | 1 | 2 | 1 |

# 12.4.1 Interpreting EDID

The other area that needs implementation is interpreting the EDID.  Many things are stored in the EDID.  There is a feature byte that contains the GTF flag and the preferred timing flag.  If the monitor asserts the GTF flag, it is capable of GTF timings and must have a valid range descriptor in the detailed timing section of the EDID.  If the preferred bit is set a button will be available to set that mode. There are three timing sections.

Established Timings: This field consists of 17 bits with each one corresponding to a particular mode.  Although some monitor manufacturers use this field to indicate timings that they have pre-adjusted, the EDID specification says that the bits can be set if the user can adjust the monitor to display that particular mode.

Standard Timings: The timings here are for VESA standard discrete monitor timings that are not included in section one. The same issue exists here about whether or not these modes are pre-adjusted for a centered display.

Detailed Timings: This section has four blocks. These blocks can hold a detailed timing, several strings and range descriptors.

For non-GTF monitors: Optimal is defined as the highest refresh rate for the resolution that exists in any of the three timing sections. When determining what the maximum refresh rate allowed for a given mode is, we will rely on the actual horizontal and vertical rates of the 3dfx standard modes and compare with the range descriptor (if present) and/or the rates in the registry from the monitor .INF file.

For GTF monitors: Optimal is defined as the highest of either the refresh rates in the EDID or the highest GTF pre-calculated mode that is inside the range of the monitor’s range descriptor.  When determining what the maximum refresh rate allowed for a given mode, rely on the actual horizontal and vertical rates of the 3dfx  standard modes and the pre-calculated GTF modes and compare with the range descriptor and/or the rates in the registry from the monitor .INF file.

Allow the user to select any refresh rate directly, but limit Optimal to the above conditions.

### ACPI Requirements Table

|  | Win2000 | NT4.0 | Win98 | Win95 |
| --- | --- | --- | --- | --- |
| Napalm AIC will support all controller enabled power management states | 1 | N/A | 1 | N/A |

# 13. Performance

13.1 Benchmark Suite

Table 13.1  Benchmark Support

| All tests run with 85Hz default refresh rate | Target Perf. Intel 500MHz PIII, 440BX Null Driver | Target Perf. Intel 500MHz PIII, 440BX V3-2000 | Target Perf. Intel 500MHz PIII, 440BX V3-3000 |
| --- | --- | --- | --- |
| Win98 / Win2000 - 3D WB99/00 (1024x768) (16bpp, 16bit Z, Triple Buffer Flip Mode) | Null – TBD | Closest to Null | Closest to Null |
| Win98 / Win2000 - 3D WB99/00 (1024x768) (32bpp, 16bit Z, Triple Buffer Flip Mode) | n / s | n / s | n / s |
| Win98 – BUSINESS / HIGH-END WINMARK 99 (1024x768) | Null – TBD | Closest to Null | Closest to Null |
| Win98 – BUSINESS / HIGH END WINMARK 99 (1280x1024) | Null – TBD | Closest to Null | Closest to Null |
| Win98 / WinNT 4.0 / 5.0- BUSINESS / HIGH-END WINMARK 99 (1024x768) | Null – TBD | Closest to Null | Closest to Null |
| Win98 / WinNT 4.0 / 5.0- BUSINESS / HIGH END WINMARK 99 (1280x1024) | Null – TBD | Closest to Null | Closest to Null |
| CGW GAME GAUGE 2 (IN FPS) (800x600x16BPP) | Null – TBD | Closest to Null | Closest to Null |
| 3D MARK 99Max - 800x600 16BPP/16-BIT Z, DOUBLE BUFFERED | Null – TBD | Closest to Null | Closest to Null |
| ViewPerfDX03 (10x7; 12x10) | Null – TBD | Closest to Null | Closest to Null |

- Note: The null driver estimates to be calculated

# 14. Quality / Reliability / Compatibility

14.1 DPPM

DPPM levels will be specified for pre-production and full production as follows:

Table 14.1  DPPM Levels

| Production Level | DPPM |
| --- | --- |
| Production ramp | 1500 |
| Full production | 500 |

It is assumed these DPPM levels will be measured through lot audit methodologies.

14.1.2  DPPM Reduction Strategies

DPPM reduction may be realized through extensive use of ICT, thorough functional test suites, and quality control audits.

14.2  MTBF

MTBF is specified as a minimum of 87,660 hrs or 10 years of continuous operation.

14.3  H/W Compatibility

Engineering will use and own/update the following test matrices (MS XLS spreadsheets) to ensure compatibility of all hardware requirements:

| File | Description |
| --- | --- |
| Island_invt.xls | Complete computer systems specifying processor and motherboard (for OEMs and Retail) |
| International Test Matrix 0.00.xls | International test matrix specifying HW, OS, graphics and monitor |
| BIOS-Games.xls | BIOS Games per modes and color depth |
| Motherboards_list1.xls | All motherboards test matrix (chipset, bus speed, jumper settings) |

14.4  S/W Compatibility

Engineering will use and own/update the following test matrices (MS XLS spreadsheets) to ensure compatibility of all software requirements:

| File | Description |
| --- | --- |
| Napalm Test Matrix 0.00.xls | All game titles (publisher, developer) specifying API, modes, and color depth |
| Allgames.xls | All game titles bug list |
| Napalm Project Win95 Matrix.xls | Microsoft Win95 WHQL test matrix |
| Napalm Project Win98 Matrix.xls | Microsoft Win98 WHQL test matrix |
| NT 40 xx-xx-99.xls | Microsoft WinNT 4.0 WHQL test matrix |
| Napalm Project WinNT 5 Matrix.xls | Microsoft WinNT 5.0 (Win2000) WHQL test matrix |
| OpenGL V3-xx-xx-99.xls | OpenGL ICD test matrix |

# 15. Certifications / Compliance

15.1 USA

FCC Class B limits for emissions certification and markings.

15.2 ROW

This product must meet CE certifications and markings.

This product must meet C-TICK certifications and markings.

# 16. Manufacturing Considerations

Napalm AIC must meet or exceed all requirements for optimal manufacturing at the site in Mexico.

16.1 Component Packaging:

548 PBGA

16.2 New Technologies

New technologies associated with Napalm AIC include digital flat panel monitor support via the DFP connector and high-speed Panel Link TMDS serial digital data.

16.3 Test Coverage

The product is acceptable for release to volume manufacturing when a minimum of 95% functional test coverage is attained. The product must also have at least 95% ICT coverage to be considered released to production.

16.4 Test Plan

# 17. Documentation

17.1 Manuals

See section 12.2.2 for localization requirements for all documentation. All software/documentation will denote the company name as 3dfx with the lower case “d”

# 18. Packaging

18.1 OEM

Packaging must meet requirements set forth by the individual OEM receiving the product.

18.2 Channel

All products shipped into the Channel markets will utilize ‘white’ box configurations unless otherwise stipulated by an individual customer.   Package will include the following:

18.3 Retail

All products shipped into the Retail markets will utilize new ‘branded’ box configurations unless otherwise stipulated by an individual customer.

Table 18.1  Napalm  Packaging contents

| Country | Part # | Box Type | Clam Shell | Driver CD | Color Manual English | User’s Guide B/W | FCC/CE C-Tick | Warranty Card | 10 Year Warr. Sticker | 3 Year Warr. Sticker | Lifetime Warr. Sticker | Mkg. Insert | Lang. Sticker | Soft DVD | Game Bundle |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Size & Style of PKG |  | 14½”x3”x9 ½” Corrugated Cardboard – Flap location TBD | Large US/Euro Small Asia | Mylar Sleeve | 4¾”x4¾” Multiple Page Saddle Stitch | 5½”x8½” White paper stock – Saddle Stitch | 4”x6” White paper stock printed front & back | 4 7/8”x5 ½” In folded format. Quad-fold w/last fold scored for Mail in Card | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| US | 110-xxxx-xxx | US/UK | Large | ML CD | Yes | None | yes | US Life | No | No | No | TBD | - | TBD | TBD |
| UK | 111-xxxx-xxx | US/UK | Large | ML CD | Yes | None | yes | Euro 10 year | Yes | No | No | TBD | - | TBD | TBD |
| Canada | 113-xxxx-xxx | French | Large | ML CD | Yes | ML | yes | US Life | No | No | TBD | TBD | - | TBD | TBD |
| North Europe | 114-xxxx-xxx | North Euro | Large | ML CD | yes | ML | yes | Euro 10 year | No | No | No | TBD | - | TBD | TBD |
| South Europe | 115-xxxx-xxx | South Euro | Large | ML CD | yes | ML | yes | Euro 10 year | No | No | No | TBD | - | TBD | TBD |
| France | 116-xxxx-xxx | French | Large | ML CD | yes | ML | yes | Euro 10 year | No | No | No | TBD | - | TBD | TBD |
| Japan | 112-xxxx-xxx | Japan | Small | Asia CD | yes | Japanese | yes | Asian 3 year | No | TBD | No | TBD | Asia (if US box used | TBD | TBD |
| PRC | 117-xxxx-xxx | PRC | Small | Asia CD | yes | Simplified Chinese | yes | Asian 3 year | No | TBD | No | TBD | Asia (if US box used | TBD | TBD |
| Asian | 118-xxxx-xxx | Asian | Small | Asia CD | yes | Traditional Chinese | yes | Asian 3 year | No | TBD | No | TBD | Asia (if US box used | TBD | TBD |
| Korea | 119-xxxx-xxx | Korea | Small | Asia CD | yes | Korean | yes | Asian 3 year | No | TBD | No | TBD | Asia (if US box used | TBD | TBD |
| G1 White box | 110-xxxx-028 | White box | None | ML CD | no | Eng. OEM | yes | US Life | No | No | No | No | No | No | No |
| Euro White box | 111-xxxx-208 | White box | None | ML CD | no | ML | yes | Euro 10 year | No | No | No | No | No | No | No |

ML CD = Multi-lingual : English, Dutch, French, German, Italian, Spanish, and Swedish ONLY!

Asia CD = English, Traditional Chinese, Simplified Chinese, Korean, and Japanese ONLY!

ML (User’s Guide) = French, Italian, German, Spanish

Asean = Singapore, Malaysia, Thailand, Philippines, and Indonesia

Marketing Inserts – includes Magazines, other 3dfx products, etc.