Post-Silicon Validation Plan

Napalm

Revision 1.0

June 17, 1999

| Abstract: | This document is the post-silicon validation plan for Napalm |
| --- | --- |

1.	Objective	4

2.	Required Software	4

2.1	Initialization Code	4

2.2	Other Software	4

3.	Required Hardware	5

PCI Testing	6

3.1	PCI Configuration Testing	6

3.2	Register Access	6

3.3	PCI Interrupts	6

3.4	ACPI Testing	7

3.5	subSystem and  subVendor Information	7

3.6	Write Combining	7

4.	Baseline Device Operation	7

4.1	Default PLL output	7

4.2	Register Testing	8

4.3	Memory Integrity	8

5.	VGA Unit Testing	8

5.1	VGA Bring Up	8

5.2	Elpin Systems  Test Suite	8

5.3	Elpin Systems VGA Core Test	9

5.4	QAFE Video Test	9

5.5	VESA Standards VGA tests	9

5.5.1	VESA BIOS Extensions	9

5.6	Basic Acceptance for 3D Application Performance	10

5.7	Run EGAVGA.EXE	10

5.8	PC Tech VGA Compatibility Test 1.0	10

5.9	DPMS	11

5.10	VGA Test v2.0 - DMU Pro	11

5.10.1	VGA Mode Switching Tests	11

5.10.2	Additional DOS Games Tests	11

6.	Extended Video Mode Testing	12

6.1	Windows bring up (Mode 0x12)	12

6.2	Windows Bring up (Extended Modes)	12

6.3	Hardware Cursor	12

6.4	Pixel Format and chroma keying at some sample video refresh rates	12

6.5	Tile addressing	14

6.6	CLUT bypass and select	14

6.7	Different resolutions and 2X mode	14

6.8	Video Overlay	14

6.8.1	Dynamic test	14

6.8.2	Scaling (bilinear and point sample)	15

6.8.3	Filters (2x2 box, 4x1 tap)	15

6.9	Swap Buffer Tests (Overlay and desktop multiple buffering)	15

6.10	TV OUT	16

6.11	DDC Test	16

7.	2D Testing	16

7.1	Internal Diagnostics	16

7.1.1	Test write-only operations with block-write disabled	16

7.1.2	2 Test write-only operations with block-write enabled	17

7.1.3	3 Test blts	17

7.1.4	4 Rerun 1-3 with different strides	17

7.1.5	Rerun 1-3 with increased primitive sizes	18

7.1.6	Run performance tests	18

8.	3D Testing	18

8.1	Internal Diagnostics	19

8.2	3D Applications	19

8.3	VID_CLK	19

9.	IO Characteristics	20

9.1	DC Tests	20

9.1.1	Leakage of output/input buffers:	20

9.2	POWER NOISE & GROUND BOUNCE	21

9.3	RELIABILITY	21

10.	Power Characteristics	21

11.	Operating Condition Tests	21

12.	AGP Testing	21

12.1	Diagnostic Support	21

12.2	Latency Measurements	21

13.	ROM Interface	21

14.	Command FIFO	22

15.	Comparative Benchmarking	Error! Bookmark not defined.

16.      SCANLINE INTERLEAVING & ANTI-ALIASING	25

- APPLE 1555 SUPPORT	25

18.      FLAT PANEL SUPPORT	25

19.      PLATFORM TESTING	25

# Objective

The objective of this document is to describe the post-silicon validation and test plan for 3Dfx VLSI chips.  The goals of the test plan are to:

- Identify the software and hardware requirements.

- List the features to be tested and how they will be tested.

# Required Software

## Initialization Code

Code to be used by software to initialize the device should be found in the  Programmers Guide.

## Other Software

Testing will require certain pieces of software.  VGA games that test compatibility are not listed here but can be obtained from the SW QA group.  The type of software used fits into three categories:

- Diagnostic test software (denoted D)

- Utilities. (denoted U)

- Bench Marking Programs  (denoted B)

- Required API Operating System or Driver Layer (denoted R)

This table lists the software required for bring up:

| Software | Version | D/U/B/R | Source | Purpose |
| --- | --- | --- | --- | --- |
| PCI.EXE |  | D/U | Intel |  |
| Elpin Systems Test |  | D | ELPIN |  |
| QAFE |  | D |  |  |
| VBETEST.EXE |  | D |  |  |
| WINTECH.EXE |  | D |  |  |
| VGAHDW |  | D |  |  |
| EGAVGA.EXE |  | D | PC Magazine |  |
| RENTST20.EXE |  | D |  |  |
| PARANOID.EXE |  | D |  |  |
| PC Tech |  | D |  |  |
| DPMS.EXE |  | D |  |  |
| VGA Challenge |  | D |  |  |
| Korean DOS |  | D |  |  |
| Japanese DOS |  | D |  |  |
| FRACTINT |  | D |  |  |
| Winbench 96 |  | B |  |  |
| WinStone 96 |  | B |  |  |
| PCBENCH | 8.0 | B |  |  |
| 3Dbench |  |  |  |  |
| REPLAY.EXE |  | D | 3Dfx Interactive |  |
| FIGHT.EXE |  |  | 3Dfx Interactive |  |
| FLY.EXE |  |  | 3Dfx Interactive |  |
| SOFTICE |  | U |  |  |
| NNANSI.COM |  | U |  |  |
| CLOCK Utility |  | U | 3Dfx Interactive |  |
| Flash Utility |  | U | 3Dfx Interactive |  |
| Memory Dump |  | U | 3Dfx Interactive |  |
| ior/iow |  | U | 3Dfx Interactive | I/O access at DOS prompt |
| MS DOS | 6.0,7.0 |  | Microsoft |  |
| VGA BIOS |  | R | Elpin Systems |  |
| GUI Registers |  | U | 3Dfx Interactive | GUI register programming |

# Required Hardware

- WinIce

- P6 Adapter

- P5 Adapter

- Host System

- Systems for:

- WIN 95 (w/ Mono setup)

- WIN NT (w/ Mono setup)

- WIN 98 (w/ Mono setup)

- 2D Diags (2 headed setup)

- 3D Diags (2 headed setup)

- VGA/BIOS debug (w/ Mono setup)

- Digital Storage Scope (for analog work)

- Wavecrest for PLL / DAC characterization

- Logic Analyzer (for general bus analysis)

- PII CPU Dissembler

- PCI probe card

- AGP2X/4X probe card

- Bring up Board

- AGP2X/4X Analyzer

- PCI / AGP extender cards

- Monitors

- DDC Capable

- Two high resolution P817

- Monochrome

- Foxfire II PCI compatibility test card

- Vector scope to validate TV.

# PCI Testing

## PCI Configuration Testing

This test should be conducted twice, once using a PCI card (33Mhz), and once using an AGP card  (66Mhz).  Using a monochrome card in one slot, boot the system such that the monochrome is the primary display (through system set up).  Be sure the system is configured to boot to DOS.  Test Napalm configuration from PCI.EXE.

Confirm that the default configuration table is as shown.

| Default Configuration Table (33Mhz PCI, Multimedia, FAST Device) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x (low nibble of Register Offset) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| x : | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | A | B | C | D | E | F |
| 0x: | 00 | 00 | 00 | 00 | 00 | 00 | 90 | 00 | 01 | 00 | 00 | 04 | 00 | 00 | 00 | 00 |
| 1x: | 00 | 00 | 00 | FE | 08 | 00 | 00 | FE | 01 | FF | FF | FF | 00 | 00 | 00 | 00 |
| 2x: | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 12 | 34 | 56 | 00 |
| 3x: | 00 | 80 | FF | FF | 60 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 05 | 01 | 00 | 00 |
| 4x: | 01 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 1F | 00 | 00 | 00 |
| 5x: | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
| 6x: | 01 | 00 | 21 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
| 7x: | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
| 8x: | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
| 9x: | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
| Ax: | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
| Bx: | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
| Cx: | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
| Dx: | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
| Ex: | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
| Fx: | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |

Confirm the following:

- Writeable registers are byte writeable

- Appropriate bits in each register are writeable and have the desired effect.

- Device Rev is correct

## Register Access

Verify access to 2D, 3D, command, VGA and initialization registers are working.  The simplest way of doing this is via an in house I/O read write utility.

## PCI Interrupts

Avenger supports interrupts on INTA# when enabled.  The Windows Driver supports IRQ’s to do Direct Draw Page Flipping.  Enable IRQ’s using the Window’s Driver and verrify that the INT Handler is called.  Verify that the IntrCtrl status is as expected.

| PCI Interrupts | IRQ Enabled | IRQ Disabled |
| --- | --- | --- |
| IRQ Generated | Yes | No |
| IntrCtrl Correct | Yes | No |
| Handler Executed | Yes | No |
| IRQ Reset | Yes | No |

## ACPI Testing

ACPI works by:

- setting PCI Command register [4] to zero to disable IO and Memory Mapped IO,

- setting  PCI PM Power to D3 or (3).

This will be tested two ways.  The first is to use PCI.exe to set the command register to zero and then PM Power to 3.   Pm Power will  be restored and then PCI Command to three.  The system will be restored by an in house program.

The second is to run the test on a ACPI compliant motherboard.  This is accomplished using a Intel Seattle BX Motherboard and the Windows Driver to Power Down and restore the device.

| State Change | Monitor Powers Down | Monitor Powers Up | SSID Held | SSID Reset |
| --- | --- | --- | --- | --- |
| D0  D3 | Yes | No | Yes | No |
| D3  D0 | Yes | No | Yes | No |

## subSystem and  subVendor Information

Read and write the sub vendor and subsystem ID.  Confirm that the sub Ids are loaded from the BIOS under all BIOS sizes settings.  Confirm that the Sub Ids are writeable through the configuration space when the write enable bit is set.

## Write Combining

Use the Perl script and the Windows driver to enabled Write Combining in the Memory Type Range Registers for P6, PII, PIII, and AMD processors.   The Perl scripts can be used to get a automated result whereas the Windows driver is a visual result.

| Processors | P6 | PII | PIII | AMD |
| --- | --- | --- | --- | --- |
| Perl Script | Yes | Yes | Yes | No – Unsupported |
| Windows Driver | Yes | Yes | Yes | Yes |

# Baseline Device Operation

This section describes what portions of the chip must operate correctly before any unit may be tested.

## Default PLL output

Do the PLLs initialize to the proper frequencies (or wiggle at all?)

|  | Frequency |
| --- | --- |
| GRXCLK | 50 |
| VIDCLK | 50 |

## Register Testing

The purpose of this section is to verify that base line register access works for each section.  This does not include the full register read/ write diagnostics listed in other portions.  It does include simple access to each section to determine if it is functional or not.

Initialization registers

VGA registers

2D registers

3D registers

Command Registers

## Memory Integrity

The purpose of this test is to verify that the memory interface is sound for both linear and tiled memory. Memory configuration for different amounts and types of SGRAM and SDRAM should be tested and modified for optimal performance.

Important: The memory interface must yield good results.  If this does not happen, tests results from other sections may be deemed ambiguous.

In addition, memory stability should be tested through each of the CPU to memory paths, as indicated in the following chart:

A0000 - BFFFF

MEMBASE1

YUV

3DLFB

In order to verify that the CPU to Memory path is good, the following methods should be used:

- Read Modify Write Test, (copy  data at address to address+1 iterative)

- Block Copy Test (PCI bursts, for high bandwidth transfers)

see FMEMTEST.C from /devel/sst1/diags/TOOLS for other tests that apply.

# VGA Unit Testing

## VGA Bring Up

## Elpin Systems  Test Suite

The Elpin VGA Core - Version 5 Beta test suite contains DOS executables, bitmap images for comparison, and helper libraries for the DOS tests.

## Elpin Systems VGA Core Test

Run the ELPIN systems core test and report any identified failures.

## QAFE Video Test

Execute the QAFE Video tests and record the results.

## VESA Standards VGA tests

### VESA BIOS Extensions

Run VBETEST.EXE (version 5.1a) - Check VBE for 2.0 or later

Select Interactive Visual Compliance tests.   Run 4-32 bit per pixels modes.

| 4 bit per pixels | 800x600x4bpp |
| --- | --- |
| 8 bit per pixels | 320x200x8bpp |
|  | 320x240x8bpp |
|  | 400x300x8bpp |
|  | 512x384x8bpp |
|  | 640x400x8bpp |
|  | 640x480x8bpp |
|  | 800x600x8bpp |
|  | 1024x768x8bpp |
|  | 1280x1024x8bpp |
| 16 bit per pixels | 320x200x16bpp |
|  | 320x240x16bpp |
|  | 400x300x16bpp |
|  | 512x384x16bpp |
|  | 640x400x16bpp |
|  | 640x480x16bpp |
|  | 800x600x16bpp |
|  | 1024x768x16bpp |
| 24 bit per pixel | 320x200x24bpp |
|  | 320x240x24bpp |
|  | 400x300x24bpp |
|  | 512x384x24bpp |
|  | 640x400x24bpp |
|  | 640x480x24bpp |
|  | 800x600x24bpp |
|  | 1024x768x24bpp |
|  | 1280x1024x24bpp |
| VBE/PM | On |
|  | Stand-By |
|  | Suspend |
|  | Off |

## Basic Acceptance for 3D Application Performance

Included are the Ziff Davis' 2D Winbench 99 tests used for verification of Direct Draw and Windows GDI functionality, Ziff Davis' 3D Winbench 99 which measures D3D functionality and competitiveness, and Game Gauge 1 numbers, which verify Glide, D3D, and  OpenGL functionality for a variety of "real-world" applications.

## Run EGAVGA.EXE

Key: 	1: Color Bar Test

2: Characters

3: Attributes

4: Line Pattern

P: Pass

F: Fail

| Mode | Resolutions |
| --- | --- |
| 0h | 40x25x16 text |
| 01h | 40x25x16 text |
| 02h | 80x25x16 |
| 03h | 80x25x16 |
| 04h | 320x200x4 |
| 05h | 320x200x4 |
| 06h | 640x200x2 |
| 07h (Mono disp) | 80x25x2 |
| 08h | 160x200x16 |
| 0Dh | 320x200x16 |
| 0Eh | 640x200x16 |
| 0Fh | 640x350x2 |
| 10h | 640x350x16 |
| 11h | 640x480x2 |
| 12h | 640x480x16 |
| 13h | 320x200x256 |

## PC Tech VGA Compatibility Test 1.0

BIOS: Alternate Select

BIOS: Character generator

BIOS: Cursor operation

BIOS: Data Area compatibility

BIOS: Graphics input/output

BIOS: Light Pen Support

BIOS: Mode support

BIOS: Multiple page

BIOS: Palette/DAC

BIOS: Save / Restore Video State

BIOS: Screen scrolling

BIOS: Text input/output

Hardware: Attribute registers

Hardware: DAC registers

Hardware: General registers

Hardware: Graphics registers

Hardware:CRTC registers

Hardware: Sequencer registers

## DPMS

Run DPMS.exe in DOS and use DMPS.SCR screen saver in Windows.

DPMS.exe

DPMS.SCR

## VGA Test v2.0 - DMU Pro

| IBM BIOS Modes | Text Mode/ 40x25 character |
| --- | --- |
| IBM BIOS Modes | Text Mode/40x25 character |
| IBM BIOS Modes | Text Mode/40x25 character |
| IBM BIOS Modes | Text Mode/80x25 |
| IBM BIOS Modes | Text Mode/80x25 |
| IBM BIOS Modes | Text Mode/80x25 |
| IBM BIOS Modes | Text Mode/80x25 |
| IBM BIOS Modes | Text Mode/80x25 |
| IBM BIOS Modes | Graphics Mode/320x200x4 |
| IBM BIOS Modes | Graphics Mode/640x200x2 |
| IBM BIOS Modes | Graphics Mode/320x200x16 |
| IBM BIOS Modes | Graphics Mode/640x200x16 |
| IBM BIOS Modes | Graphics Mode/640x350x2 |
| IBM BIOS Modes | Graphics Mode/640x350x16 |
| IBM BIOS Modes | Graphics Mode/640x480x2 |
| IBM BIOS Modes | Graphics Mode/640x480x16 |
| IBM BIOS Modes | Graphics Mode/320x200x256 |
| Multiple fonts |  |
| Screen Blanking |  |
| Undocumented Registers | Processor Latches |
| Undocumented Registers | Attribute Register State |
| Undocumented Registers | Attribute Index |

### VGA Mode Switching Tests

Test switch to all video modes.

### Additional DOS Games Tests

Look for fallout or misbehavior.

Quake

Doom

Captain Keen

# Extended Video Mode Testing

- Bring up of  8/16 BPP 640x480 mode is high priority.

- Windows driver in High Res is necessary.

see BARS.BAT, GRIDS.BAT, FF.C and VIDTEST.C in /devel/sst1/diags/TOOLS

## Windows bring up (Mode 0x12)

Windows should be brought up in mode 0x12 (default VGA mode with Windows standard drivers).  With MS Plus! installed, move windows around, looking for speed degradation and memory fallout.  Test Paint and DOS box applications for the same symptoms.

Mode 0x12

Move windows

Paint Test

DOS Test

## Windows Bring up (Extended Modes)

Extended Modes

Hardware Cursor

Video Overlay

## Hardware Cursor

HW Cursor

MS Cursor

X11 Cursor

Occluded

Partially Occluded

## Pixel Format and chroma keying at some sample video refresh rates

- 640x480x60Hz (vid_clk_2 = 25MHz in 1X mode) Chroma key disabled

| Overlay/desktop | 8bpp | 16bpp | 24bpp | 32bpp |
| --- | --- | --- | --- | --- |
| Undithered RGB16 |  |  |  |  |
| Dithered RGB16 |  |  |  |  |
| YUYV422 |  |  |  |  |
| UYVY422 |  |  |  |  |

Chroma key enabled (chroma key set to a color so that part of the desktop shows through the overlay)

| Overlay/desktop | 8bpp | 16bpp | 24bpp | 32bpp |
| --- | --- | --- | --- | --- |
| Undithered RGB16 |  |  |  |  |
| Dithered RGB16 |  |  |  |  |
| YUYV422 |  |  |  |  |
| UYVY422 |  |  |  |  |

- 1280x1024x73Hz  (vid_clk_2 = 135MHz in 1X mode)

Chroma key disabled

| Overlay/desktop | 8bpp | 16bpp | 24bpp | 32bpp |
| --- | --- | --- | --- | --- |
| Undithered RGB16 |  |  |  |  |
| Dithered RGB16 |  |  |  |  |
| YUYV422 |  |  |  |  |
| UYVY422 |  |  |  |  |

Chroma key enabled (chroma key set to a color so that part of the desktop shows through the overlay)

| Overlay/desktop | 8bpp | 16bpp | 24bpp | 32bpp |
| --- | --- | --- | --- | --- |
| Undithered RGB16 |  |  |  |  |
| Dithered RGB16 |  |  |  |  |
| YUYV422 |  |  |  |  |
| UYVY422 |  |  |  |  |

- 1600x1200x85Hz (vid_clk_2 = 115MHz in 2X mode)

Chroma key disabled (Please put Pass-P or Fail-F along with PRS # below)

| Overlay/desktop | 8bpp | 16bpp | 24bpp | 32bpp |
| --- | --- | --- | --- | --- |
| Undithered RGB16 |  |  |  |  |
| Dithered RGB16 |  |  |  |  |
| YUYV422 |  |  |  |  |
| UYVY422 |  |  |  |  |

Chroma key enabled (chroma key set to a color so that part of the desktop shows through the overlay)

| Overlay/desktop | 8bpp | 16bpp | 24bpp | 32bpp |
| --- | --- | --- | --- | --- |
| Undithered RGB16 |  |  |  |  |
| Dithered RGB16 |  |  |  |  |
| YUYV422 |  |  |  |  |
| UYVY422 |  |  |  |  |

## Tile addressing

Tile Addressing

Desktop surface reside in tile space

Overlay surface reside in tile space

Desktop surface in linear & overlay in tile

Desktop surface in tile and overlay in linear.

## CLUT bypass and select

CLUT bypass and select

Bypass the CLUT for the desktop and/or the overlay

Not bypass the CLUT, and select bank 0/1 for the desktop and the overlay

## Different resolutions and 2X mode

- Equipment: a 1600 x 1200 monitor

- Features to be tested:

| Freq. (Hz) | 60 |  | 65 |  | 70 |  | 75 |  | 80 |  | 85 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 640x480x8bpp |  |  | P |  | P |  | P |  | P |  | P |  |
| 640x480x16bpp | P |  | P |  | P |  | P |  | P |  | P |  |
| 640x480x24bpp | P |  | P |  | P |  | P |  | P |  | P |  |
| 800x600x8bpp | P |  | P |  | P |  | P |  | P |  | P |  |
| 800x600x16bpp | P |  | P |  | P |  | P |  | P |  | P |  |
| 800x600x24bpp | P |  | P |  | P |  | P |  | P |  | P |  |
| 1024x768x8bpp | P |  | P |  | P |  | P |  | P |  | P |  |
| 1024x768x16bpp | P |  | P |  | P |  | P |  | P |  | P |  |
| 1280x1024x8bpp | P |  | P |  | P |  | P |  | P |  |  |  |

*** Not tested in BIOS, must test via Window 95/VGA in order to set to different frequencies.

Note: How do I modify existing diags/tests for the above matrix?

## Video Overlay

### Dynamic test

Video Overlay (Dynamic Test)

Move the overlay window around

Move the cursor around

Stretch up & down the overlay Window to an extent that it is no smaller than the original overlay

### Scaling (bilinear and point sample)

Video Overlay (Scaling- bilinear and point sample)

Bilinear scale up with overlay pixel format = RGB16 and YUV422

Point Sample scale up with overlay pixel format RGB16 and YUV422

Point sample scale up in resolution 1600 x 1200

### Filters (2x2 box, 4x1 tap)

Video Overlay (Filters – 2x2 box, 4x1 tap)

3D frames in dithered RGB565 enable above filter

3D frames in undithered RGB565 enable above filter

## Swap Buffer Tests (Overlay and desktop multiple buffering)

Swap Buffer Tests (Overlay & Desktop muliple buffering)

Overlay double buffering with sync to vsync enabled

Overlay double buffering with sync to vsync disabled

Desktop bufer swapping (see below)

- Features to be tested:

- Desktop buffer swapping

desktop "swap" workaround:

while (currentOverlayStartAddress (h/w reg read) != lastIssuedOverlayStartAddress (driver global))

; /* wait */

direct write desktop start address register w/ new display address

queue new display address (new front buffer) thru command fifo (to overlay start address register)

queue swap buffer command thru command fifo

lastIssuedOverlayStartAddress = new display address

## TV OUT

- With the TV out capable bring up board check the composite and Svideo capability of the card.

- Use a Vector scope to check the video signal quality

## DDC Test

Check the signal quality of the DDC signals.  With an appropriate test program, check that you can reliably read the EDID data from a compliant monitor.  Check that the timing meets the DDC specification.

# 2D Testing

Testing of the 2D section will be accomplished in various ways.  Initially the device will be brought up in a two headed system. Once Windows drivers are available, 2D may also be tested with standard Windows tests and/or programs.

## Internal Diagnostics

These diagnostics consist of specific tests generated in house to test the feature set. These diagnostics should provide information about the basic working condition of the part as well as indicate what performance is achievable.

A list of the diagnostics to be run can be found in the mustpass_gui, list from the source tree.

The diags listed below must be run multiple times with various parameters.  The parameters to be varied are listed below.  The *, #, or ## symbols listed with the diags should be replaced with numbes from the tables.

| Dst Pixel Format | -u* |
| --- | --- |
| 8 BPP | 1 |
| 16 BPP | 3 |
| 24 BPP | 4 |
| 32 BPP | 5 |

| Tiled/Linear Dst Surface | -y# |
| --- | --- |
| Linear | 0 |
| Tiled | 1 |

| Tiled/Linear Src/Dst Surface | -y## |
| --- | --- |
| Linear/linear | 0 |
| tiled/tiled | 1 |
| Linear/tiled | 2 |
| tiled/linear | 3 |

### Test write-only operations with block-write disabled

The BLOCK_WRITE_THRESHOLD environment variable should not be defined.

#### Test:

Write-only ops w/ blk-wr disabled

Rectangle Fills

Solide lines

Stippled lines

Polygons

### 2 Test write-only operations with block-write enabled

The BLOCK_WRITE_THRESHOLD environment variable should be set to 2.

#### Test:

Write-only ops w/ blk-wr enabled

Rectangle Fills

Solide lines

Stippled lines

Polygons

### 3 Test blts

#### Test:

Blt tests

Blts

Stretch blts

Host Blts

### 4 Rerun 1-3 with different strides

For 24bpp and 8bpp, try strides that are odd, maybe 507 for instance.

#### Test write-only operations with block-write disabled

8 bpp and 24bpp try odd stride like 507

Write-only ops w/ blk-wr disabled

Rectangle Fills

Solid lines

Stippled lines

Polygons

#### Test write-only operations with block-write enabled

8 bpp and 24bpp try odd stride like 507

Write-only ops w/ blk-wr enabled

Rectangle Fills

Solide lines

Stippled lines

Polygons

#### Test Blts

8 bpp and 24bpp try odd stride like 507

BLT test

Blts

Stretch blts

Host Blts

### Rerun 1-3 with increased primitive sizes

Increase the -t for each diag to 500-2000.

#### Test write-only operations with block-write disabled

Write-only ops w/ blk-wr disabled

Rectangle Fills

Solid lines

Stippled lines

Polygons

#### Test write-only operations with block-write enabled

i

Write-only ops w/ blk-wr enable

Rectangle Fills

Solide lines

Stippled lines

Polygons

#### Test Blts

increase –t for each diag to 500-2000

BLT test

Blts

Stretch blts

Host Blts

### Run performance tests

Performance testing using 2D Windows performance regressions run by the Performance group.

# 3D Testing

Like the 2D section, testing of the 3D section will be brought up in a two headed system. Once Windows drivers and/or GLIDE is available, 3D may also be tested with other external programs.

## Internal Diagnostics

These diagnostics consist of specific tests generated in house to test the feature set. These diagnostics should provide information about the basic working condition of the part as well as indicate what performance is achievable.

A list of the diagnostics to be run can be found in the mustpass_fbi, and mustpass_trex lists from the source tree.

All 3D diags Run

## 3D Applications

These programs test standard operations via an API.

Extensive SW QA-ing of Glide and D3D games

## VID_CLK

This test should be conducted to indicate the operating characteristics of the VID_CLK PLL.  This clock is observable on the VMI_PIXCLK pin.  Note that this clock is actually vid_clk_2, or vid_clk depending on the state of dacMode bit 0.

| Measure | 50 Mhz | 100 Mhz | 200 Mhz | 250 Mhz | 300Mhz | 350 Mhz |
| --- | --- | --- | --- | --- | --- | --- |
| - Theoretical Freq. | - 49.915 | - 99.205 | - 202.500 | - 252.955 | - 297.102 | - 346.023 |
| - M,N,K | - 7, 249, 3 - f91f | - 5, 192, 2 - c016 | - 5, 196, 1 - c415 | - 1, 104, 1 - 6805 | - 2, 164, 1 - a409 | - 1, 143, 1 - 8f05 |
| - Frequency | - 49.75 | - 99.34 | - 202.70 | - 253.16 | - 296.74 | - 343.98 |
| - Phase Jitter | - 400/400 | - 400/400 | - 500/500 | - 600/640 | - 820/880 | - 1160/920 |
| - Duty Cycle | - 52.2/47.8 | - 54.6/45.4 | - 60.8/39.2 | - 60.0/40.0 | - 55.0/45.0 | - 55.0/45.0 |
| - VCO Freq. | - 398.00 | - 397.36 | - 405.40 | - 506.32 | - 593.48 | - 687.96 |

| 0x3c2 [3:2] | Expected Freq. | Measured Freq. |
| --- | --- | --- |
| 00 | 25.175 Mhz | 25.189 |
| 01 | 28.322 Mhz | 28.302 |
| 10 | 50 Mhz | No stable value |
| 11 | PLL |  |

# IO Characteristics

The following test procedure is a pre-liminary document addressing the different tests that

need to be performed. It does not show specific details in some tests because the configuration for the tests need to be worked out.

Possible shorts/opens can be found at wafer sort.

There are three different groups of I/O’s. One is the 5V tolerant group which is  used for PCI/AGP as well as VMI interface; The other one is the regular 3.3V I/O’s used primarily for the SGRAM interface and the final one is the Analog I/O’s.

The different groups of I/O buffers are:

5V tolerant:

pad_bdpci3v5

pad_bd4m3v5

pad_bd8m3v5

pad_bd12m3v5

pad_input3v5

3.3v regular:

pad_bd4m3v

pad_bd8m3v

pad_bd12m3v

pad_input3v

Analog:

pad_wire

pad_wirelf

## DC Tests

### Leakage of output/input buffers:

Look at any leakage due to a 5V power supply.

| 5V tolerant output Buffers (tri-stated) | 3.3V | 4.0V | 4.5V | 5.0V | 5.5V |
| --- | --- | --- | --- | --- | --- |
| Pad_bdpci3v5 | <1uA | <1uA | <1uA | <1uA | <1uA |
| Pad_bd4m3v5 | <1uA | <1uA | <1uA | <1uA | <1uA |
| Pad_bd8m3v5 | <1uA | <1uA | <1uA | <1uA | <1uA |
| Pad_bd12m3v5 | <1uA | <1uA | <1uA | <1uA | <1uA |
| Pad_input3v5 | <1uA | <1uA | <1uA | <1uA | <1uA |

| 5V tolerant output Buffers (in an off condition) | 3.3V | 4.0V | 4.5V | 5.0V | 5.5V |
| --- | --- | --- | --- | --- | --- |
| Pad_bdpci3v5 | <1uA | <1uA | <1uA | <1uA | <1uA |
| Pad_bd4m3v5 | <1uA | <1uA | <1uA | <1uA | <1uA |
| Pad_bd8m3v5 | <1uA | <1uA | <1uA | <1uA | <1uA |
| Pad_bd12m3v5 | <1uA | <1uA | <1uA | <1uA | <1uA |
| Pad_input3v5 | <1uA | <1uA | <1uA | <1uA | <1uA |

## POWER NOISE & GROUND BOUNCE

Measure Digital Power noise and Ground bounce: Two power rails Core,  I/O. Measure Analog Power for stability of power supply voltage. There are three analog rails: DAC, PLL, AGP PLL.

Ground Bounce measurement procedure: Using a Differential Probe connected to two locations. One under BGA the second point 3 inches away, in the Memory area. Both points are connected through a twisted pair wire to the probe. Probe Ground, connected to Chassis ground.

All Power measurements Digital/Analog measured on closest device to the BGA.

Test soft ware: Speedy running full screen, screen to screen copy’s.

## RELIABILITY

Reliability test is in process.  Results will be documented separately and checked into doc Control.

# Power Characteristics

Power Consumption at different operating frequencies (running applications), as well as in ACPI modes.

# Operating Condition Tests

Avenger needs to be tested with different voltages and temperatures to show that it works well under all of the specified conditions.  Check process monitor effect from voltage and temperature.

# AGP Testing

## Diagnostic Support

- Needs System Driver and stand alone tests for this to work.

- Could AGP option in our diags be modified for bench testing?

- Usage of Windows VXD? (Looks Like Windows 98 is our option)

## Latency Measurements

Measure time from packet issued to data returned.  This data is useful for calculating the optimum size of the AGP DATA return FIFO..

# ROM Interface

Issue

- ROM reads happen by default during power up, sub vendor and device ID load.

- Flash Program Test (requires new utility to program image of ROM)

ROM Access during VMI activity

# Command FIFO

| Issue | Test method |
| --- | --- |
| - Frame-Buffer Command FIFO | Verify with silicon diags and with real-world apps. |
| - AGP-based Command FIFO | Verify with silicon diags and with real-world apps. |

# Comparative Benchmarking

# Scanline Interleaving and Anti-Aliasing Support

Check the interface for 4-way & 2-way SLI with AA support in Analog/ Digital modes. Also compare performance.

# Apple 1555 Support

3D rendering, Video Display, and 2D testing.

# Flat Panel Support

Check the LCD centering capability.

# Platform Testing

Avenger needs to be tested on comparable platforms. Chipsets should include Intel, VIA, SIS, and ALI.