Napalm Registry Keys

This document describes the registry keys for the Napalm W9x drivers.  These keys will be ignored if running on Voodoo3.

Registry keys reside in:

# My Computer\HKEY_LOCAL_MACHINE\System\CurrentControlSet\Services\Class\Display\xxxx\D3D\RegKeyNames

| Registry Key | Key Description |
| --- | --- |
| 32BPP_RENDERING | Enables or disables 32bpp rendering. |
| DISABLE_SLI | Disables SLI modes. |
| ENABLE_AA | Enables AA rendering. |
| DIGITAL_SLI_AA | Enables or disabled digital SLI/AA. |
| 2PPC | Enables or disabled 2 pixels per clock. |
| 2PPC_LOG2_BAND_HEIGHT | Selects band height for 2 pixels per clock. |
| GUARDBAND_CLIPPING | Enables or disables guardband clipping. |
| CBC | Selects band control for triangle iterators. |
| SLI_BAND_HEIGHT | Selects band height for SLI modes. |
| SWAPBUFFER_ALGO | Selects swap buffer algorithm for SLI modes. |

32bpp Rendering Control

Name: 32BPP_RENDERING

Default: 1

1: Enable 32bpp rendering.

0: Disable 32bpp rendering.

Implementation:  Registry key is a Boolean which enables or disables 32bpp rendering.  This registry key is for debugging purposes only, and should not be exported to users:

ddsurf.c - ddCanCreateSurface fails Z buffer support in 32bpp

d3init.c - mask off DDBD_32 before this statement:

_D3(d3d).hwCaps = caps to disable 3D rendering in 32bpp

Disable SLI Modes

Name: DISABLE_SLI

Default: 0

1: Disable SLI configuration.

0: No effect on configuration.

Implementation:  When DISABLE_SLI is 1, SLI mode will never be enabled.  When DISABLE_SLI is 0, this registry key is ignored, and SLI mode setup will depend on the number of chips, and the AA setup.  This registry entry provides a means to turn off SLI if this causes the application to become ill behaved.

Enable AA Rendering

Name: ENABLE_AA

Default: 0

1: Enable AA rendering.

0: No effect on rendering.

Implementation:  When ENABLE_AA is 1, AA rendering will be enabled even when not requested by the application.  When ENABLE_AA is 0, this registry key is ignored, and AA rendering setup will depend on the application requesting DDSCAPS_ANTIALIASING.  This registry entry provides a means to enable AA rendering independently from the application.  It also allows AA rendering to be enabled on versions of DirectX that do not support DDSCAPS_ANTIALIASING.

Enable digital SLI/AA

Registry key is a Boolean which enables digital SLI/AA.

Name: DIGITAL_SLI_AA

Default: 1

1: Enable digital SLI.

0: Disable digital SLI.

Implementation: see minivdd\sliaa.c

Note: The Napalm configuration can be completely determined from DISABLE_SLI, ENABLE_AA, and DIGITAL_SLI_AA, in addition to the number of chips present.  For a complete description of Napalm SLI and AA configuration, refer to the Napalm Software Architecture specification.

2 pixel-per-clock Control

Registry key is a Boolean which enables/disables 2 pixel-per-clock 3D rendering mode.

Name: 2PPC

Default: 1

1: Enable 2 pixel-per-clock mode.

0: Disable 2 pixel-per-clock mode.

Implementation: Registry key overrides REND2PIX_PER_CLK flag in ModeList[], we then set _FF(ModeUses2PixPerClkRender) in h3.c. _FF(ModeUses2PixPerClkRender) is used in d6mt.c to set bit(29) of CombineMode to enable or disable 2 pixel-per-clock 3D rendering. When in dual-texturing, disable 2 pixel-per-clock rendering regardless of the registry value.

2 pixel-per-clock Band Height

Registry key contains the log2 of the number of scanlines rendered by each texture unit when 2 pixel-per-clock rendering is enabled.

Name: 2PPC_LOG2_BAND_HEIGHT

Value: Log2 of band height (0..7) to put in RenderMode bit(24..22).

Implementation: TBD

Guardband Clipping Control

Registry key is a Boolean which enables/disables guardband clipping.

Name: GUARDBAND_CLIPPING

Default: 1

1: Enable guardband clipping.

0: Disable guardband clipping.

Implementation: Registry key overrides bit(21) of RenderMode to enable/disable guardband clipping.

Triangle Iterators band control

Registry key specifies the value to put in bit(31, 30) of fbzColorPath register.

Name: CBC

Default: 0

Value: 0=column-of-8, 1=column-of-16, 2=column-of-32, 3=column-of-4

Implementation: Completed by Bob Seitsinger.

SLI band height

Name: SLI_BAND_HEIGHT

Default: 1

Implementation: see minivdd\sliaa.c

Swapbuffer Algorithm

Name: SWAPBUFFER_ALGO

Default: 1

1: use SLI syncin/syncout

0: use vsync

Implementation: Set bit 25 of cfgInitEnable register.

Programmable dither rotation for 16-bit AA

Each chip will have its own values.

Name:	DITHER_MATRIX, DITHER_MATRIX_ALPHA_BLEND,

DITHER_MATRIX_AA, DITHER_MATRIX_ALPHA_BLEND_AA

Default: TBD

Implementation: Bit 12-19 of FogMode register.

Programmable AA sub-sample jitter

Jitter values for AA, each chip will have its own values.

Name:	PRI_BUF_VERTEX_OFFSET_X, PRI_BUF_VERTEX_OFFSET_Y,

2ND_BUF_VERTEX_OFFSET_X, 2ND_BUF_VERTEX_OFFSET_Y

Default: "0.0"

Implementation: Convert string in float format double by calling ddatof(), then the driver converts to 3.4 format and put in bit 0-27 of aaCtrl register.