3dfx Interactive, Inc.

Napalm 3dfx Tools

and

Software Installation Requirements

## Marketing Requirements Document

Revision History

| Revision | Description | By | Date |
| --- | --- | --- | --- |
| 0.1 | Initial description | AF | 11/1/99 |
| 0.2 | Updates | AF | 12/18/99 |
| 0.3 | - Hotkey - Referenced Reid’s phase2spec.doc - Removed DUDE - SLI/AA Configuration Page | AF | 2/28/00 |
| 0.4 | - Removed MAC requirements from this doc…Bryan Speece? - Removed monitor control page - Removed artwork requirements here, referenced them in other document. - Added table of contents page. - Added overclocking/v sync page | AF | 3/31/00 |
| 0.5 | - Added LOD bias (MIP map level of detail) requirements per Scott Sellers to the Advanced Features section - Cleaned up the doc a bit | AF | 6/8/00 |

Table of Contents

| Section | ##### Description |
| --- | --- |
| 1 | Product Description |
| 2 | Control Panel Features Overview |
| 3 | Additional Requirements - Help Files - Artwork - Licensing - Localization |
| 4 | Individual Control Panel Software Requirements |
| 5 | Installation Requirements |

## To Do:

- Gamer’s Edge definition for Rampage

Notes:

- Product Description

What is 3dfx Tools?

What is our goal with 3dfx Tools?

What can you do with it?

Why would users like it?

How does it improve their product experience?

1.1  Brief Description

3dfx Tools is a set of control panels that allows users to change various settings on their 3dfx card.  Each user has specific settings they like to customize for their computer that simple controls do not always allow them to set.  The goal of 3dfx Tools is to allow beginner and advanced users the options they require to maximize their enjoyment of their 3dfx video card.  By providing users with an interface to a vast amount of settings, we are ensuring enjoyment of our product by giving users maximum flexibility of their 3dfx product.

1.2  Outline

The following is a brief outline of how the user can interface with 3dfx Tools

Anti-Aliasing

Color Control

Etc, more tabs

Display Properties

Help File

Etc, more apps

3dfx Hub

3dfx Tray Icon

Start menu

Access to EdgeHub via Taskbar and Start Menu

- Software Feature Overview

2.1 Windows/Mac Required Control Panels

2.1.1  Info Page

The Advanced info page will provide users with access to all information about their 3dfx card including: driver and bios versions, memory configurations, and 3dfx Tools versions.  By showing this information to the user, it can help them determine if any software update is necessary.  In addition to providing the user with system information, 3dfx Technical Support will also benefit with one central location for video card information.  The user will have the ability to print, save, or even email their card information to provide Technical support with as much information as possible to help diagnose the problem.

### 2.1.2  3dfx Advanced Feature

The Advanced Tweak is designed for advanced users to fine-tune their 3dfx card to achieve maximum performance and visual quality.  The user will have many options exposed for tweaking that are often not uncovered in other cards.  But the Advanced Tweak will not segregate new users.  One of the goals of 3dfx is to educate the consumer on 3D graphics.  Therefore, included on this page will be a glossary of definition that will help users learn what each options change and what they can gain changing that option.

### 2.1.3  3dfx Color

The Color Control page will provide the user with the ability to fine tune the RGB settings of their card while viewing a real time bitmap changing.  Users can get the best possible color from their video card in conjunction with their room’s lighting and their monitor.  (Win/Mac)

### 2.1.4  3dfx Anti-Aliasing

This page is used to highlight and control the Anti-Aliasing level of your Voodoo4/5 card.

### 2.1.4  DUDE

DUDE from 3Dfx Interactive transparently updates 3dfx hardware drivers and game patches.   The user is saved the hassle of entering directories, entering the control panel, unzipping, or any other related messiness.

Designed for the “casual” user, DUDE uses one interface, simplifying the installation procedure (browse, winzip, ctrl panel) for the 3Dfx lines of product.  Additionally, this product installs the latest game patches and lists software products that exploit the full power of the user’s 3dfx card, making his complete ownership experience more enjoyable.

The product will be available for download at the 3Dfx website.  In the future, product distribution may be part of the shipping CD, to be included with the Napalm.

### 2.1.5  3dfx TVOut

The TVOut page will allow users to setup and display their standard computer output to display on standard NTSC, PAL, and HDTV TVs.  The user will be able to control output settings from this menu as well as choose which feed to send through (composite, coaxial, RCA).

2.1.6  3dfx Overclocking

The overclocking page will provide reviewers and gamers who want to really push the envelope of their 3dfx card to the maximum.  Users will be able to overclock their board on a sliding scale as well as disable V Sync so they can attain the highest possible framerate from their video card.

### 2.1.7  Monitor Control Page

The Video Control page is an onscreen tool that acts much like the GUI menu screens that are built into most monitors today.  The benefits of using 3dfx Tools’ controls rather than the ones on the monitor are that they are more intuitive and laid out in a straightforward manner, rather than having to use + and – buttons and navigate through many menus.

2.1.8  3dfx DigitalOut

The DigitalOut Page will provide users with an easy to use interface with their DFP monitors can adjust the placement of the screen on it.

2.2 Windows Specific Required Control Panels

### 2.2.1  3dfx Tray Icon

The Taskman on the Taskbar will serve as quick access to all of 3dfx Tools, launch bundled software, and give users access to the standard display properties.  By being on the start menu, users can quickly get to it without having to close down all their windows.

### 2.2.2  3dfx Hub

The central location of all 3dfx Tools will be the 3dfx Hub.  It will act as a one stop application for users to get to all the control panel pages.  It will act much like the Creative BlasterControl in which all control panels features for a card are located under one tab on the Display Properties/Advanced Properties.  Visually and functionally, one place for the all tools is better, and it makes it easier to access the tools from the Taskman/Taskbar.  The 3dfx Hub will be the starting point for all 3dfx Tools.

2.3 Mac Specific Required Control Panels

2.3.1  Fonts

MacOS supports the ability to cache fonts to speed up the desktop.  The Mac portion of the 3dfx Tools control panel must be able to support a slider to change the cache.  The result is an increase in performance leading to a more pleasing user experience.

2.3.2 Resolutions

MacOS allows for hot keys to be defined so the user can change resolutions quickly.  By giving users the option for hot switching, we are bringing ourselves up to par with features offered by the competition.

2.3.3  Visuals

The Visuals page for the Mac control panel will have some additional features to turn on to make the desktop more visually appealing.  These features are not requirements, but make the experience better for the user.

2.3.4  Zoom

The Zoom control panel for Mac Napalm cards will allow users to Zoom to portions of the desktop, allowing them to display at really high resolutions, while at the same time allowing them to highlight portions of the screen they want to see close up with all the details.

- Additional Requirements

3.1  Help Files

3.1.1  Online Help Files (Must Have Features)

- Overview of 3dfx Tools

- Individual Control Panel Page Help Files

  - Right click over each item to get help.

  - Summary of what can be changed on that page

  - Description of each item/button and what features it controls

    - The Advanced Feature page will need definitions for the terms there.

3.1.2  Multimedia tour

- How to install the power supply

- Etc.

3.2  Artwork

#### SEE 3DFX TOOLS ARTWORK REQUIREMENTS DOC

3.3  Licensing

Must have click through licensing agreement for SW Installation.

3.4  Localization

See Product MRD

- Individual Control Panel Software Requirements

4.1 Windows/Mac Required Control Panels

4.1.1 3dfx Info

4.1.1.1  Must Have Features

- All the current features that are used in the Napalm Info page.

- The Save option allows the user to save the information to a file formatted as text or HTML.

- The Print option allows the user to print the information to an installed printer.

- A Button to email this information to help@3dfx.com or some other tech support alias.

  - Must ask the user if they are connected to the Internet.  (Else the rest fails)

  - Must have an input box to allow the user to add an email address.

  - Ask the user if it's ok to send general PC info to technical support.  (Just to make sure people know that we will not take data from their PC without their permission)

- Include the small graphic logo's that were used in the first betas (like above picture).  The little pictures of the video cards, floppy disks and monitors add a little splash to the dull data.

  - 3dfx  Icon for 3dfx Tools version

- Current Display Adapted Information

  - List the information found above

  - Must display the number of chips the user has

- Current Display Driver Informaton

  - Relevant information: driver name, driver date, driver version, etc. DIGITAL SIGNAGE FOR WHQL???? THOM

- Current Monitor Information

  - Monitor Max resolution

  - Driver info from PNP monitor support and MS driver

  - Remove additional monitor listed, just display current monitor

- Previous Display Driver Information

  - Provides the users with a listing of previous drivers installed.  They can figure out which driver worked on their card the best…?

4.1.1.2  Additional Features (not required)

- Any type of test programs that could be beneficial to the user and/or tech-support to help diagnose a sick video card

### 4.1.2 3dfx Advanced Features

4.1.2.1  Must Have Features

- The Tweak page must be backward compatible with all Voodoo3 Series, Velocity Series and Napalm products.  Those options that are not backward compatible must AUTO-HIDE when not detected and the FORMAT of the page should change so that not to look like there are HOLES in the page.  Since this page has so many new options, a new layout may need to be made

- Required Controls

  - OpenGL(Glide) Features

    - Glide Splash Screen

      - Options: Enable/disable

      - Definition: This will turn on and off the 3dfx splash screen that appears before any Glide application is started.  It will remain on to give us a bit of press.

    - Alpha Blending

      - Options: Automatic/Sharper/Smoother

      - Definition: This is the old 16bit vs. 24bit color issue we faced with Voodoo3.  We will include this to keep our 16bit color mode superior to others.

    - 3D Filter Quality

      - Options: Automatic/Normal/High

      - Definition: This control changes the amount of overlay filter used by the card.  The medium setting will use a 4x1 filter, the high setting is a 2x2.

    - MIP Map dithering

      - Options: Disable/Enable

      - Definition: Blends between different MIP map levels of the textures to produce a better looking, continuous texture.

    - LOD Bias Control (for MIP mapping)

      - Options: (Per Scott Sellers)  Values range from –2 to +2, increments of 0.25

      - Definition: LOD bias modifies the calculation of texture level of detail parameter LOD. Often a texture is oversampled or filtered such that the texture is band limited at lower frequencies in one or more dimensions. The result is that texture-mapped primitives appear excessively blurry. LOD bias provides biases in the LOD calculation to compensate for under or over sampled texture images. MIP mapped textures can be made to appear sharper or blurrier by supplying a negative or positive bias respectively.

    - Triple Buffering

      - Options: Disable/Enable

      - Definition: Allocates a 3rd frame buffer.  This can increase performance by allowing the card to render the same time an app performs a task, but it eats up memory.

    - Rendering Color Depth

      - Options: Software Controlled/Force 32 bpp

      - Definition: Forces old Glide apps to render to 32bpp polygons, not textures.)

    - Anti-Aliasing

      - Options: V4 – 0/2 Sample Size

      - Options: V5 – Single Ship/Fastest Performance/2/4 Sample Size

      - Definition: The AA subsample setting.

  - Direct3D

    - Alpha Blending

      - Same as OpenGL/Glide

    - 3D Filter Quality

      - Same as OpenGL/Glide

    - LOD Bias control

      - Same as OpenGL/Glide

    - Anti-Aliasing

      - Same as OpenGL/Glide

- V Sync Enable/Disable Checkbox (WEB RELEASE ONLY, DISABLE IN RETAIL DRIVERS)

  - Defaults to off, click checkbox to disable   IDEAL TO DO THIS HERE, FOR NOW IN OVERCLICKING PAGE

  - Option must be there for DirectX, Glide, and OpenGL

- Help File

  - Help files for every option here accessible on right clicking

- Resent to Previous and Factory Default settings must be there.

4.1.2.2  Additional Features (not required)

### 4.1.3  3dfx Color

4.1.3.1  Must Have Features

- Gamma Controls

  - The color page must be able to control Desktop D3D, Glide/OGL, and Video Overlay gamma through radio buttons

  - Slider to change individual RGB channels or a Lock Gamma checkbox to change all at once

  - Shows a picture being modified by the Gamma level – SEE LOADABLE BITMAP

- Contrast/Brightness

  - The brightness and contrast of the Desktop an Video Overlay selections can be adjusted using the Contrast and Brightness buttons.

  - Shows a picture being modified by the Gamma level – SEE LOADABLE BITMAP

- Customization

  - Customizable Favorite Color settings - This section saves any of the settings changed on this page and allows the user to enter in a custom name for it.

  - User defined schemes can be added and deleted using the Save and Delete buttons. Clicking the Save button, the user will be prompted for a colour scheme name.  On clicking the Delete button, the user will be asked to confirm the delete operation before the scheme is deleted.

  - Color Temp Controls - ie Slider or DropDown Menu - Warm/Cool or Actual Temp like 5000k, 6500k or 9300k.

- Reset To Previous

  - The previously applied color settings can be reapplied by clicking the Reset To Previous button.

- Factory Defaults

  - The factory default color settings can be set by clicking the Factory Defaults button.

- Loadable Bitmap

  - The addition of the color picture with the additional ability to allow the user to load a graphic (.BMP) has been added in this section along with the ability to save the Color Adjustment 'scheme'.

4.1.3.1  Additional Features (not required)

### 4.1.4  DUDE

4.1.4.1  Must Have Features

- DUDE must verify that the driver is still valid, and if not request the user to update the drivers.

- Must support compression for driver packages

- Must be able to detect type of card and driver from the registry

- Must support regression!

  - If the latest drivers are buggy or cause crashes, DUDE must be able to uninstall the driver and replace it with an earlier version (that is kept in a backup directory).

  - This can be in the control panel with an option called “RESTORE PREVIOUS DRIVERS” which can allow users to simply press if they find the current drivers aren’t meeting their needs.

- When connected to 3dfx site for DUDE, window opens

  - DUDE then checks users driver version and compares to 3dfx web version.  If newer version available, proceed to download

  - BUTTON FOR WHAT’S NEW TO SEE WHAT’S NEW IN EACH BUILD

- Transparent installation of drivers and patches once the user has selected the drivers/patches he wants.

  - The user may be prompted for whether he wants to upgrade or not, but will not see directories being created, drivers being install, or applications downloading the files through FTP.

  - The user should get a status bar indicated the status of the download (file size, time remaining, etc) and a status bar for when the drivers/patches are being installed.

4.1.4.2  Additional Features (not required)

- Game patches????

  - Ability to detect games on hard disk and compare to 3dfx server and download new patches??

4.1.4  Anti-Aliasing Page

4.1.4.1  Must Have Features

- Options to control Direct3D and OpenGL/Glide AA settings.

  - Interface

    - A slider to choose the correct setting is the best way to implement this

    - OpenGL/Glide and Direct3D will be separate from each other in case the user wants to choose different values.

  - Values

    - Voodoo4

      - Single Chip/2 Sample

    - Voodoo5

      - Single Chip/Fastest Performance/2 Sample/4 Sample

### 4.1.5  TVOut Page

4.1.5.1  Must Have Features

- All the current features that are used in the Voodoo3/Voodoo4/Voodoo5 TV-out page.  Current TV-out page is shown below.

- Connections

  - List Box to select one of the following: Auto, Composite, S-Video

- Signal type

  - Allows the user to select which signal type they want by country… listed at the very top of this list are the actual signal types so the power user can change the actual signal type on demand.

- “Enable output to TV” CHECKBOX

- “When using TV out, don't use my monitor” CHECKBOX (10 second confirmation is used to make sure mode changes ok)

- Control Buttons - Centering:  Allows the user to move around the video image on the TV.

- Picture Settings

  - Adjust List Box to select the following the modify: Screen Size, Flicker Filter, Color, Brightness

  - A Slide Bar to use with above List box.

- Monitor Image / TV Image: Depending on whether or not the user has selected "Enable output to TV" this feature will show the correct picture to represent which mode they are in.

- Grey out the buttons for standard TV (NTSC/PAL) FIND OUR MORE FROM KYLE

- Software decoding

- Ability to support both 640x480 and 800x600 resolution modes while in TV out mode.  These modes will be selectable via Quick Res. or the standard resolution option via windows control panel.

- OEM: Gateway specific requirements requested see section 2.13.1 for specific information. Mass consumption of this document is only authorized by the removal of the OEM section.

4.1.5.2  Additional Features (not required)

### 4.1.6  Overclocking Page

4.1.5.1  Must Have Features

- Overclocking

  - Activating in the control panel

    - Since WHQL states we cannot ship with a overclocking/v sync utility, we will have a small app that turns on this control panel in the registry.  A small app will just write into the registry and turn it on, making the control panel now appear.

  - Interface

    - A small slider will be present to allow users to slide the clock speed higher to their desired speed.

    - Once they have change speed, they must be prompted to restart in order to reload Windows with the card overclocked.

    - A check/warning utility should be created to warn the user that they are overclocking and force them to approve the new speed.  This way we can at least feel confident that the people that don’t know what they are doing will be saved from crashing.

- V Sync

  - Activating in the control panel

    - Since WHQL states we cannot ship with a overclocking/v sync utility, we will have a small app that turns on this control panel in the registry.  A small app will just write into the registry and turn it on, making the control panel now appear.

  - Interface

    - Checkboxes for Direct3D and OpenGL/Glide

4.1.6.2  Additional Features (not required)

### 4.1.7  Video Control Page

4.1.7.1  Must Have Features

- Must have same functionality of Napalm Video Control Page below.

- Screen Adjustments - Position

  - Position modification via control panel buttons

  - Position modification via keyboard keys

- Screen Adjustment - Size

  - Position modification via control panel buttons

  - Size modification via keyboard keys

- Monitor Mode Info - DDC info

  - Intelligent DDC polling of unknown monitor type.

- Monitor timing and adjustments

- DDC2Bi Polling for Custom Control Options - Some monitors allow the use of DDC2B/I2C commands to be used to modify the screens actual settings.  If polling is successful, all video settings will be usable though simulated buttons.

- CENTER BUTTON FOR ABOVE 4 KEY DIRETIONS SHOULD BE ACT LIKE RESET TO PREVIOUS

- Moving left/right moves to far while up down goes at a reasonable distance

  - Can we fix this so that the movements are similar?

- Current resolution, color depth, frequency, and monitor you are currently displaying at

4.1.7.2  Additional Features (not required)

4.1.8  DigitalOut Page

4.1.8.1  Must Have Features

- All the current features that are used in the V3/V4/V5 DigitalOUT page

.

- The ability to allow the user though the control panel to NOT use scaling when changing to a video size that is below the natural resolution of the monitor.  This image that is now smaller than the actual screen size should also be centered and not just thrown in the upper left hand corner like so many laptops do.

  - (REASON:   So many software products these days switch the mode of the monitor to better suite the program [ie. Games] and sometimes these programs go into a mode that doesn't scale well with our part [say 800x600].)

4.1.8.2  Additional Features (not required)

4.2 Windows Specific Required Control Panels

### 4.2.1  Taskman/Taskbar

4.2.1.1  Must Have Features

- Enhanced Taskbar Icon Functionality

This is the small icon that is located down in the taskbar.  Most of the functionality behind this icon is actually located in the 'Right-Click Menu'.

  - Right-Click-Hold use: Brings up Enhanced Menu

  - Left-Click use: TBA - Use not defined - just left open for expandability

  - Double-Left-Click use: Launch 3dfs Hub, Anti-Aliasing page

- Enhanced Menu

This is the small 'Windows Smart Menu' like menu that is launched when you right click on the taskbar icon.   This will launch the following applications:

DIVIDE UP THE SECTIONS and SEPARATE WITH HORIZONTAL LINE.  PUT THEM IN THIS ORDER AS THEY WOULD APPEAR ON THE ENHANCED MENU

  - Launch 3dfx Hub

  - HORIZONTAL LINE

  - 3dfx Resolutions

    - Choose resolution then color depth.

    - Must approve each setting

  - HORIZONTAL LINE

  - Weblinks

    - www.3dfx.com

    - www.3dfxgamers.com

    - 3dfx Overclocking

  - HORIZONTAL LINE

  - 3dfx Tools Help

  - About 3dfx Tools

  - Exit 3dfx Tools

ACTUAL TASKBAR STARTS DOWN HERE

- 3dfx Tools Preferences (use 2-3 tabs to divide up the below )

  - First tab

    - Checkbox to allow the user to display or not display 3dfx Tools at startup

    - Checkbox to allow the user to display or not display warning message when changing Advanced Tweak menu settings

  - Second Tab

    - Lets the user choose how each page will display under Edge Hub (or the tabs)

    - Possible choices for color schemes/text/background for 3dfx Tools (SECONDARY PRIORITY)

  - Third Tab

    - User defined resolutions

    - User can choose which ones to appear on

  - Fourth Tab

    - Lets the user choose which items to launch on Taskbar/Taskman through checkboxes

    - Choose user defined links (games)

- Side Bar Label - This is the label that is placed on the side of the menu much like Windows 98 is in the blue fading bar.  This is something used just to give the menu a little jaz.

- About ET / Menu - This should be a small dialog showing the standard about info box.

4.2.2.1  Additional Features (not required)

### 4.2.2  3dfx Hub

4.2.2.1  Must Have Features

- BASICALLY IT WILL JUST HAVE ALL THE TOOLS ON IT.  DO BELOW AT A LATER REV RAMPAGE!!

- Main EdgeHub Center that will allow users to have one tab under Display Properties/Advanced Properties.  The user can then click on clipart at the top and “browse” through the sections of 3dfx Tools and edit what they choose to do.

- Double Clicking on the Taskbar/Taskman icon will bring up EdgeHub preferences

- Users must be allowed to change the icons/options for 3dfx Tools page through the Taskbar/Taskman icon by double clicking on the icon

- Users will be allowed to change

  - Skins for EdgeHub

    - Predefined ones

    - Custom ones that allow the user to change

  - The look of the buttons

    - They can be like web links and change from grayed out images to color when the mouse moves over them

    - They can act like buttons and will be sticking out until pressed and then will be depressed in.

  - Background

    - Watermark

      - 3dfx Tools image

      - Product Branding Image

      - Custom Image

    - Color

      - Solid Color

      - Gradient (user can choose two colors)

        - Will go from Left to right

  - Any other options that we may want to change.

4.2.2.2  Additional Features (not required)

4.3 Mac Specific Required Control Panels

4.3.1  Fonts

4.3.1.1  Must Have Features

- Ability to toggle font caching and set its associated on-board memory allocation

  - Sliding bar to choose between Off, 256, 512, 1024, 2048, and 4096, 8192 Kb (or however high until it becomes negligible)

- Meter displaying amount of memory being used for the amount of memory selected above

4.3.1.2  Additional Features (not required)

4.3.2  Resolutions

4.3.2.1  Must Have Features

- Ability to change displayed resolution and corresponding display timings (Must be compatible with Monitors and Sounds Control Panel)

- Ability to set the hotkeys associated with the selection of pre-defined resolutions and display timings at startup

- A test mode to check for all the usable resolutions at a user-supplied horizontal frequency. If a resolution is not displayable the screen remains black. After 15 seconds the previous resolution is reinstated. If a resolution is displayable the user is given 15 seconds to approve the choice before the previous resolution is reinstated.

  - It is believed that this test mode is built into MacOS 8x and higher

4.3.2.2  Additional Features (not required)

4.3.3  Visuals

4.3.3.1  Must Have Features

- Display window contents when dragging: software toggle to activate the display of window contents while dragging. (The Mac would ordinarily substitute a marquee box for the window contents while dragging.)

- Cursor Size: switch toggle to choose between large and normal sized cursors

4.3.3.2  Additional Features (not required)

4.3.4  Zoom

4.3.4.1  Must Have Features

- Ability to assign hot keys and associated magnitudes of Zoom to each hot key (2x, 3x and 4x)

- Radio buttons to set zoom mode to traverse virtual desktops that exceed actual screen resolution or choose to lock desktop to center (ADD UPPER CORNER???)

4.3.4.2  Additional Features (not required)

- Installation Requirements

  - Software Installation Requirements

- Drivers, 3dfx Tools, Bundled Applications, Online Help, Online Registration ETC

5.2  General Installation Requirements

- Minimize user intervention by auto-installing as much of the software as possible

- Installer should install the following: 2D drivers, 3D Drivers, Video drivers (where applicable), 3D Runtime (OGL, D3D, GLIDE), and all components of 3dfx Tools (Macintosh and Linux will not install D3D or Windows only 3dfx tools pages)

- Minimize Reboots necessary to install all 3dfx provided software to one reboot

- All requirements are applicable to Windows, Macintosh and Linux (except where noted)

- Give user the ability to access 3dfx Software from multiple launch points (Start Menu, Quick Install Page, etc)

5.3  Specific Installation Requirements:

- Compliance Tests

- WHQL Compliant: All Windows based installations must pass WHQL (INF Install)

- Must NOT install ANY Microsoft redistributable files.  Must NOT require an NT service pack to run. (Except MSVCRT.DLL)

- Uninstall Process

- Unattended Install: Must be able to do unattended install at OS install time

- Uninstallation: Windows-works per plug n play spec, removes all registry entries and all Voodoo4/5 specific software

- Remote network installation

- License: User must agree to click-thru license agreement, prior to installation

- 8.3 DOS Naming Convention (except Linux and Macintosh)

- Compatible with Multimonitor setup (with 3dfx or other Controller vendor)

- Kits

- CD Only: One installation for all OS’s

- Web Only: Separate Installation for all OS’s

- Online HTML Registration (for all installations, see requirements below)

- 3dfx Program Group added to Start/Programs Menu (see requirements below, Windows Only)

5.4  Installer Components

5.4.1  Quick Install Page Requirements

- Re-Install Voodoo driver

- Re-Install 3dfx Tools

- Re-Install Microsoft  DirectX7 (DirectX8 when available)---Not required on Macintosh or Linux)

- Install 3Deep/True Internet Color

- Guided Tour Link to launch Guided Tour Application (see requirements below)

- View Voodoo4/5 User’s Guide

- Includes HTML user guide

- View Voodoo4/5 Installation

- Includes ability to view the MPEG or AVI video

- Buttons needed

- Print icon

- Back ICON	Back to previous page

- FWD ICON	Go to next page

- Print ICON	Prints all pages of Hardware Installation Instructions

- Exit button

    - Guided Tour Application:

- Answers the following questions: What did I Install, What does it do, How do I get to it?

- Autorun after user reboots from Driver/3dfx Tools installation (user should be able to cancel)

- Should allow user to:

- View all 3dfx tools CTRL Panels (ideally with interactive instruction on how to use)

- How to use online Help

- How to use DUDE driver/patch updates

- View Taskman Taskbar

- View ToolHub

- View Start Menu (or similar on Mac/Linux) Program Group (let users know that this is where launch buttons for bundled apps go)

- Describes what OGL and D3D, and Glide drivers do

- Windows Only: Napalm Program Group Available under Start Menu

- Links to www.3dfxgamers.com

- Guided Tour ends prompting users to visit www.3dfxgamers.com

5.4.3  Start Menu Program Group

- Launch 3dfx Hub

- Launch 3dfx Tools Manager

- Launch 3dfx Tools Taskman Icon

- Links

- www.3dfx.com

- www.3dfxgamers.com

- 3dfx Overclocking

- Launch Help File

5.4.4  Online Registration

- SEE ERIK JOHNSON’S ONLINE REGISTRATION DOCUMENT

5.4.5  Artwork Requirements

- SEE 3DFX TOOLS ARTWORK REQUIREMENTS DOCUMENT

5.4.6  User Experience

5.4.6.1  CD Installation

- Windows (Linux and Windows should mimic installation, but without use of Plug n Play or INF install)

- Scenario 1:

- No Vodooo4/5 in machine

- CD Placed in machine:

- AutoRun the CD to the "QuickInstall Hardware Installation" page

- User is prompted to watch the Installation Video or view and print the documents

- Once user agrees to license, user should be prompted to turn off machine and install Voodoo4/5 board (user should be notified to leave CD in machine for upcoming SW installation)

- Voodoo4/5 Card should be detected by Plug n Play

- Windows installs “Standard VGA Driver” and asks for Display adapter drivers

- Hit next a couple of times and come to following choices:

- Install Standard VGA adapter, reboot

- Upon reboot, CD needs to either be run by double clicking on Autorun, the CD in explorer, or simply ejecting it and putting it back in.  This will launch dialog to Prompt user with these choices:

- “Install BOTH Voodoo driver and 3dfx Tools

- (RECOMMENDED FOR ALL USERS)

- Install Voodoo driver ONLY

- (NOT RECOMMENDED)”

- If the user selects Install BOTH Voodoo driver and 3dfx Tools, the standard 3dfx Tools setup program is ran.  The Voodoo driver and the Tools are installed.  System reboots

- Upon reboot Online registration is run.

- If the user selects Voodoo driver ONLY, a warning pops up stating the following.

“You must install the 3dfx Tools software to access and control all of your 3dfx card’s exciting new features, like Full-Scene Anti-Aliasing.  Your computer will restart after installing 3dfx Tools and then you will be prompted to register your product online with 3dfx to receive:

- Complete customer support

- Information about upcoming 3dfx products

- Exclusive deals just for registered 3dfx customers!

Do you want to install the Voodoo driver and 3dfx Tools now?

YES 	NO

(Selecting “No” will install the Voodoo driver only)

- If the user selects YES here, they will launch the standard 3dfx Tools setup.

- If the users selects NO, Voodoo driver only will install with no further warning about Installing the tools, online registration

- WE HAVE TO HAVE THE NO OPTION IN ORDER TO PASS WHQL

- Scenario 2:

- Standard INF installation

- No installation of 3dfx Tools or Online registration

- Can we get around this????

5.4.6.2  Installation (assumes this is a driver update, not a fresh installation)

- Step 1: User downloads compressed driver installation (compression standard based on OS (windows=compressed .exe)

- Step 2: User clicks on file to unzip and begin installation

- Should immediately launch setup.exe type installation (without forcing user to click another time)

- A 3dfx/Voodoo4/5 bitmap should come up full screen signaling the beginning of the installation

- Step 3: Drivers and 3dfx Tools are installed

- Step 4: User reboots

- Step 5: User is ready to run product