|  | Application Note Developer Relations 33 Commerce Valley Drive East Thornhill, Ontario Canada L3T 7N6 Tel: (905) 882-2600 extension 6000 Fax: (905) 882-2620 |
| --- | --- |

Subject: ATI Graphic Card Listing			Date:	Jan 15, 1997

P/N:	APPL0016-02

Thank you for your inquiry regarding the capabilities and/or hardware configurations of ATI graphics card. On the following pages you will find a set of quick reference charts which list all retail and the most common OEM product names along with some basic information about the capabilities and hardware.

This is NOT an exhaustive list but does cover the products that applications are most likely to encounter in the installed base. The following product are not covered in this application note:

·	Any graphics products introduced before the VGAWonder series. These include CGA/monochrome cards such as the Graphics Solution, EGA cards such as the EGAWonder and the VIP.

·	Motherboard graphics which use ATI chipsets. These designs are not under the control of ATI and therefore we cannot make any representations in regards to them.

·	OEM products which are sold under another vendor's trade name. The listed OEM product names are either generic or products which the vendor publicly made it known that they where manufactured by ATI. The terms of our contract with other vendors prevent us from disclosing their information.

The information is accurate to the best of our knowledge at this date. ATI may make changes to listed products or introduce new products at any time without notice. The SVGA, mach8 and mach32 family products have not been shipped for some time and therefore this information can be considered to be stable. The mach64 GX family products are currently in its end of life phase and several products are no longer shipped. It is unlikely that any new products based on the mach64 GX will be introduced. ATI is actively developing additional variants of the mach64 chipset with exciting new functionality. There will be a number of new products and using the new chipsets in existing products over the remainer of the year. Please monitor the ATI BBS, Compuserve Forum. Web site and/or Fax Back service for new product announcements.

| Product Name | Bus | Chipset(s) | Memory 2 |  | DAC 5 |  |  | Max Color Depth (bits per pixel) 3 |  |  |  |  |  |  |  |  |  |  |  | H/W 4 |  |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| r = retail o =OEM | Type 1 |  | VGA/SVGA | Accel. |  | Types |  |  | 640 |  | 800 |  | 1024 |  | 1280 |  | 1600 |  |  |  |  |  |  |  |
| SVGA (non-accelerated) Family |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| VGAWonder V3 r | i | 18800-0 | 256/512 |  |  | IMS G176 |  |  | 8 |  | 8 |  | 4 |  |  |  |  |  | chevb |  |  |  |  |  |
| VGAWonder V4 r | i | 18800-1 | 256/512 |  |  | IMS G176 |  |  | 8 |  | 8 |  | 4 |  |  |  |  |  | chevb |  |  |  |  |  |
| VGAWonder V5 r | i | 18800-1 | 256/512 |  |  | IMS G176 |  |  | 8 |  | 8 |  | 4 |  |  |  |  |  | chevb |  |  |  |  |  |
| VGAWonder + V1 r | i | 28800-2 | 256/512 |  |  | IMS G176 |  |  | 8 |  |  |  | 4 |  |  |  |  |  | chevi |  |  |  |  |  |
| VGAWonder + 1Mb r VGA 1024D o | i | 28800-4/5 | 256/512/1M |  |  | IMS G176 |  |  | 8 |  | 8 |  | 8 |  |  |  |  |  | chevi |  |  |  |  |  |
| VGAWonder XL r | i | 28800-5 | 256/512/1M |  |  | SC 11483 |  |  | 15 |  | 8 |  | 8 |  |  |  |  |  | chevi |  |  |  |  |  |
| VGAWonder XL24 r VGA 1024 DXL o | i | 28800-5/6 | 256/512/1M |  |  | 20C491 Bt481 |  |  | 24 |  | 15 |  | 8 |  |  |  |  |  | chevi |  |  |  |  |  |
| VGA Basic 16 r | i | 28800-2/4/5 | 256 |  |  | IMS G176 |  |  | 4 |  |  |  |  |  |  |  |  |  | chev |  |  | no SVGA |  |  |
| VGA Edge r | i | 18800-1 | 256 |  |  | IMS G176 |  |  | 8 |  | 4 |  |  |  |  |  |  |  | chev |  |  |  |  |  |
| VGA Edge 16 r | i | 18800-1 | 256 |  |  | IMS G176 |  |  | 8 |  | 4 |  |  |  |  |  |  |  | chev |  |  |  |  |  |
| VGA Integra r | i | 28800-5 | 256/512 |  |  | IMS G176 |  |  | 8 |  | 8 |  | 4 |  |  |  |  |  | chev |  |  |  |  |  |
| VGA 640 o | i | 18800-0 28800-2/4/5 26300 | 256 |  |  | IMS G176 |  |  | 8 |  |  |  |  |  |  |  |  |  | chev |  |  |  |  |  |
| VGA 800 o | i | 18000-0 28800-2/4/5 28300 | 256/512 |  |  | IMS G176 |  |  | 8 |  | 8 |  |  |  |  |  |  |  | chev |  |  |  |  |  |
| VGA 1024 o VGA Charger o | i | 18800 0/1 28800 2/4/5 | 256/512/1M |  |  | IMS G176 |  |  | 8 |  | 8 |  | 8 |  |  |  |  |  | chev |  |  | numerous flavors with different configurations some w/ & w/o mouse |  |  |
| VGA 1024 VLB o | v | 68800 3/6 | 1M d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | chev2 |  |  | a short lived NON-accelerated SVGA compatible with the VGAWonder + 1Mb |  |  |
| VGAStereo F/X r | i | 28800-5 | 256/512 |  |  | SC 11483 |  |  | 15 |  | 8 |  | 4 |  |  |  |  |  | chev |  |  | essentially a XL with a SB comp sound card |  |  |

| Product Name | Bus |  | Chipset(s) |  | Memory 2 |  |  |  |  | DAC 5 |  | Max Color Depth (bits per pixel) 3 |  |  |  |  |  |  |  |  |  |  | H/W 4 |  |  | Comments |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| r = retail o =OEM | Type 1 |  |  |  | VGA/SVGA |  | Accel. |  |  |  | Types |  | 640 |  | 800 |  | 1024 |  | 1280 |  | 1600 |  |  |  |  |  |  |  |
| mach8 Accelerator Family |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8514/Ultra r |  | im |  | 38800 |  |  |  | 512/1M v | IMS G176 |  |  |  |  | 8 |  | 8 |  | 8 |  | 4 |  |  |  |  | 8 |  |  | dual bus 'flippy' |
| Graphics Ultra r |  | i |  | 28800-5 38800 |  | 256/512 d |  | 512/1M v | IMS G176 |  |  |  |  | 8 |  | 8 |  | 8 |  | 4 |  |  |  |  | chev8i |  |  |  |
| Graphics Vantage r |  | i |  | 28800-5 38800 |  | 256/512 d |  | 512/1M d | IMS G176 |  |  |  |  | 8 |  | 8 |  | 8 |  | 4 |  |  |  |  | chev8i |  |  |  |
| VGAWonder GT r |  | i |  | 28800-5 38800 |  | 512 d |  | 1M v | IMS G176 |  |  |  |  | 8 |  | 8 |  | 8 |  | 4 |  |  |  |  | chev8i |  |  | H/W is identical to a Graphics Ultra |
| 8514/Ultra AT o |  | i |  | 38800 |  |  |  | 512/1M v | IMS G176 |  |  |  |  | 8 |  | 8 |  | 8 |  | 4 |  |  |  |  | 8 |  |  | AT bus only |
| 8514/Ultra MCA o |  | m |  | 38800 |  |  |  | 512/1M v | IMS G176 |  |  |  |  | 8 |  | 8 |  | 8 |  | 4 |  |  |  |  | 8 |  |  | microchannel bus only |
| 8514 Vantage AT o |  | i |  | 38800 |  |  |  | 512/1M d | IMS G176 |  |  |  |  | 8 |  | 8 |  | 8 |  | 4 |  |  |  |  | 8 |  |  | AT bus only |
| mach32 Accelerator Family |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Graphics Ultra + r |  | i |  | 68800-3/6 |  | 1M d |  | 1/2M d | 68875 |  |  |  |  | 24 |  | 24 |  | 16 |  | 8 |  |  |  |  | chev82b |  |  |  |
| Graphics Ultra Pro r |  | ie |  | 68800-3/6 |  | 1M v |  | 1/2M v | 68875 |  |  |  |  | 24 |  | 24 |  | 16 |  | 8 |  |  |  |  | chev82b |  |  | VGA could be disabled only ISA has mouse |
| Graphics Ultra Pro r |  | mv |  | 68800-3/6 |  | 1M v |  | 1/2M v | 68875 |  |  |  |  | 24 |  | 24 |  | 16 |  | 8 |  |  |  |  | chev82 |  |  |  |
| Graphics Ultra Pro r |  | p |  | 68800 AX |  | 1M v |  | 1/2M v | 68875 |  |  |  |  | 24 |  | 24 |  | 16 |  | 8 |  |  |  |  | chev82 |  |  |  |
| Graphics Wonder r |  | iv |  | 68800 3/6 |  | 1M d |  | 1M d | Bt481 20C491 82C490 |  |  |  |  | 24 |  | 16 |  | 8 |  | 4 |  |  |  |  | chev82 |  |  |  |
| Graphics Ultra XLR o |  | v |  | 68800 3/6 |  | 1M v |  | 1/2M v | 68830 68875 |  |  |  |  | 16 24 |  | 16 24 |  | 16 16 |  | 8 8 |  |  |  |  | chev82 |  |  | Gateway 2000 both memory & DAC upgradable |
| Graphics Ultra AXO o |  | p |  | 68800 AX |  | 1M d |  | 1/2M d | Bt481 68875 |  |  |  |  | 24 |  | 24 |  | 16 |  | 8 |  |  |  |  | chev82 |  |  | Gateway 2000 |
| VLB mach32-D o |  | v |  | 68800 3/6 |  | 1M d |  | 1/2M d | 68830 20C491 82C490 |  |  |  |  | 16 |  | 16 |  | 16 |  | 8 |  |  |  |  | chev82 |  |  | some with 1M non- upgradable memory |
| PCI mach32-D o |  | p |  | 68800 AX |  | 1M d |  | 1/2M d | Bt481 20C491 |  |  |  |  | 24 |  | 24 |  | 16 |  | 8 |  |  |  |  | chev82 |  |  | some with 1M non- upgradable memory |
| ISA mach32 o |  | i |  | 68800 3/6 |  | 1M |  | 1/2M d/v | 68875 |  |  |  |  | 24 |  | 24 |  | 16 |  | 8 |  |  |  |  | chev82b |  |  | came with or without mouse port |

| Product Name |  | Bus |  | Chipset(s) |  | Memory 2 |  |  |  | DAC 5 |  | Max Color Depth (bits per pixel) 3 |  |  |  |  |  |  |  |  |  | H/W 4 |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| r = retail o =OEM |  | Type 1 |  |  |  | VGA/SVGA |  | Accel. |  | Types |  | 640 |  | 800 |  | 1024 |  | 1280 |  | 1600 |  |  |  |  |
| mach64 Accelerator Family |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Graphics Xpression r | ivp p |  | GX C/D or CT C/D |  | 1/2M d |  | 1/2M d |  | ST 1702 CH8398 20C408 68875 internal |  | 32 |  | 32 |  | 16 |  | 8 |  |  |  | v |  |  |  |
| Graphics PRO TURBO r | ivp |  | GX C/D |  | 2/4M v |  | 2/4M v |  | 68860 B/C |  | 32 |  | 32 |  | 32 |  | 24 |  |  |  | v |  |  |  |
| Win Boost o | ivp p |  | GX C/D or CT C/D |  | 1/2M d |  | 1/2M d |  | ST 1702 CH8398 20C408 internal |  | 32 |  | 32 |  | 16 |  | 8 |  |  |  | v |  |  |  |
| Win Turbo r | ivp |  | GX C/D |  | 2M v |  | 2M v |  | 68860 B/C |  | 32 |  | 32 |  | 16 |  | 8 |  |  |  | v |  |  |  |
| Win Turbo o | vp |  | GX C/D |  | 2/4M v |  | 2/4M v |  | 68860 B/C |  | 32 |  | 32 |  | 32 |  | 24 |  |  |  | v |  | some upgradable to 2M and others to 4M |  |
| Graphics PRO TURBO 1600 r | p |  | GX F |  | 2/4M v |  | 2/4M v |  | RGB 514 |  | 32 |  | 32 |  | 32 |  | 24 |  | 16 |  | v |  | Available Q3 '95 |  |
| Video Xpression r | p |  | 264VT |  | 2M d |  | 2M d |  | internal |  | 32 |  | 32 |  | 16 |  | 8 |  |  |  | v |  |  |  |
| Video Boost o | p |  | 264 VT or 264 VT 2 |  | 2M d |  | 2M d |  | internal |  | 32 |  | 32 |  | 16 |  | 8 |  |  |  | v |  |  |  |
| Video Xpression+ r Video Boost S o | p |  | 264 VT 2 |  | 2M s |  | 2M s |  | internal |  | 32 |  | 32 |  | 16 |  | 8 |  |  |  | v |  |  |  |
| 3D Xpression r | p |  | 3D RAGE |  | 2M d |  | 2M d |  | internal |  | 32 |  | 32 |  | 16 |  | 8 |  |  |  | v |  |  |  |
| 3D Xpression+ r | p |  | 3D RAGE II |  | 2/4M s |  | 2/4 d |  | internal |  | 32 |  | 32 |  | 32 |  | 24 |  |  |  | v |  |  |  |
| 3D Xpression+ PC2TV r | p |  | 3D RAGE II |  | 2/4M s |  | 2/4 d |  | internal |  | 32 |  | 32 |  | 32 |  | 24 |  |  |  | v |  | equipped with ImpacTV video output |  |
| 3D PRO TURBO PC2TV r | p |  | 3D RAGE II |  | 4/8M g |  | 4/8M g |  | internal |  | 32 |  | 32 |  | 32 |  | 32 |  | 16 |  | v |  | equipped with ImpacTV video output |  |
| All-In-Wonder | p |  | 3D RAGE II |  | 2/4M d |  | 2/4M d |  | internal |  | 32 |  | 32 |  | 16 |  | 8 |  |  |  | v |  | equipped with ImpacTV video output, video input and TV tuner |  |

1 i = ISA, e = EISA, m = microchannel, v = VLB, p = PCI

2 -mach8 based products use separate controllers and memory for VGA and for accelerator modes

-mach32 based products share a common pool of memory between VGA and accelerator modes but the VGA hardware can only access 1M

-mach64 based products share a common pool of memory between VGA and accelerator modes and all memory is accessible in either VESA VBE or accelerator modes

- d = dram, v = vram, s = sdram, g = sgram

3 - based on maximum memory installed

- in the case of mach8/32 for accelerator modes only, VGA modes are limited to 8bpp

4 - H/W compatibility c = CGA, e = EGA, h = Hercules, v = VGA, 8 = 8514/a, 2 = ATI 28800, b = Bus mouse port, i = InPort mouse port

5 - See Application Note #10 for more detail