# Maintenance and Installation

This chapter describes opening Intergraph’s desktop and deskside 3D graphics workstations to replace a GLZ1T board set.  It also describes installing a dual screen option in the deskside workstation.

## Static Precautions

Observe the following precautions when working with the internal components of the system.

- Avoid servicing the system in areas with electrostatic problems.  Rugs and carpets, for example, cause static build-up.

- Handle all printed circuit boards as little as possible and by the edges only.  Leave the boards in their protective packaging until you install them.

CAUTION

Discharge personal static charge by touching a bare metal surface on the workstation before handling printed circuit boards.

## Opening the Desktop Workstation

WARNING

Before opening the workstation, turn off the system power.  Leave the power cord connected to maintain ground.  Use caution when removing the cover to avoid injury.

ä To open the desktop workstation:

1.	Pull out the spring-lock plunger on the back panel and turn it clockwise until it locks in the open position.

4715A

ISA I/O

Panel

Spring-lock

Plunger

Figure 2-1.  Desktop Top Cover Removal

2.	Lift up the top cover above the ISA I/O panel and pull up and back to remove the top cover.

3.	Attach an antistatic wrist strap to a non-painted, metallic surface on the workstation or to the ground loop (formed in the metal) above the ISA I/O panel.  Ensure that the elastic loop in the strap snugly contacts your bare wrist.

## Replacing GLZ1T Board Set in the Desktop

This section describes replacing a GLZ1T in a desktop workstation.  The GLZ1T board set consists of two boards (Rasterization Accelerator/Frame Buffer and Texturing Accelerator) joined together as one unit.

ä To replace a GLZ1T board set in the desktop workstation:

1.	Disconnect the cables from the Video and VGA ports on the back panel of the base unit.

2.	Using a quarter-inch nutdriver, remove the screws from the GLZ1T board set.  Retain the screws.

3.	Remove the board set from the workstation.

4745 (new)

GLZ1T

Board Set

Figure 2-2.  GLZ1T Board Set Removal

4.	Install the new GLZ1T board set into the slots from which the previously installed board set was removed.

Note:	The GLZ1T board set resides in PCI slots 2 and 3.

4746

Slot 1

Slot 2

Screws

GLZ1T Board Set

Figure 2-3.  GLZ1T Board Set Installation

5.	Install the screws previously removed to secure the GLZ1T board set to the workstation.

## Completing the Installation for the Desktop

Follow these steps to reassemble the workstation and reconnect the monitor.

ä To complete the installation:

1.	Replace the top cover on the workstation by aligning the tabs on the front of the top cover with the notches behind the faceplate.

2.	Lower the back of the top cover and slide it into place.

3.	Turn the spring-lock plunger to lock the top cover to the base unit.

4.	Connect the monitor cable to the Video Out port in PCI slot 2 as shown in Figure 2-4.

5.	Connect the VGA loopback cable to the VGA In port and to the VGA Out port.

4732A

VGA In PortVGA Out

Port

VGA Loopback Cable

Video Out Port

Figure 2-4.  Monitor Cable and VGA Loopback Cable Connection

6.	If you need to configure the display driver, refer to “Configuring the Video Display” in Chapter 1.

## Opening the Deskside Workstation

WARNING

Before opening the workstation, turn off the system power.  Leave the power cord connected to maintain ground.  Use caution when removing the cover to avoid injury.

ä To open the deskside workstation:

1.	Using a quarter-inch nutdriver, remove the screw from the top cover.  Retain the screw.

4136B + MOD (deleted SCSI connector)

Screw

Figure 2-5.  Deskside Top Cover Removal

2.	Remove the top cover by sliding it toward the back of the workstation.

3.	Remove the footstands.

4.	Remove the two screws from the top of the left side panel.

4636

Screws

Figure 2-6.  Deskside Left Side Panel Removal

5.	Remove the left side panel by grasping the tab between the screw holes.  Then, slide the panel up and out from the workstation.

6.	Lay the workstation on its right side.

7.	Remove the three screws from the PCI access panel on the bottom of the workstation.

4696

Screw

PCI Access

Panel

Screws

Figure 2-7.  Deskside PCI Access Panel Removal

8.	Remove the PCI access panel by sliding it toward the back of the workstation.

9.	Attach an antistatic wrist strap to a non-painted, metallic surface on the workstation.  Ensure that the elastic loop in the strap snugly contacts your bare wrist.

## Replacing GLZ1T Board Set in the Deskside

This section describes replacing a GLZ1T board set in a deskside workstation.  The GLZ1T board set consists of two boards (Rasterization Accelerator/Frame Buffer and Texturing Accelerator) joined together as one unit.

ä To replace a GLZ1T board set in the deskside workstation:

1.	Disconnect the cable(s) from the Video and VGA ports on the back panel of the base unit.

2.	If replacing a secondary GLZ1T board set in a dual screen configuration, disconnect the clock sync cable.

4747

Clock Sync Cable

Secondary

GLZ1T

Board Set

Figure 2-8.  Clock Sync Cable Removal

3.	Using a quarter-inch nutdriver, remove the screws from the GLZ1T board set.  Retain the screws.

4.	Remove the board set from the workstation.

5.	Install the new GLZ1T board set into the slots from which the previously installed board set was removed.

Note:	The primary GLZ1T board set resides in PCI slots 2 and 3.  In a dual screen configuration, the secondary GLZ1T board set should reside in PCI slots 4 and 5.  Refer to Figure 2-9 for board and PCI slot location.

6.	Secure the GLZ1T board set to the base unit using the screws removed previously.

7.	If you replaced a GLZ1T board set in a dual screen configuration, install the clock sync cable.

CAUTION

The clock sync cable installs only one way.  Do not force the keyed connector.  Ensure pin 1 of both cable connectors engages pin 1 on both board connectors.  The black wire connects to pin 1.

## Installing a Dual Screen Option in the Deskside

The Intergraph GLZ1T dual screen option kit provides the following components to upgrade from a single screen to a dual screen configuration.  Verify that you have the following items.

- GLZ1T board set

- Diskette containing the Intergraph GL display driver software

- GLZ1T Hardware User’s Guide

- FIFO status cable (only used if Geometry Accelerator option board is present)

- Clock sync cable

- Antistatic wrist strap

If an item is missing or damaged, notify Intergraph Customer Response Center at 1-800-633-7248 immediately.

ä To install a dual screen option:

1.	Using a quarter-inch nutdriver, remove the blanking plates from PCI slots 4 and 5.  Refer to Figure 2-9 for slot locations.  Retain the screws.

2.	Install the second GLZ1T board set into PCI slots 4 and 5.

4632

PCI Slot 1

PCI Slot 2

PCI Slot 3

PCI Slot 6

PCI Slot 5

PCI Slot 4

Figure 2-9.  Deskside PCI Slots

3.	Install the screws to secure the GLZ1T board set to the workstation.

4.	Install the clock sync cable between the two GLZ1T board sets.  Refer to Figure 2-10.

CAUTION

The clock sync cable installs only one way.  Do not force the keyed connector.  Ensure pin 1 of both cable connectors engages pin 1 on both board connectors.  The black wire connects to pin 1.

4747 (new)

Clock Sync Cable

Secondary

GLZ1T

Board Set

Figure 2-10.  Clock Sync Cable Installation

## Completing the Installation for the Deskside

Follow these steps to reassemble the workstation and reconnect the monitor.

ä To complete the installation:

1.	Install the PCI access panel on the workstation.

2.	Raise the workstation to its normal standing position.

3.	Install the side panel of the workstation.

4.	Install the footstands.

CAUTION

To avoid injury or damage to the equipment, replace the stabilizing footstands before operating the unit.

5.	Install the top cover of the workstation.

6.	Connect the VGA Loopback cable and monitor cable for single monitor configurations as follows:

VGA Loopback:  Connect the VGA Loopback cable to the VGA Out port and to the VGA In port as shown in Figure 2-11.

Single monitor:  Connect the monitor cable to the Video Out port in PCI slot 2.

4709B

Video

Out Port

VGA Out Port          VGA In Port

VGA Loopback

Cable

Figure 2-11.  Single Monitor Cable Connections

7.	Connect the VGA Loopback cable and monitor cables for dual monitor configurations as follows:

VGA Loopback:  Connect the VGA Loopback cable to the VGA Out port and to the VGA In port as shown in Figure 2-12.

Primary monitor:  Connect the monitor cable to the Video Out port in PCI slot 2.

Secondary monitor:  Connect the monitor cable to the Video Out port in PCI slot 4.

4729

VGA Out Port         VGA In Port

Video

Out Port

(Secondary)

Video

Out Port

(Primary)

VGA Loopback

Cable

Figure 2-12.  Dual Monitor Cable Connections

8.	If you need to configure the GL display driver, refer to “Configuring the Video Display” in Chapter 1.