Contents

Preface

Using the Document

Defining Document Conventions

FCC Statement

Safety Information

Cautions

Warnings

Getting Telephone Support

Chapter 1  Getting Started	1

Starting Windows NT	2

Configuring the Video Display	3

Dual-Screen Behavior	4

Gamma Correction	5

Ending the Video Configuration Session	6

Changing the Default Display Driver	6

Turning Off Full Drag	7

Chapter 2  Maintenance and Installation	9

Static Precautions	9

Opening the Desktop Workstation	10

Replacing GLZ1T Board Set in the Desktop	11

Completing the Installation for the Desktop	12

Opening the Deskside Workstation	14

Replacing GLZ1T Board Set in the Deskside	16

Installing a Dual Screen Option in the Deskside	18

Completing the Installation for the Deskside	20

Chapter 3  Driver Installation and Troubleshooting	23

GL Display Driver Installation	23

Troubleshooting	24

Diagnostics	24

Video Display	25

Obtaining a Usable Video Resolution	25

Determining a Defective Unit	27

Chapter 4  Technical Description	29

Features	29

Rasterization Accelerator/Frame Buffer	30

Texturing Accelerator	33

Functions	34

Rasterization Accelerator/Frame Buffer	35

Texturing Accelerator	37

Chapter 5  Hardware Description	39

Rasterization Accelerator/Frame Buffer Components	39

PCIDMA	40

Graphics Engine	40

Resolver	41

Video Memory	42

Video Selector and Mapper	42

Digital to Analog Converter	42

Texturing Accelerator Components	43

Texture Processor	43

Texture Memory	43

Chapter 6  Specifications	45

Drawing Performance	45

3D Performance	45

2D Performance	46

Monitor Resolutions	46

Interfaces	46

Video Port	47

VGA Loopback Port	48

Stereo Port	49

Index	51

Figures

2-1.  	Desktop Top Cover Removal	10

2-2.  	GLZ1T Board Set Removal	11

2-3.  	GLZ1T Board Set Installation	12

2-4.  	Monitor Cable and VGA Loopback Cable Connection	13

2-5.  	Deskside Top Cover Removal	14

2-6.  	Deskside Left Side Panel Removal	15

2-7.  	Deskside PCI Access Panel Removal	16

2-8.  	Clock Sync Cable Removal	17

2-9.  	Deskside PCI Slots	19

2-10.  Clock Sync Cable Installation	20

2-11.  Single Monitor Cable Connections	21

2-12.  Dual Monitor Cable Connections	22

4-1.  	GLZ1T OpenGL Processing Block Diagram	34

4-2.  	Rasterization Accelerator/Frame Buffer Block Diagram	36

4-3.  	Texturing Accelerator Block Diagram	37

5-1.  	Rasterization Accelerator/Frame Buffer Board	39

5-2.  	Texturing Accelerator Board	43

6-1.  	Video Port	47

6-2.  	VGA Loopback Port	48

6-3.  	Stereo Port	49

Tables

1-1.  	GLZ1T Video Configuration Options	3

1-2.  	Gamma Correction Options	5

3-1.  	Video Configuration Problems	25

6-1.  	3D Graphics Performance	45

6-2.  	2D Graphics Performance	46

6-3.  	Monitor Resolution and Refresh Rate	46

6-4.  	Video Port Pinouts	47

6-5.  	VGA Loopback Port Pinouts	48

6-6.  	Stereo Port Pinouts	49

# Preface

The GLZ1T Hardware User’s Guide provides instructions for using Intergraph’s GLZ1T advanced three dimensional (3D) graphics in conjunction with Intergraph’s 3D graphics workstations.  GLZ1T includes rasterization and texturing hardware.  This guide provides hardware and driver installation procedures, troubleshooting information, technical and hardware descriptions, and specifications.

## Using the Document

Chapter 1 introduces the GLZ1T board set and provides instructions for starting Windows NT, configuring the video display, changing the default display driver, and turning off full drag.

Chapter 2 gives instructions for replacing the GLZ1T board set.  It also contains a packing list and instructions for installing a dual screen option in the deskside workstations.

Chapter 3 gives instructions for installing the GL display driver and includes troubleshooting procedures.

Chapter 4 covers the technical features and functions of the GLZ1T board set.

Chapter 5 contains a hardware description of the GLZ1T board set.

Chapter 6 lists the GLZ1T board set specifications.

## Defining Document Conventions

FCC Statement

This document uses the following conventions:

Bold	Commands, words or characters that you type literally.

Italic	Cross-references; variables that you supply.

Monospace	Output that displays on the screen.

small caps	Type names on the keyboard, such as d, alt or f3.

UPPERCASE	Names of files and directories.  Type filenames and directory names in the dialog boxes or the command line in uppercase unless directed otherwise.

alt, shift, f	Press keys sequentially (press alt, then shift, then f).

C:\>	Command line prompt, indicating the character-based interface to the operating system.

ctrl+d	Press a key while simultaneously pressing another key; for example, ctrl and d.

Click	Press and immediately release a mouse button.

Double-click	Press and release a mouse button twice in succession.

Drag	Press and hold down a mouse button while moving the mouse, and then release the button.

Press 	Press and hold a mouse button, or press and release a key.

Select	Make a choice using an available selection method.  For example, place the screen cursor on an object and click a mouse button, or press tab and then enter.

Open	Use an available selection method to open a window, dialog, or folder.

Key in	Type input at the keyboard and press enter.

This equipment has been tested and found to comply with the limits for a Class A digital device, pursuant to part 15 of the FCC Rules.  These limits are designed to provide reasonable protection against harmful interference when the equipment is operated in a commercial environment.  This equipment generates, uses, and can radiate radio frequency energy.  If the equipment is not installed and used in accordance with the instruction manual, it may cause harmful interference to radio communications.  Operation of this equipment in a residential area is likely to cause harmful interference in which case the user will be required to correct the interference at his own expense.

This digital apparatus does not exceed the Class A limits for radio noise emissions from digital apparatus set out in the Radio Interference Regulations of the Canadian Department of Communications.

## Safety Information

This section provides the cautions and warnings for the equipment.

### Cautions

Changes or modifications made to the system that are not approved by the party responsible for compliance could void the user's authority to operate the equipment.

THIS PRODUCT CONFORMS TO THE APPLICABLE REQUIREMENTS OF 21 CFR SUBCHAPTER J AT DATE OF MANUFACTURE.

Read all safety and operating instructions before using the equipment.  Keep these instructions for future reference.  All warnings on the equipment or in the operating instructions should be followed.

### Warnings

To reduce the risk of electrical shock, do not attempt to open the equipment unless instructed.  Do not use a tool for purposes other than instructed.

## Getting Telephone Support

If you experience problems with your Intergraph workstation or software, or have questions about the information in this document, you can contact Intergraph for help.

- In the United States, call the Customer Response Center at 1-800-633-7248 between the hours of 7:00 a.m. and 7:00 p.m. CST, Monday through Friday (except holidays).

- At international locations, call the Intergraph subsidiary or distributor from which you purchased your Intergraph workstation or software.

- Have the following information available when you call:

-	The product’s name and serial number

-	Your name and telephone number

-	A brief description of the problem