Contents

Preface

Using the Document

Defining Document Conventions

FCC Statement

Safety Information

Cautions

Warnings

Getting Telephone Support

Chapter 1  Getting Started	1

Starting Windows NT	2

Configuring the Video Display	3

Dual-Screen Behavior	4

Gamma Correction	5

Ending the Video Configuration Session	6

Changing the Default Display Driver	7

Turning Off Full Drag	7

Using Software Cursor with OpenGL Applications	8

Chapter 2  Maintenance and Installation	9

Static Precautions	9

Opening the Desktop Workstation	10

Replacing GLZ2 Module in the Desktop	11

Completing the Installation for the Desktop	12

Opening the Deskside Workstation	13

Replacing GLZ2 Module in the Deskside	16

Installing a Dual Screen Option in the Deskside	17

Completing the Installation for the Deskside	19

Chapter 3  Driver Installation and Troubleshooting	21

GL Display Driver Installation	21

Troubleshooting	22

Diagnostics	22

Video Display	23

Obtaining a Usable Video Resolution	23

Determining a Defective Unit	24

Chapter 4  Technical Description	27

Features	27

Rasterization Accelerator	28

Frame Buffer	29

Functions	31

Chapter 5  Hardware Description	35

Rasterization Accelerator Components	35

PCIDMA	35

Graphics Engine	36

Resolver	37

Digital to Analog Converter	38

Frame Buffer Components	38

Video Memory	38

Video Selector and Mapper	39

Video Graphics Adapter Controller	39

Chapter 6  Specifications	41

Drawing Performance	41

3D Performance	41

2D Performance	42

Monitor Resolutions	42

Interfaces	43

Video Port	44

Stereo Port	45

Index	47

Figures

2-1.  	Desktop Top Cover Removal	10

2-2.  	GLZ2 Module Installation	11

2-3.  	Monitor Cable Connection	12

2-4.  	Deskside Top Cover Removal	13

2-5.  	Deskside Left Side Panel Removal	14

2-6.  	Deskside PCI Access Panel Removal	15

2-7.  	Clock Sync Cable Removal	16

2-8.  	Deskside PCI Slots	18

2-9.  	Clock Sync Cable Installation	19

2-10.  Single Monitor Cable Connections	20

2-11.  Dual Monitor Cable Connections	20

4-1.  	GLZ2 OpenGL Processing Block Diagram	31

4-2.  	GLZ2 Block Diagram	33

5-1.  	Rasterization Accelerator Board	35

5-2.  	Frame Buffer Board	38

6-1.  	Video Port	44

6-2.  	Stereo Port	45

Tables

1-1.  	GLZ2 Video Configuration Options	3

1-2.  	Gamma Correction Options	5

3-1.  	Video Configuration Problems	23

6-1.  	3D Graphics Performance	41

6-2.  	2D Graphics Performance	42

6-3.  	Intergraph Multiple Sync Monitor Resolutions	42

6-4.  	Standard Multi-sync Monitor Resolution and Refresh Rate	43

6-5.  	Video Port Pinouts	44

6-6.  	Stereo Port Pinouts	45

# Preface

The GLZ2 Hardware User’s Guide provides instructions for replacing the GLZ2 graphics option in Intergraph’s 3D graphics workstations and upgrading the deskside workstation from a single monitor to dual monitors.  This guide provides hardware and driver installation procedures, troubleshooting information, technical and hardware descriptions, and specifications.

## Using the Document

Chapter 1 introduces the GLZ2 module and provides instructions for starting Windows NT, configuring the video display, changing the default display driver, turning off full drag, and using software cursors.

Chapter 2 gives instructions for replacing the GLZ2 module.  It also contains a packing list and instructions for installing a dual screen option in the deskside workstation.

Chapter 3 gives instructions for installing the GL display driver and includes troubleshooting procedures.

Chapter 4 covers the technical features and functions of the GLZ2 module.

Chapter 5 contains a hardware description of the GLZ2 module.

Chapter 6 lists the GLZ2 module specifications.

## Defining Document Conventions

FCC Statement

This document uses the following conventions:

Bold	Commands, words or characters that you type literally.

Italic	Cross-references; variables that you supply.

Monospace	Output that displays on the screen.

small caps	Type names on the keyboard, such as d, alt or f3.

UPPERCASE	Names of files and directories.  Type filenames and directory names in the dialog boxes or the command line in uppercase unless directed otherwise.

alt, shift, f	Press keys sequentially (press alt, then shift, then f).

C:\>	Command line prompt, indicating the character-based interface to the operating system.

ctrl+d	Press a key while simultaneously pressing another key; for example, ctrl and d.

Click	Press and immediately release a mouse button.

Double-click	Press and release a mouse button twice in succession.

Drag	Press and hold down a mouse button while moving the mouse, and then release the button.

Press 	Press and hold a mouse button, or press and release a key.

Select	Make a choice using an available selection method.  For example, place the screen cursor on an object and click a mouse button, or press tab and then enter.

Open	Use an available selection method to open a window, dialog, or folder.

Key in	Type input at the keyboard and press enter.

This equipment has been tested and found to comply with the limits for a Class A digital device, pursuant to part 15 of the FCC Rules.  These limits are designed to provide reasonable protection against harmful interference when the equipment is operated in a commercial environment.  This equipment generates, uses, and can radiate radio frequency energy.  If the equipment is not installed and used in accordance with the instruction manual, it may cause harmful interference to radio communications.  Operation of this equipment in a residential area is likely to cause harmful interference in which case the user will be required to correct the interference at his own expense.

This digital apparatus does not exceed the Class A limits for radio noise emissions from digital apparatus set out in the Radio Interference Regulations of the Canadian Department of Communications.

## Safety Information

This section provides the cautions and warnings for the equipment.

### Cautions

Changes or modifications made to the system that are not approved by the party responsible for compliance could void the user's authority to operate the equipment.

THIS PRODUCT CONFORMS TO THE APPLICABLE REQUIREMENTS OF 21 CFR SUBCHAPTER J AT DATE OF MANUFACTURE.

Read all safety and operating instructions before using the equipment.  Keep these instructions for future reference.  All warnings on the equipment or in the operating instructions should be followed.

### Warnings

To reduce the risk of electrical shock, do not attempt to open the equipment unless instructed.  Do not use a tool for purposes other than instructed.

## Getting Telephone Support

If you experience problems with your Intergraph workstation or software, or have questions about the information in this document, you can contact Intergraph for help.

- In the United States, call the Customer Response Center at 1-800-633-7248 between the hours of 7:00 a.m. and 7:00 p.m. CST, Monday through Friday (except holidays).

- At international locations, call the Intergraph subsidiary or distributor from which you purchased your Intergraph workstation or software.

- Have the following information available when you call:

-	The product’s name and serial number

-	Your name and telephone number

-	A brief description of the problem