# Specifications

Standard specifications for the GLZ3 graphics subsystem are subject to change without notice.  The following specifications apply to GLZ3.

VRAM:  12 MB

I/O ports:  One video output, one stereo sync output, and one VGA input

GLZ3 power consumption:  50 watts maximum

Note:	Refer to the TDZ workstation hardware user’s guide for environmental specifications.

## Drawing Performance

The following summarizes the GLZ3 hardware performance for some commonly used graphics operations.  Actual system performance depends on the host processor speed and the software application.

### 3D Performance

The performance values of the 3D drawing requests are obtained from randomly oriented data with Z-buffering enabled.

Table 6-1.  3D Graphics Performance

| 3D Requests | Performance |
| --- | --- |
| Triangle--25 pixel, Gouraud shaded | 440 K Triangles/sec |
| Triangle--50 pixel, Gouraud shaded | 235 K Triangles/sec |
| Vector--10 pixel, solid color | 900 K Vectors/sec |
| Vector--10 pixel, Gouraud shaded | 900 K Vectors/sec |
| Vector--10 pixel, antialiased | 425 K Vectors/sec |

### 2D Performance

The performance values of the 2D drawing requests for the Graphical User Interface (GUI) are obtained with randomly oriented data.

Table 6-2.  2D Graphics Performance

| 2D Requests | Performance |
| --- | --- |
| Vector--10 pixel, solid color | 1.2 M Vectors/sec |
| Vector--10 pixel, antialiased | 450 K Vectors/sec |
| Blit--64 Bit | 13 M Pixels/sec |
| Fill--1280 x 1024 | 200 M Pixels/sec |
| Text--9 x 13 | 195 K Chars/sec |

## Monitor Resolutions

GLZ3 supports standard multi-sync monitors, including the Intergraph        17-inch and 21-inch monitors, at the following resolutions and refresh rates.

Table 6-3.  Monitor Resolution and Refresh Rate

| Resolution | Refresh Rate (Hz) |
| --- | --- |
| 640 x 480 | 60, 72, 75, 85 |
| 800 x 600 | 60, 72, 75, 85 |
| 1024 x 768 | 60, 70, 75, 85 |
| 1152 x 864 | 60, 70, 75, 76 |

Note: 	For more information on supported resolutions, refer to the README.TXT file included on the diskette containing the Intergraph GL Display Driver software.

## Interfaces

GLZ3 has three interface connections.

- Video Port

- VGA Loopback Port

- Stereo Port

### Video Port

The video port provides connection to the monitor.  The following table provides the pin connections for the video port.

Table 6-4.  Video Port Pinouts

| Signal Name | Pin |
| --- | --- |
| Red Analog Video | 1 |
| Green Analog Video | 2 |
| Blue Analog Video | 3 |
| Ground | 4 |
| Ground | 5 |
| Ground | 6 |
| Ground | 7 |
| Ground | 8 |
| No Connect | 9 |
| Ground | 10 |
| Ground | 11 |
| Ground | 12 |
| Horizontal Sync | 13 |
| Vertical Sync | 14 |
| Ground | 15 |

The video port is female, 15-pin, D-Sub as shown in the following figure.  The recommended cable length is less than three meters.

4166

15

11

1

6

Figure 6-1.  Video Connector

### VGA Loopback Port

The VGA loopback port provides connection between the host VGA subsystem and the GLZ3 graphics subsystem.

Table 6-5.  VGA Loopback Port Pinouts

| Signal Name | Pin |
| --- | --- |
| Red Analog Video | 1 |
| Green Analog Video | 2 |
| Blue Analog Video | 3 |
| Ground | 4 |
| Ground | 5 |
| Ground | 6 |
| Ground | 7 |
| Ground | 8 |
| No Connect | 9 |
| Ground | 10 |
| Ground | 11 |
| Ground | 12 |
| Horizontal Sync | 13 |
| Vertical Sync | 14 |
| Ground | 15 |

The VGA loopback port is female, 15-pin, D-Sub as shown in the following figure.

4166

15

11

1

6

Figure 6-2.  VGA Loopback Connector

### Stereo Port

The stereo port provides connection to the LCD shutter glasses’ emitter module.  The following table provides the pin connections for the stereo port.

Table 6-6.  Stereo Port Pinouts

| Signal Name | Pin |
| --- | --- |
| Ground | 1 |
| Ground | 2 |
| +12V | 3 |
| Stereo Sync | 4 |
| No Connect | 5 |

The stereo port is female, 5-pin, mini-DIN as shown in the following figure.

1

3

4173

4

2

5

Figure 6-3.  Stereo Connector