Contents

Preface

Using the Document

Defining Document Conventions

FCC Statement

Safety Information

Cautions

Warnings

Getting Telephone Support

Chapter 1  Getting Started	1

Starting Windows NT	2

Configuring the Video Display	3

Dual-Screen Behavior	4

Gamma Correction	5

Ending the Video Configuration Session	6

Changing the Default Display Driver	6

Turning Off Full Drag	7

Using Software Cursor with OpenGL Applications	7

Chapter 2  Maintenance and Installation	9

Static Precautions	9

Opening the Desktop Workstation	10

Replacing GLZ3 Boards in the Desktop	11

Completing the Installation for the Desktop	13

Opening the Deskside Workstation	14

Replacing GLZ3 Boards in the Deskside	16

Installing a Dual Screen Option in the Deskside	18

Completing the Installation for the Deskside	20

Chapter 3  Driver Installation and Troubleshooting	23

GL Display Driver Installation	23

Troubleshooting	24

Diagnostics	24

Video Display	25

Obtaining a Usable Video Resolution	25

Determining a Defective Unit	26

Chapter 4  Technical Description	29

Features	29

Geometry Accelerator	30

Rasterization Accelerator/Frame Buffer	31

Functions	34

Geometry Accelerator	35

Rasterization Accelerator/Frame Buffer	36

Chapter 5  Hardware Description	39

Geometry Accelerator Components	39

PCIDMA	39

I/O FIFO	40

DSPs	40

Sequence Controller	40

Rasterization Accelerator/Frame Buffer Components	41

PCIDMA	41

Graphics Engine	41

Resolver	43

Video Memory	43

Video Selector and Mapper	44

Digital to Analog Converter	44

Chapter 6  Specifications	45

Drawing Performance	45

3D Performance	45

2D Performance	46

Monitor Resolutions	46

Interfaces	46

Video Port	47

VGA Loopback Port	48

Stereo Port	49

Index	51

Figures

2-1.  	Desktop Top Cover Removal	10

2-2.  	FIFO Status Cable Removal	11

2-3.  	GLZ3 Board Installation	12

2-4.  	VGA Loopback Cable and Monitor Cable Connections	13

2-5.  	Deskside Top Cover Removal	14

2-6.  	Deskside Left Side Panel Removal	15

2-7.  	Deskside PCI Access Panel Removal	16

2-8.  	FIFO Status and Clock Sync Cables	17

2-9.  	Deskside PCI Slots	19

2-10.  Clock Sync Cable and FIFO Status Cable Installation	20

2-11.  Single Monitor Cable Connections	21

2-12.  Dual Monitor Cable Connections	22

4-1.  	GLZ3 OpenGL Processing Block Diagram	34

4-2.  	Geometry Accelerator Block Diagram	35

4-3.  	Rasterization Accelerator/Frame Buffer Block Diagram	37

5-1.  	Geometry Accelerator Board	39

5-2.  	Rasterization Accelerator/Frame Buffer Board	41

6-1.  	Video Connector	47

6-2.  	VGA Loopback Connector	48

6-3.  	Stereo Connector	49

Tables

1-1.  	GLZ3 Video Configuration Options	3

1-2.  	Gamma Correction Options	5

3-1.  	Video Configuration Problems	25

6-1.  	3D Graphics Performance	45

6-2.  	2D Graphics Performance	46

6-3.  	Monitor Resolution and Refresh Rate	46

6-4.  	Video Port Pinouts	47

6-5.  	VGA Loopback Port Pinouts	48

6-6.  	Stereo Port Pinouts	49

# Preface

The GLZ3 Hardware User’s Guide provides instructions for using Intergraph’s  GLZ3 advanced three dimensional (3D) graphics subsystem in conjunction with Intergraph’s 3D graphics workstations.  GLZ3 is a complete 3D graphics subsystem including geometry acceleration and rasterization acceleration.  This guide provides hardware and driver installation procedures, troubleshooting information, technical and hardware descriptions, and specifications.

## Using the Document

Chapter 1 introduces the GLZ3.  It provides instructions for starting Windows NT, configuring the video display, changing the default display driver, turning off Full Drag, and using software cursors.

Chapter 2 gives instructions for opening a desktop or deskside workstation and replacing a GLZ3 board.  It also describes installing a dual screen option in the deskside workstation.

Chapter 3 includes instructions for installing the GL display driver and troubleshooting procedures.

Chapter 4 covers the technical features and functions of the GLZ3 graphics subsystem.

Chapter 5 contains a description of the GLZ3 hardware.

Chapter 6 lists the GLZ3 specifications.

## Defining Document Conventions

FCC Statement

This document uses the following conventions:

Bold	Commands, words or characters that you type literally.

Italic	Cross-references; variables that you supply.

Monospace	Output that displays on the screen.

small caps	Type names on the keyboard, such as d, alt or f3.

UPPERCASE	Names of files and directories.  Type filenames and directory names in the dialog boxes or the command line in uppercase unless directed otherwise.

alt, shift, f	Press keys sequentially (press alt, then shift, then f).

C:\>	Command line prompt, indicating the character-based interface to the operating system.

ctrl+d	Press a key while simultaneously pressing another key; for example, ctrl and d.

Click	Press and immediately release a mouse button.

Double-click	Press and release a mouse button twice in succession.

Drag	Press and hold down a mouse button while moving the mouse, and then release the button.

Press 	Press and hold a mouse button, or press and release a key.

Select	Make a choice using an available selection method.  For example, place the screen cursor on an object and click a mouse button, or press tab and then enter.

Key in 	Type input at the keyboard and press enter.

Open	Use an available selection method to open a window, dialog, or folder.

This equipment has been tested and found to comply with the limits for a Class A digital device, pursuant to part 15 of the FCC Rules.  These limits are designed to provide reasonable protection against harmful interference when the equipment is operated in a commercial environment.  This equipment generates, uses, and can radiate radio frequency energy.  If the equipment is not installed and used in accordance with the instruction manual, it may cause harmful interference to radio communications.  Operation of this equipment in a residential area is likely to cause harmful interference in which case the user will be required to correct the interference at his own expense.

This digital apparatus does not exceed the Class A limits for radio noise emissions from digital apparatus set out in the Radio Interference Regulations of the Canadian Department of Communications.

## Safety Information

This section provides the cautions and warnings for the equipment.

### Cautions

Changes or modifications made to the system that are not approved by the party responsible for compliance could void the user's authority to operate the equipment.

THIS PRODUCT CONFORMS TO THE APPLICABLE REQUIREMENTS OF 21 CFR SUBCHAPTER J AT DATE OF MANUFACTURE.

Read all safety and operating instructions before using the equipment.  Keep these instructions for future reference.  All warnings on the equipment or in the operating instructions should be followed.

### Warnings

To reduce the risk of electrical shock, do not attempt to open the equipment unless instructed.  Do not use a tool for purposes other than instructed.

## Getting Telephone Support

If you experience problems with your Intergraph workstation or software, or have questions about the information in this document, you can contact Intergraph for help.

- In the United States, call the Customer Response Center at 1-800-633-7248 between the hours of 7:00 a.m. and 7:00 p.m. CST, Monday through Friday (except holidays).

- At international locations, call the Intergraph subsidiary or distributor from which you purchased your Intergraph workstation or software.

- Have the following information available when you call:

-	The product’s name and serial number

-	Your name and telephone number

-	A brief description of the problem