# Maintenance and Installation

This chapter describes opening Intergraph’s desktop and deskside 3D graphics workstations to replace a GLZ3 board.  It also describes installing a dual screen option in the deskside workstation.

## Static Precautions

Observe the following precautions when working with the internal components of the system.

- Avoid servicing the system in areas with electrostatic problems.  Rugs and carpets, for example, cause static build-up.

- Handle all printed circuit boards as little as possible and by the edges only.  Leave the boards in their protective packaging until you install them.

CAUTION

Discharge personal static charge by touching a bare metal surface on the workstation before handling printed circuit boards.

## Opening the Desktop Workstation

WARNING

Before opening the workstation, turn off the system power.  Leave the power cord connected to maintain ground.  Use caution when removing the cover to avoid injury.

ä To open the desktop workstation:

1.	Pull out the spring-lock plunger on the back panel and turn it clockwise until it locks in the open position.

4715A

ISA I/O

Panel

Spring-lock

Plunger

Figure 2-1.  Desktop Top Cover Removal

2.	Lift up the top cover above the ISA I/O panel and pull up and back to remove the top cover.

3.	Attach an antistatic wrist strap to a non-painted metallic surface on the workstation or to the ground loop (formed in the metal) above the ISA I/O panel.  Ensure that the elastic loop in the strap snugly contacts your bare wrist.

## Replacing GLZ3 Boards in the Desktop

This section describes replacing the Rasterization Accelerator/Frame Buffer board or the Geometry Accelerator board in the desktop workstation.

ä To replace a GLZ3 board in the desktop workstation:

1.	If replacing the Rasterization Accelerator/Frame Buffer board, disconnect the cables from the video and VGA ports on the back panel of the base unit.

2.	Disconnect the FIFO status cable as shown in the following figure.

4705

FIFO Status Cable

Geometry Accelerator Board

Rasterization Accelerator/Frame Buffer Board

Figure 2-2.  FIFO Status Cable Removal

3.	Using a quarter-inch nutdriver, remove the screw securing the board to the workstation.  Retain the screw.

4.	Remove the board from the workstation.

5.	Install the new board into the same slot from which the previously installed board was removed.

Note:  	The Geometry Accelerator board resides in PCI slot 1.  The Rasterization Accelerator/Frame Buffer board resides in PCI slot 2.

Slot 2

Rasterization Accelerator/

Frame Buffer Board

Screw

Geometry Accelerator Board

Slot 1

Figure 2-3.  GLZ3 Board Installation

6.	Secure the board to the workstation using the screw previously removed.

7.	Install the FIFO status cable between the Rasterization Accelerator/Frame Buffer board and the Geometry Accelerator board.

Note: 	The connector of the FIFO status cable can be installed in any orientation.

## Completing the Installation for the Desktop

Follow these steps to reassemble the workstation and reconnect the monitor.

ä To complete the installation:

1.	Replace the top cover on the workstation by aligning the tabs on the front of the top cover with the notches behind the faceplate.

2.	Lower the back of the top cover and slide it into place.

3.	Turn the spring-lock plunger to lock the top cover to the base unit.

4.	Connect the VGA loopback cable to the VGA In port and to the VGA Out port.

4732A

VGA In PortVGA Out

Port

VGA Loopback Cable

Video Out Port

Figure 2-4.  VGA Loopback Cable and Monitor Cable Connections

5.	Connect the monitor cable to the Video Out port in PCI slot 2.

6.	If you need to configure the GL display driver, refer to “Configuring the Video Display” in Chapter 1.

## Opening the Deskside Workstation

WARNING

Before opening the workstation, turn off the system power.  Leave the power cord connected to maintain ground.  Use caution when removing the cover to avoid injury.

ä To open the deskside workstation:

1.	Using a quarter-inch nutdriver, remove the screw from the top cover.  Retain the screw.

4136B + MOD (deleted SCSI connector)

Screw

Figure 2-5.  Deskside Top Cover Removal

2.	Remove the top cover by sliding it toward the back of the workstation.

3.	Remove the footstands.

4.	Remove the two screws from the top of the left side panel.

4636

Screws

Figure 2-6. Deskside Left Side Panel Removal

5.	Remove the left side panel by grasping the tab between the screw holes.  Then, slide the panel up and out from the workstation.

6.	Lay the workstation on its right side.

7.	Remove the three screws from the PCI access panel on the bottom of the workstation.

4696

Screw

PCI Access Panel

Screws

Figure 2-7.  Deskside PCI Access Panel Removal

8.	Remove the PCI access panel by sliding it toward the back of the workstation.

9.	Attach an antistatic wrist strap to a non-painted metallic surface on the workstation.  Ensure that the elastic loop in the strap snugly contacts your bare wrist.

## Replacing GLZ3 Boards in the Deskside

This section describes replacing the Rasterization Accelerator/Frame Buffer board or the Geometry Accelerator board in the deskside workstation.  The Geometry Accelerator board resides in PCI slot 1.  The Rasterization Accelerator/Frame Buffer board resides in PCI slot 2.

ä To replace a GLZ3 board in the deskside workstation:

1.	If replacing the Rasterization Accelerator/Frame Buffer board, disconnect the cables from the Video and VGA ports on the back panel of the base unit.

2.	Disconnect the FIFO status cable.  If replacing the Rasterization Accelerator/Frame Buffer board in a dual screen configuration, disconnect the clock sync cable.

4706

FIFO Status Cables

Clock Sync Cable

Rasterization Accelerator/

Frame Buffer Boards

Geometry Accelerator Board

Figure 2-8.  FIFO Status and Clock Sync Cables

3.	Using a quarter-inch nutdriver, remove the screw securing the board to the workstation.  Retain the screw.

4.	Remove the board from the workstation.

5.	Install the new board into the same slot from which the previously installed board was removed.

Note:  	In a dual screen configuration, the Geometry Accelerator board resides in PCI slot 1 and the primary Rasterization Accelerator/Frame Buffer board resides in PCI slot 2.  The secondary Rasterization Accelerator/Frame Buffer board resides in PCI slot 3.  Refer to Figure 2-9 for board and PCI slot location.

6.	Secure the board to the base unit using the screw removed previously.

7.	Install the FIFO status cable between the Rasterization Accelerator/Frame Buffer board and the Geometry Accelerator board.

Note: 	The connector on the FIFO status cable can be installed in any orientation.

8.	If you replaced a graphics board set in a dual screen configuration, install the clock sync cable.

CAUTION

The clock sync cable installs only one way.  Do not force the keyed connector.  Ensure pin 1 of both cable connectors engages pin 1 on both board connectors.  The black wire connects to pin 1.

## Installing a Dual Screen Option in the Deskside

The Intergraph GLZ3 dual screen option kit provides the following components to upgrade your deskside workstation from a single screen to a dual screen configuration.  Verify that you have the following items:

- GLZ3 Rasterization Accelerator/Frame Buffer board

- Diskette containing the Intergraph GL display driver software

- GLZ3 Hardware User’s Guide

- FIFO status cable

- Clock sync cable

- Antistatic wrist strap

If an item is missing or damaged, notify Intergraph’s Customer Response Center at 1-800-633-7248 immediately.

ä To install the dual screen option:

1.	Using a quarter-inch nutdriver, remove the blanking plate from PCI
slot 3.  Retain the screw.  Refer to Figure 2-9 for board and PCI slot locations.

2.	Install the second Rasterization Accelerator/Frame Buffer board into PCI slot 3.

4632

PCI Slot 1

PCI Slot 2

PCI Slot 3

PCI Slot 6

PCI Slot 5

PCI Slot 4

Figure 2-9.  Deskside PCI Slots

Note: 	For dual monitors, two Rasterization Accelerator/Frame Buffer boards are required.  The primary board should reside in PCI slot 2 and the secondary board in PCI slot 3.

3.	Install the screw to secure the Rasterization Accelerator/Frame Buffer board to the workstation.

4.	Install the clock sync cable between the two Rasterization Accelerator/Frame Buffer boards.  Refer to Figure 2-10.

CAUTION

The clock sync cable installs only one way.  Do not force the keyed connector.  Ensure pin 1 of both cable connectors engages pin 1 on both board connectors.  The black wire connects to pin 1.

4698

FIFO Status Cables

Clock Sync Cable

Rasterization Accelerator/

Frame Buffer Boards

Geometry Accelerator Board

Figure 2-10.  Clock Sync Cable and FIFO Status Cable Installation

5.	Install the FIFO status cable between the secondary Rasterization Accelerator/Frame Buffer board and the Geometry Accelerator board.

Note: 	The connector on the FIFO status cable can be installed in any orientation.

## Completing the Installation for the Deskside

Follow these steps to reassemble the workstation and reconnect the monitor.

ä To complete the installation:

1.	Install the PCI access panel on the workstation.

2.	Raise the workstation to its normal standing position.

3.	Install the side panel of the workstation.

4.	Install the footstands.

CAUTION

To avoid injury or damage to the equipment, replace the stabilizing footstands before operating the unit.

5.	Install the top cover of the workstation.

6.	Connect the VGA Loopback cable and monitor cable for single monitor configurations as follows:

VGA Loopback:  Connect the VGA Loopback cable to the VGA Out port and to the VGA In port as shown in Figure 2-11.

Single monitor:  Connect the monitor cable to the Video Out port in PCI slot 2.

4709B

Video

Out Port

VGA Out Port          VGA In Port

VGA Loopback

Cable

Figure 2-11.  Single Monitor Cable Connections

7.	Connect the VGA Loopback cable and monitor cables for dual monitor configurations as follows:

VGA Loopback:  Connect the VGA Loopback cable to the VGA Out port and to the VGA In port as shown in Figure 2-12.

Primary monitor:  Connect the monitor cable to the Video Out port in PCI slot 2.

Secondary monitor:  Connect the monitor cable to the Video Out port in PCI slot 3.

4764

Video

Out Port

(Secondary)

Video

Out Port

(Primary)

VGA Out Port   VGA In Port

VGA Loopback

Cable

Figure 2-12.  Dual Monitor Cable Connections

8.	If you need to configure the GL display driver, refer to “Configuring the Video Display” in Chapter 1.