# Technical Description

GLZ4 supports many features and functions previously found only in high end workstation graphics.  This chapter provides a technical description of the GLZ4 graphics subsystem by discussing its features and functions.

## Features

The two PCI bus devices listed below work together to provide full hardware acceleration of graphics tasks.  The features listed for each device enhance both the performance and the image quality of the GLZ4.

- Geometry Accelerator:

- Industry standard PCI bus

- High-speed graphics DMA engine

- MIMD DSP architecture

- Sequence Controller

- Rasterization Accelerator/Frame Buffer:

- Industry standard PCI bus

- High-speed graphics DMA engine

- Multiple vertex data formats

- Gouraud shading support

- Symbolized vector support

- Antialiased vector support

- Sub-pixel accuracy

- 92 video planes

- True color frame buffer

- Z buffer

- Video lookup tables

- Window mode double buffering

- Gamma correction

- Hardware cursor

- Stereoscopic viewing

- Multiple video resolutions

- High speed screen refresh

- Genlock support for dual monitors

### Geometry Accelerator

The Geometry Accelerator employs the following features to accelerate the geometry processing requirements of computer graphics.

Industry Standard PCI Bus -- The Geometry Accelerator uses the PCI bus to transfer data from the host processor and transfer data to the Rasterization Accelerator.  The PCI bus is a high performance bus capable of operating up to 33 MHz.  The Geometry Accelerator is a 32-bit, PCI bus peripheral with bus mastership capability.

High Speed Graphics DMA Engine -- The Geometry Accelerator uses high speed Direct Memory Access (DMA) engines to accelerate PCI bus data transfers.  Two independent DMA engines transfer data in sixteen, 32-bit word bursts.  The DMAs free the host processor from transferring massive amounts of graphics data over the PCI bus.

MIMD DSP Architecture -- The Geometry Accelerator features a Multiple Instruction Multiple Data (MIMD) architecture to harness the power of four advanced floating point Digital Signal Processors (DSPs) operating in parallel.  The DSPs speed the compute intensive geometry calculations of 3D graphics.  Each DSP can execute 100 Million FLoating point Operations Per Second (MFLOPS), and contains 10 DMA engines to ensure nonstop data flow to the Sequence Controller.

Sequence Controller -- To achieve maximum efficiency in a MIMD architecture, multiple processors operate independently on multiple data sets.  The four Geometry Accelerator DSPs process graphics requests in turn and may generate output requests in a different order than the input requests.  The Sequence Controller places the output requests back into the proper time sequence.

### Rasterization Accelerator/Frame Buffer

The Rasterization Accelerator/Frame Buffer has many features that accelerate the rasterization requirements of computer graphics and greatly enhance the visual impact of GLZ4 graphics.

Industry Standard PCI Bus -- The Rasterization Accelerator uses the PCI bus to transfer data to and from the host processor.  The PCI bus is a high performance bus capable of operating up to 33 MHz.  The Rasterization Accelerator/Frame Buffer is a 32-bit PCI bus peripheral with bus mastership capability.

High Speed Graphics DMA Engine -- The Rasterization Accelerator uses a high speed DMA engine to accelerate PCI bus data transfers.  The DMA engine transfers data in sixteen, 32-bit word bursts.  The DMA frees the host processor from transferring massive amounts of graphics data over the PCI bus.

Multiple Vertex Data Formats -- The Rasterization Accelerator allows the software to send vertex data for triangles and vectors in multiple formats.  The formats include 8-bit packed integer, 16-bit packed integer, 32-bit integer, single-precision floating point, and double-precision floating point.

The host processor achieves higher performance because it does not convert data that is native to a particular application.  To conserve bandwidth, the application optionally loads constant color and Z values to avoid sending them with each vertex.

Gouraud Shading Support -- The Rasterization Accelerator supports Gouraud shading of 3D graphics triangle meshes.  High performance hardware accomplishes the shading of the entire triangle including color and depth derivative calculations and pixel interpolation.  The hardware supports Gouraud shaded vectors and allows maximum update rates with minimum load on the host processor.

Symbolized Vector Support -- The rasterization hardware supports symbolized vectors that allow the application of any user-supplied, 32-bit, two-color pattern without performance degradation.  Dotted or dashed lines are examples of symbolized vectors.

Antialiased Vector Support -- Hardware support of antialiased vectors eliminates the jagged appearance of vectors on standard raster displays.

Sub-pixel Accuracy -- The Rasterization Accelerator uses sub-pixel accuracy for improving the quality of the graphics image.  Sub-pixel accuracy ensures adjacent polygons and vectors join smoothly.

92 Video Planes -- The following summarizes the 92 video plane set for the GLZ4 configurations.

Image (red, green, blue), front buffer	24 bits

Image (red, green, blue), back buffer	24 bits

Overlay, front buffer	  1 bit

Overlay, back buffer	  1 bit

Image, VLT selection, front buffer	  3 bits

Image, VLT selection, back buffer	  3 bits

Z buffer	24 bits

Mask (pixel access control)	  4 bits

Image, window control	  4 bits

Overlay, window control	  4 bit

The image planes consist of 8 bits each of red, green, and blue to provide 16.7 million possible colors.  There are two copies, or buffers, of image data, front and back.  This allows smooth animation by updating one buffer while displaying from the other buffer.

Both image buffers have access to one of eight Video Lookup Tables (VLTs).  The image VLT selection planes select the VLT to use.  A double buffered overlay plane provides the ability to annotate text and graphics with a 3D image in the image buffer.

The Z buffer provides 24 bits of depth information.  The mask planes allow for access control of the various drawing planes (for example, image or overlay).  Finally, the image and overlay window control planes determine the buffer to display on a pixel-by-pixel basis.

True Color Frame Buffer -- A 24-bit, double buffered, true color frame buffer provides 8 bits of information for each red, green, and blue primary color.  The frame buffer provides two separate buffers, so that one updates while the other displays.

Z Buffer -- A 24-bit, Z buffer provides accurate display of scenes with large depth separation between objects.

Video Lookup Tables -- The Frame Buffer includes eight Video Lookup Tables (VLTs).  The VLTs provide multiple color palettes simultaneously for color remapping and advanced color refinement.

Window Mode Double Buffering -- Window mode double buffering allows multiple windows to display simultaneously from different buffers.

Gamma Correction -- State of the art 10-bit gamma VLTs, and 10-bit Digital to Analog Converters (DACs) allow user control of gamma values without color degradation.

Hardware Cursor -- A hardware cursor minimizes cursor tracking tasks of the host processor.

Stereoscopic Viewing -- GLZ4 supports the display of stereo images.  Stereoscopic viewing requires a stereo-ready monitor and infrared shutter glasses with emitter module.

Multiple Video Resolutions -- GLZ4 includes programmable pixel clock generation and video timing logic.  This allows support of a wide variety of monitors with multiple video resolutions and refresh rates.  Programmable video timing also allows future monitor upgrades with only a software driver change.  Supported resolutions range from 640 x 480 up to 1600 x 1280.  Refer to Chapter 6 for additional information on supported monitor resolutions.

High Speed Screen Refresh -- High speed screen refresh provides flicker-free displays at 76 Hz and above for all screen resolutions.

Genlock Support for Dual Monitors -- Genlock eliminates the visual beat frequency effect that may occur when placing two monitors close to each other.  GLZ4 supports Genlock for dual monitors by using a clock sync cable installed between the two Rasterization Accelerator/Frame Buffer modules.

## Functions

GDI is the library of functions that Microsoft Windows NT uses to create and manipulate 2D computer images.  The GLZ4 rasterization hardware accelerates many of the GDI functions.  OpenGL is an industry standard library of functions used to create and manipulate 3D computer images.  OpenGL functions, as built in to the Microsoft Windows NT operating system, can be performed by the host processor.  However, the GLZ4 hardware accelerators greatly improve OpenGL performance.

SD0028

Figure 4-1.  GLZ4 OpenGL Processing Block Diagram

As shown in Figure 4-1, the GLZ4 provides OpenGL acceleration using the Geometry Accelerator and the Rasterization Accelerator.  This allows the host processor to perform application and database operations while the accelerators perform OpenGL processing.  This parallel processing results in unprecedented speed to generate and manipulate realistic images.

### Geometry Accelerator

The Geometry Accelerator receives OpenGL requests from the host processor through the PCIDMA and I/O FIFO.   The requests are routed to four independent 32-bit, floating point Digital Signal Processors (DSPs).  The four high power DSPs process OpenGL requests in parallel.  The output requests from each DSP flow through dedicated ports to the Sequence Controller.  The Sequence Controller time-orders and formats the requests, and then writes them back to the I/O FIFO.  A DMA in the PCIDMA ASIC moves the processed requests to the Rasterization Accelerator.

The processing performed by each of the four DSPs includes coordinate transformations, lighting calculations, and viewport clipping. Clipping is the process of removing elements, or pieces of elements, not contained within the viewing volume (viewport).  The Geometry Accelerator can process each of the primitive types specified by OpenGL, however, the Geometry Accelerator does not process primitives which are not supported by the Rasterization Accelerator.  For example, when the Rasterization Accelerator does not contain a texturing subsystem, the processing of textured primitives is performed by the host processor.

SD0022 modified to say glz4 (we have no assigned drawing file for this one)

Figure 4-2.  Geometry Accelerator Block Diagram

### Rasterization Accelerator/Frame Buffer

The Rasterization Accelerator consists of the PCIDMA, Graphics FIFO, and Graphics Engine.  The Frame Buffer consists of the Resolvers, Video Memory, Video Selector and Mappers (VSMs), and Digital to Analog Converter (DAC).  The standard PCI bus provides high bandwidth support for the transfer of graphics data between the host processor, the Geometry Accelerator, and the Rasterization Accelerator.

The Rasterization Accelerator receives GDI or OpenGL element requests from the host processor, or from the Geometry Accelerator, through the PCIDMA and Graphics FIFO.  Element types include triangles, triangle meshes, lines, line strings, and others.  The graphics engine ASIC converts the elements to pixels and routes them through the Resolvers to the Video Memory.  The Rasterization Accelerator also performs region fills, bit expands, and BIT BLock Transfer (BITBLT) operations.  The VSMs accept digital pixel data from the Video Memory, and send it to the DAC where it is converted to an analog voltage and presented to the monitor.

SD0030

SD0030

Figure 4-3.  Rasterization Accelerator/Frame Buffer Block Diagram