# Driver Installation and Troubleshooting

This chapter describes installing the GL display driver when it becomes necessary to upgrade to the latest driver or when upgrading the workstation hardware configuration.  It also contains troubleshooting procedures to determine which hardware assembly to return for repair.

## GL Display Driver Installation

First, power on the system and log on to Windows NT.  Then, install the GL driver as described in the following procedure.

ä To install the GL display driver:

1.	Open Control Panel in the Main group of Program Manager.

2.	Open Display in the Control Panel window.  The Display Settings dialog displays.

3.	Select Change Display Type.  The Display Type dialog displays.

4.	Select Change.  The Select Device dialog displays.

5.	Confirm that the Disable current driver check box is marked.

6.	Select Other.  The Install from Disk dialog displays.

7.	Insert the diskette containing the Intergraph GL display driver into the floppy disk drive.

8.	Select OK.  After a few seconds, the Intergraph GL Display Driver highlights.

9.	Select Install.  When prompted, confirm you want to change your system configuration.

Note:  	If the GL display driver is already installed, the Windows NT Setup dialog displays, allowing you to choose the currently installed driver or a new driver.  Select New to install the GL display driver.

10.	When prompted, confirm the directory path for the driver.  The Video Configuration dialog displays.

11.	Select and change the option settings according to system configuration and your preferences.  Refer to Chapter 1 or use Help for information about each option.

12.	Select OK to apply the attributes.

13.	Remove the diskette from the floppy disk drive.

14.	Shut down and restart the workstation for the driver to take effect.

## Troubleshooting

If you suspect a hardware problem, use this section to verify the graphics hardware, and to determine which hardware assembly to return for repair.

If you must reinstall the GL display driver, refer to the previous section, 
“GL Display Driver Installation.”  If problems persist, contact the Customer Response Center at 1-800-633-7248.

### Diagnostics

Diagnostic utilities for checking the graphics boards and instructions for using them are available on the Intergraph Bulletin Board Service (IBBS).  Refer to the TDZ hardware user’s guide for instructions to access the IBBS and obtain files.

### Video Display

The following table lists some common video configuration problems.

Table 3-1.  Video Configuration Problems

| Problem | Action |
| --- | --- |
| Display is black, not synchronized, or distorted. | Restart Windows NT in VGA mode. |
| Monitor does not support a selected resolution or refresh rate. | Select a supported resolution and refresh rate. |
| Two GLZ4 modules are installed, but you cannot select the Dual Screen option. | Verify proper installation of both GLZ4 modules. |
| A dim line, rolling vertically, is visible on the screen. | Verify proper installation of the clock sync cable. |
| No display on the screen(s). | Check the monitor, monitor cable, and power to the monitor(s). Verify proper installation of GLZ4 modules. |

### Obtaining a Usable Video Resolution

The system operates in VGA mode when the GL graphics driver is not running in order to accommodate all monitor types.  VGA mode is used during initial installation of the GL display driver and when experiencing video problems.

There are two methods for obtaining a usable display:  using the Last Known Good configuration in Windows NT, or restarting the system in VGA mode.

If you select a resolution that causes the monitor to display incorrectly, do not press ctrl+alt+del to log on to the Windows NT operating system; this will overwrite your Last Known Good configuration.  Instead, use the Last Known Good option to correct the display problem.

ä To use the Last Known Good configuration option:

1.	Restart the workstation.

2.	Select the Last Known Good configuration in Windows NT by pressing the space bar at the following prompt:

Press space bar NOW to invoke the Last Known Good Menu

If the above procedure fails to correct the display, complete the following instructions.

ä To use the VGA mode option:

1.	Restart the workstation.

2.	At the initial OS Loader V3.51 boot screen of Windows NT, select the following option:

Windows NT Workstation Version 3.51 [VGA mode]

3.	Open Video Config in Control Panel.

4.	Configure the video display attributes to correct the problem.  Select the Help button or use context sensitive help for information about each option.

5.	To reset the default display driver, refer to “Changing the Default Display Driver” in Chapter 1.

### Determining a Defective Unit

To determine if you have defective hardware, follow these steps.

ä To determine a defective unit:

1.	Save and exit from all files, if possible.

2.	Shut down Windows NT, if possible.

3.	Turn off the power to the monitor and to the workstation.

CAUTION

Always turn off the power to the workstation before connecting or disconnecting the cables.

4.	Check and reseat all board and cable connections as needed.

5.	Turn on the power to the monitor.  If the monitor Power On LED does not illuminate, return the monitor.

After a period of inactivity, the monitor enters a power saving mode.  On Intergraph InterVue Multiple Sync monitors, a yellow LED (on the Deflection/LVPS board) will light to indicate the power saving mode is active.  The yellow LED is visible by looking into the back of the monitor on the top, left-hand side.  On Intergraph Multi-sync monitors, the Power On LED on the front of the monitor will change from green to yellow to indicate the power saving mode is active.

6.	Turn on the power to the workstation.  If the Power On LED on the workstation does not illuminate or if the BIOS beep codes indicate a failure, return the workstation.

7.	If the monitor display is distorted, ensure the software setup is correct for the monitor type.

If the software setup is correct, use a similar type monitor (if available and functioning properly) to determine if the workstation is defective.