Contents

Preface

Using the Document

Defining Document Conventions

FCC Statement

Safety Information

Cautions

Warnings

Getting Telephone Support

Chapter 1  Getting Started	1

Starting Windows NT	2

Configuring the Video Display	3

Dual-Screen Behavior	4

Gamma Correction	5

Ending the Video Configuration Session	6

Changing the Default Display Driver	7

Turning Off Full Drag	7

Using Software Cursor with OpenGL Applications	8

Chapter 2  Maintenance and Installation	9

Static Precautions	9

Opening the Deskside Workstation	10

Replacing GLZ4 Boards in the Deskside	13

Installing a Dual Screen Option in the Deskside	16

Completing the Installation for the Deskside	19

Chapter 3  Driver Installation and Troubleshooting	21

GL Display Driver Installation	21

Troubleshooting	22

Diagnostics	22

Video Display	23

Obtaining a Usable Video Resolution	23

Determining a Defective Unit	24

Chapter 4  Technical Description	27

Features	27

Geometry Accelerator	28

Rasterization Accelerator/Frame Buffer	29

Functions	32

Geometry Accelerator	33

Rasterization Accelerator/Frame Buffer	34

Chapter 5  Hardware Description	37

Geometry Accelerator Components	37

PCIDMA	37

I/O FIFO	38

DSPs	38

Sequence Controller	38

Rasterization Accelerator Components	39

PCIDMA	39

Graphics Engine	39

Resolver	41

Digital to Analog Converter	41

Frame Buffer Components	42

Video Memory	42

Video Selector and Mapper	42

Video Graphics Adapter Controller	43

Chapter 6  Specifications	45

Drawing Performance	45

3D Performance	45

2D Performance	46

Monitor Resolutions	46

Interfaces	47

Video Port	48

Stereo Port	49

Index	51

Figures

2-1.  	Deskside Top Cover Removal	10

2-2.  	Deskside Left Side Panel Removal	11

2-3.  	Deskside PCI Access Panel Removal	12

2-4.  	Disconnecting the FIFO Status Cable	13

2-5.  	Disconnecting the Clock Sync Cable	14

2-6.  	Deskside PCI Slots	16

2-7.  	Disconnecting the FIFO Status Cable	17

2-8. 	GLZ4 Cable Installation	18

2-9.  	Single Monitor Cable Connections	19

2-10.  Dual Monitor Cable Connections	20

4-1.  	GLZ4 OpenGL Processing Block Diagram	32

4-2.  	Geometry Accelerator Block Diagram	33

4-3.  	Rasterization Accelerator/Frame Buffer Block Diagram	35

5-1.  	Geometry Accelerator Board	37

5-2.  	Rasterization Accelerator Board	39

5-3.  	Frame Buffer Board	42

6-1.  	Video Port	48

6-2.  	Stereo Port	49

Tables

1-1.  	GLZ4 Video Configuration Options	3

1-2.  	Gamma Correction Options	5

3-1.  	Video Configuration Problems	23

6-1.  	3D Graphics Performance	45

6-2.  	2D Graphics Performance	46

6-3.  	Intergraph Multiple Sync Monitor Resolutions	46

6-4.  	Standard Multi-sync Monitor Resolution and Refresh Rate	47

6-5.  	Video Port Pinouts	48

6-6.  	Stereo Port Pinouts	49

# Preface

The GLZ4 Hardware User’s Guide provides instructions for replacing the GLZ4 graphics option in Intergraph’s 3D graphics workstations and upgrading the deskside workstation from a single monitor to dual monitors.  GLZ4 is a complete 3D graphics subsystem including geometry acceleration and rendering acceleration.  This guide provides hardware and driver installation procedures, troubleshooting information, technical and hardware descriptions, and specifications.

## Using the Document

Chapter 1 introduces the GLZ4 graphics subsystem.  It also provides instructions for starting Windows NT, configuring the video display, changing the default display driver, turning off full drag, and using software cursors.

Chapter 2 gives instructions for replacing the GLZ4 graphics subsystem.  It also contains a packing list and instructions for installing a dual screen option in the deskside workstation.

Chapter 3 gives instructions for installing the GL display driver and includes troubleshooting procedures.

Chapter 4 covers the technical features and functions of the GLZ4 graphics subsystem.

Chapter 5 contains a hardware description of the GLZ4 graphics subsystem.

Chapter 6 lists the GLZ4 graphics subsystem specifications.

## Defining Document Conventions

FCC Statement

This document uses the following conventions:

Bold	Commands, words or characters that you type literally.

Italic	Cross-references; variables that you supply.

Monospace	Output that displays on the screen.

small caps	Type names on the keyboard, such as d, alt or f3.

UPPERCASE	Names of files and directories.  Type filenames and directory names in the dialog boxes or the command line in uppercase unless directed otherwise.

alt, shift, f	Press keys sequentially (press alt, then shift, then f).

C:\>	Command line prompt, indicating the character-based interface to the operating system.

ctrl+d	Press a key while simultaneously pressing another key; for example, ctrl and d.

Click	Press and immediately release a mouse button.

Double-click	Press and release a mouse button twice in succession.

Drag	Press and hold down a mouse button while moving the mouse, and then release the button.

Press 	Press and hold a mouse button, or press and release a key.

Select	Make a choice using an available selection method.  For example, place the screen cursor on an object and click a mouse button, or press tab and then enter.

Open	Use an available selection method to open a window, dialog, or folder.

Key in	Type input at the keyboard and press enter.

This equipment has been tested and found to comply with the limits for a Class A digital device, pursuant to part 15 of the FCC Rules.  These limits are designed to provide reasonable protection against harmful interference when the equipment is operated in a commercial environment.  This equipment generates, uses, and can radiate radio frequency energy.  If the equipment is not installed and used in accordance with the instruction manual, it may cause harmful interference to radio communications.  Operation of this equipment in a residential area is likely to cause harmful interference in which case the user will be required to correct the interference at his own expense.

This digital apparatus does not exceed the Class A limits for radio noise emissions from digital apparatus set out in the Radio Interference Regulations of the Canadian Department of Communications.

## Safety Information

This section provides the cautions and warnings for the equipment.

### Cautions

Changes or modifications made to the system that are not approved by the party responsible for compliance could void the user's authority to operate the equipment.

THIS PRODUCT CONFORMS TO THE APPLICABLE REQUIREMENTS OF 21 CFR SUBCHAPTER J AT DATE OF MANUFACTURE.

Read all safety and operating instructions before using the equipment.  Keep these instructions for future reference.  All warnings on the equipment or in the operating instructions should be followed.

### Warnings

To reduce the risk of electrical shock, do not attempt to open the equipment unless instructed.  Do not use a tool for purposes other than instructed.

## Getting Telephone Support

If you experience problems with your Intergraph workstation or software, or have questions about the information in this document, you can contact Intergraph for help.

- In the United States, call the Customer Response Center at 1-800-633-7248 between the hours of 7:00 a.m. and 7:00 p.m. CST, Monday through Friday (except holidays).

- At international locations, call the Intergraph subsidiary or distributor from which you purchased your Intergraph workstation or software.

- Have the following information available when you call:

-	The product’s name and serial number

-	Your name and telephone number

-	A brief description of the problem