# Hardware Description

The following describes the GLZ4 graphics subsystem and discusses the architecture and hardware implementation.

## Geometry Accelerator Components

This section describes the major components of the Geometry Accelerator board.

4699

PCIDMASequenceDSPs

Controller

I/O FIFO            DSPs

Figure 5-1.  Geometry Accelerator Board

### PCIDMA

The Geometry Accelerator employs the PCIDMA Application Specific Integrated Circuit (ASIC) as the Peripheral Component Interconnect (PCI) bus interface device.  This device has two Direct Memory Access (DMA) engines to accelerate the transfer of OpenGL data between the host processor and the Geometry Accelerator, and between the Geometry Accelerator and the Rasterization Accelerator.

The two independent DMA engines operate concurrently to transfer data in sixteen, 32-bit word bursts into and out of the I/O FIFO on the Geometry Accelerator.  These DMA engines free the host processor from the task of transferring massive amounts of graphics data across the PCI bus.

### I/O FIFO

The I/O FIFO is a temporary depository for data being moved via DMA into and out of the Geometry Accelerator.  This device can hold up to 4 KB of data.  This FIFO avoids stall conditions, allowing DMA transfers to proceed at the full speed of the PCI bus or the I/O bus.

### DSPs

The core of the Geometry Accelerator is four state of the art Digital Signal Processors (DSPs) operating in parallel.  These are 32-bit floating point processors, each capable of executing 100 Million FLoating point Operations per Second (MFLOPS).  Each DSP contains 256 KB of high speed Random Access Memory (RAM).  Each DSP also has an advanced I/O subsystem with six communication ports and ten DMA engines.  This I/O subsystem allows continual flow of data through the multiple DSPs and to the Sequence Controller with minimal host processor overhead.

### Sequence Controller

The Multiple Instruction Multiple Data (MIMD) architecture of the Geometry accelerator allows incoming data to be processed concurrently and independently by each of the four DSPs.  This means that the DSP that processes the first input request may not generate the first output request.  Data moves through the DSPs at different rates, depending upon the complexity of the request.

Since the results of some drawing operations are dependent upon the order in which they execute, this out of order data must be placed back into the order that it was received.  This task is the responsibility of the Sequence Controller.  The Sequence Controller receives data from each of the four DSPs via four communication ports.  The Sequence Controller time-orders the data, and then uses a DMA to write the data to the output side of the I/O FIFO.  The Sequence Controller also handles some clipping, and controls the flow of data to multiple Rasterization Accelerators for dual screen systems.

## Rasterization Accelerator Components

This section describes the major components of the Rasterization Accelerator board.

4687

Digital to Analog Converter

Video Port (to monitor)

Resolver

Stereo Port

PCIDMA

Graphics Engine

Figure 5-2.  Rasterization Accelerator Board

### PCIDMA

The PCIDMA ASIC is a multichannel, DMA engine capable of burst transfers of sixteen, 32-bit words.  The DMA transfer can be up to 4 MB.  For OpenGL requests, the Rasterization Accelerator uses a DMA to send vertex data through the PCIDMA to the graphics engine FIFO.

### Graphics Engine

The graphics engine ASIC is the main graphics processor for the board and the core of the Rasterization Accelerator.  It takes vertex data from the graphics FIFO and produces spans of pixel data.  The graphics engine ASIC sends the data to the frame buffer over a high speed pixel path called the Image/Z (IZ) bus.

The graphics engine uses four major logic blocks to perform the pixel rasterization.  They are:  (1) request handler, (2) slope calculation, (3) span generation, and (4) pixel interpolation.

Request Handler -- The request handler monitors the data output of the graphics FIFO.  When data arrives, the request handler analyzes it and writes it to an appropriate processing block.  The request handler routes vertex data for triangles and lines to the slope calculation block.

Drawing requests (other than vertex data for triangles and lines) such as blits, fills, and bit expands, bypass the slope calculation and span generation blocks.  The request handler routes this data directly to the pixel interpolation block.

Slope Calculation -- The slope calculation block performs the slope calculation part of the rasterization process for triangles and lines.  Floating-point math ensures the accuracy of triangle edges.

The slope calculation block calculates color derivatives, depth derivatives, and edge slopes.  It also adjusts the vertex data to a pixel center for the topmost vertex.  The slope calculation block converts all data to a fixed-point format and routes it to the span generation block.

Span Generation -- The span generation block performs the span calculation part of the rasterization process for triangles and lines.  The span generation block determines the leftmost x and y coordinates for each scanline in a triangle.  It also calculates the initial color and depth value for each scanline, and routes the data to the pixel interpolation block.

Pixel Interpolation -- The pixel interpolation block is common for all graphics drawing requests.  For triangle meshes, the pixel interpolation block expands the data from the span generation block to pixel data by successively adding the x-derivative value to the initial color and depth values (z) for each span.

The pixel interpolation block also maps the (x, y) address to a linear address before sending the address and pixel data to the frame buffer over the IZ bus.  The IZ bus is a 64-bit, 256 MB per second bus with a pixel span protocol to facilitate high data throughput.  This protocol allows burst transfers of pixel data to occur between the Rasterization Accelerator and the Frame Buffer.  The maximum burst size is 4096 pixels.

### Resolver

Four resolver ASICs perform the memory control function for reading and writing data to the Video Memory.  This includes coordinating the movement of data from the Video Memory to the Video Selector and Mapper (VSM) ASIC and DAC.

The graphics engine ASIC sends pixel data over the IZ bus to the resolver ASIC's IZ input FIFO.  The resolver IZ input FIFO is 128 words deep so that the resolver can accommodate bursts of pixel data without slowing overall system performance.  The pixel data consists of color (Red, Green, Blue, and Alpha - RGBA) and depth (Z) data.

The resolver examines pixel data from the input FIFO and determines if it should write the pixel data to the Video Memory.  The resolver performs Z, mask, and alpha tests to determine if data should be written.

The resolver can perform an Arithmetic and Logic Unit (ALU) operation in combination with the source data (data from the graphics engine) and the destination data (data in Video Memory).  The operations include logical operations, Porter and Duff style alpha blend operations, and OpenGL style alpha blend operations.

The resolver performs several different types of VRAM write cycles including reads, writes, or read-modify-writes.  It maximizes memory bandwidth by choosing an appropriate type.  The resolver performs block write cycles to maximize performance for window clear and fill requests.

### Digital to Analog Converter

The Digital to Analog Converter (DAC) converts pixel data from the VSM into an analog voltage and sends it to the monitor for display.  The integrated DAC contains a Video Lookup Table (VLT) consisting of 256 locations x 3 colors (RGB) x 10 bits per color used for gamma correction.  The DAC also provides a user-definable 64 x 64 pixel hardware cursor.

## Frame Buffer Components

This section describes the major components of the Frame Buffer board.

Video Selector and Mappers

Video Graphics

Adapter Controller

Video Memory

fs4124a

Figure 5-3.  Frame Buffer Board

### Video Memory

The core of the frame buffer is 24 MB of Video Memory.  The four resolver ASICs control the flow of data to and from the Video Memory.  These resolvers provide a 128-bit, Video Memory interface.  This wide interface allows GLZ4 to achieve a very high Video Memory access rate.  The Video Memory comprises a total of 96 VRAMs, each VRAM consisting of 2 megabits of memory.  Each Video memory pixel consists of 92 bits of memory.

### Video Selector and Mapper

The display subsystem consists of the Video Selector and Mapper (VSM) ASIC and the DAC.  The VSM ASIC receives pixel data from the Video Memory.  It selects the buffer (front or back), and the data (image or overlay) to display.  The video selection planes select one of the VSM’s eight Video Lookup Tables (VLTs) to map the image data.  The VSM sends the mapped result to the DAC.

### Video Graphics Adapter Controller

The VGA controller provides a source of compatible VGA graphics control for the monitor.  Typically the VGA controller is used during the boot process and in full screen DOS mode.  The user can select to display either VGA or GLZ4 graphics.