# Maintenance and Installation

This chapter describes opening Intergraph’s desktop and deskside 3D graphics workstations to replace a GLZ5 graphics subsystem board.  It also describes installing a dual screen option in the deskside workstation.

## Static Precautions

Observe the following precautions when working with the internal components of the system.

- Avoid servicing the system in areas with electrostatic problems.  Rugs and carpets, for example, cause static build-up.

- Handle all printed circuit boards as little as possible and by the edges only.  Leave the boards in their protective packaging until you install them.

CAUTION

Discharge personal static charge by touching a bare metal surface on the workstation before handling printed circuit boards.

## Opening the Desktop Workstation

WARNING

Before opening the workstation, turn off the system power.  Leave the power cord connected to maintain ground.  Use caution when removing the cover to avoid injury.

ä To open the desktop workstation:

1.	Pull out the spring-lock plunger on the back panel and turn it clockwise until it locks in the open position.

4715

ISA I/O

Panel

Spring-lock

Plunger

Figure 2-1.  Desktop Top Cover Removal

2.	Lift the spring-lock plunger up and remove the top cover.

3.	Attach an antistatic wrist strap to a non-painted, metallic surface on the workstation or the ground loop (formed in the metal) above the ISA I/O panel.  Ensure that the elastic loop in the strap snugly contacts your bare wrist.

## Replacing GLZ5 Boards in the Desktop

This section describes replacing either a Geometry Accelerator board or a graphics board set in a desktop workstation.  The graphics board set consists of two boards (Rasterization Accelerator/Frame Buffer and Texturing Accelerator) permanently joined together as one unit.

ä To replace a GLZ5 board in the desktop workstation:

1.	If replacing the graphics board set, disconnect the cables from the Video and VGA ports on the back panel of the base unit.

2.	Disconnect the FIFO status cable as shown in the following figure.

4703

FIFO

StatusGraphics

CableBoard Set

Geometry

Accelerator

Board

Figure 2-2.  FIFO Status Cable Removal

3.	Using a quarter-inch nutdriver, remove the screws from the Geometry Accelerator board or the graphics board set.  Retain the screws.

4.	Remove the Geometry Accelerator board or the graphics board set.

5.	Install the new Geometry Accelerator board or the graphics board set into the slot from which the previously installed board was removed.

Note:	The Geometry Accelerator board resides in PCI slot 1.  The graphics board set resides in PCI slots 2 and 3.

4707

Slot 1

Slot 2

ScrewGeometry

Accelerator Board  Graphics Board Set

Figure 2-3.  GLZ5 Board Installation

6.	Install the screw previously removed to secure the board to the workstation.

7.	Install the FIFO status cable between the Geometry Accelerator board and the graphics board set.

Note:	The connector of the FIFO status cable can be installed in any orientation.

## Completing the Installation for the Desktop

Follow these steps to reassemble the workstation and reconnect the monitor.

ä To complete the installation:

1.	Replace the top cover on the workstation by aligning the tabs on the front of the top cover with the notches in the front of the base unit.

2.	Rotate the spring-lock plunger counter clockwise until it snaps back into its closed position.

3.	Connect the monitor cable to the Video Out port and connect the VGA loopback cable to the VGA In port and the VGA Out port.

VGA In PortVGA Out

Port

4684d

Monitor Cable

VGA Loopback Cable

Video Out Port

Figure 2-4.  Monitor Cable and VGA Loopback Cable Connection

4.	If you need to configure the display driver, refer to “Configuring the Video Display” in Chapter 1.

## Opening the Deskside Workstation

WARNING

Before opening the workstation, turn off the system power.  Leave the power cord connected to maintain ground.  Use caution when removing the cover to avoid injury.

ä To open the deskside workstation:

1.	Using a quarter-inch nutdriver, remove the screw from the top cover.  Retain the screw.

fs4136b

Screw

Figure 2-5.  Deskside Top Cover Removal

2.	Remove the top cover by sliding it toward the back of the workstation.

3.	Remove the footstands.

4.	Remove the two screws from the top of the left side panel.

4636

Screws

Figure 2-6.  Deskside Left Side Panel Removal

5.	Remove the left side panel by grasping the tab between the screw holes.  Then, slide the panel up and out from the workstation.

6.	Lay the workstation on its right side.

7.	Remove the three screws from the PCI access panel on the bottom of the workstation.

4696

Screw

PCI Access Panel

Screws

Figure 2-7.  Deskside PCI Access Panel Removal

8.	Remove the PCI access panel by sliding it toward the back of the workstation.

## Replacing GLZ5 Boards in the Deskside

This section describes replacing either a Geometry Accelerator board or a graphics board set in a deskside workstation.  The graphics board set consists of two boards (Rasterization Accelerator/Frame Buffer and Texturing Accelerator) permanently joined together as one unit.

ä To replace a GLZ5 board in the deskside workstation:

1.	If replacing a graphics board set, disconnect the cables from the Video and VGA ports on the back panel of the base unit.

2.	Disconnect the FIFO status cable.  If replacing a secondary graphics board set in a dual screen configuration, disconnect the clock sync cable.

4704

Geometry Accelerator Board

FIFOSecondary

Status Graphics

CablesBoard Set

Clock Sync Cable

Figure 2-8.  FIFO Status Cable and Clock Sync Cable Installation

3.	Using a quarter-inch nutdriver, remove the screws from the Geometry Accelerator board or the graphics board set.  Retain the screws.

4.	Remove the Geometry Accelerator board or the graphics board set from the workstation.

5.	Install the new Geometry Accelerator board or the graphics board set into the slot from which the previously installed board was removed.

Note:	Refer to Figure 2-9 for PCI slot location.  The Geometry Accelerator resides in PCI slot 1.  The primary graphics board set resides in PCI slots 2 and 3.  In a dual screen configuration, the secondary graphics board set should reside in PCI slots 4 and 5.

6.	Secure the board to the base unit using the screws removed previously.

7.	Install the FIFO status cable between the graphics board set and the Geometry Accelerator board.

Note:	The connectors of the FIFO status cable can be installed in any orientation.

8.	If you replaced a graphics board set in a dual screen configuration, install the clock sync cable.

CAUTION

The clock sync cable installs only one way.  Do not force the keyed connector.  Ensure pin 1 of both cable connectors engages pin 1 on both board connectors.  The black wire connects to pin 1.

## Installing a Dual Screen Option in the Deskside

The Intergraph GLZ5 dual screen option kit provides the following components to upgrade from a single screen to a dual screen configuration.  Verify that you have the following items.

- GLZ5 graphics board set

- Diskette containing the Intergraph GL display driver software

- GLZ5 Hardware User’s Guide

- FIFO status cable

- Clock sync cable

- Antistatic wrist strap

If an item is missing or damaged, notify Intergraph Customer Response Center at 1-800-633-7248 immediately.

ä To install a dual screen option:

1.	Using a quarter-inch nutdriver, remove the blanking plates from PCI slots 4 and 5.  Refer to Figure 2-9 for slot locations.  Retain the screws.

2.	Install the second graphics board set into PCI slots 4 and 5.

4632

PCI Slot 1

PCI Slot 2

PCI Slot 3

PCI Slot 6

PCI Slot 5

PCI Slot 4

Figure 2-9.  Deskside PCI Slots

3.	Install the screws to secure the graphics board set to the workstation.

4.	Install the clock sync cable between the two graphics board sets.  Refer to Figure 2-10.

CAUTION

The clock sync cable installs only one way.  Do not force the keyed connector.  Ensure pin 1 of both cable connectors engages pin 1 on both board connectors.  The black wire connects to pin 1.

5.	Install the FIFO status cable between the secondary graphics board set and the Geometry Accelerator board.  Refer to Figure 2-10.

Note:	The connector of the FIFO status cable can be installed in any orientation.

4704

Geometry Accelerator Board

FIFOSecondary

Status Graphics

CablesBoard Set

Clock Sync Cable

Figure 2-10.  FIFO Status Cable and Clock Sync Cable Installation

## Completing the Installation for the Deskside

Follow these steps to reassemble the workstation and reconnect the monitor.

ä To complete the installation:

1.	Install the PCI access panel on the workstation.

2.	Raise the workstation into its normal standing position.

3.	Install the side panel of the workstation.

4.	Install the footstands.

CAUTION

To avoid injury or damage to the equipment, replace the stabilizing footstands before operating the unit.

5.	Install the top cover of the workstation.

6.	Connect the VGA Loopback cable and monitor cable for single monitor configurations as follows:

VGA Loopback cable connection:  Connect the VGA Loopback cable from the VGA Out port to the VGA In port as shown in Figure
2-11.

Single monitor cable connection:  Connect the monitor cable to the Video Out port in PCI slot 2.

4709B

Video

Out Port

VGA In

Port

VGA Loopback

Cable

VGA Out PortPCI Slot 2

Figure 2-11.  Single Monitor Cable Connections

7.	Connect the VGA Loopback cable and monitor cables for dual monitor configurations as follows:

VGA Loopback cable connection:  Connect the VGA Loopback cable from the VGA Out port to the VGA In port as shown in Figure
2-12.

Primary left/top monitor:  Connect the monitor cable to the Video Out port in PCI slot 2.

Secondary right/bottom monitor:  Connect the monitor cable to the Video Out port in PCI slot 4.

4729

VGA Out PortPCI Slot 4PCI Slot 2

Video

Out Port

(Secondary)

Video

Out Port

(Primary)

VGA In

Port

VGA Loopback

Cable

Figure 2-12.  Dual Monitor Cable Connections

8.	If you need to configure the GL display driver, refer to “Configuring the Video Display” in Chapter 1.