# Specifications

This chapter defines the specifications for the GLZ6.  These specifications are subject to change without notice.

## Power

The following power specifications apply to the GLZ6 graphics tower.

Power supply:  430W, manual ranging

Power rating:  90-132/180-240 VAC, 47/63 Hz, 12A

Power consumption, single monitor:

120 VAC (1.615 A, 137.28 W, 468.71 BTU/hour)

240 VAC (0.847 A, 132.22 W, 451.44 BTU/hour)

Power consumption, dual monitors:

120 VAC (2.30 A, 201.41 W, 687.61 BTU/hour)

240 VAC (1.18 A, 195.80 W, 668.08 BTU/hour)

## Drawing Performance

This section summarizes the GLZ6 hardware performance for some commonly used graphics operations in 3D and 2D operation.  Actual system performance depends on the system processor speed and the software application being used.

### 3D Performance

The performance values of 3D drawing requests are obtained from randomly oriented data with Z-buffering enabled.  Refer to Tables 4-1 and 4-2.

Table 4-1.  3D Drawing Performance

| 3D Requests | Performance |
| --- | --- |
| Triangle, 50 pixel, Gouraud shaded | 450 K Triangles/sec |
| Triangle, 25 pixel, Gouraud shaded | 515 K Triangles/sec |
| Vector, 10 pixel, solid color | 1.15 M Vectors/sec |
| Vector, 10 pixel, Gouraud shaded | 1.00 M Vectors/sec |
| Vector, 10 pixel, antialiased | 550 K Vectors/sec |

Table 4-2.  3D Texturing Performance

| Textured Areas | Performance |
| --- | --- |
| 512 x 512, trilinear interpolated, MIP-mapped | 15 megatexels/sec |

### 2D Performance

The performance values of 2D drawing requests for the Graphical User Interface (GUI) are obtained with randomly oriented data.

Table 4-3.  2D Drawing Performance

| 2D Requests | Performance |
| --- | --- |
| Vector, 10 pixel, solid color | 1.45 M Vectors/sec |
| Vector, 10 pixel, antialiased | 550 K Vectors/sec |
| Blit, screen-to-screen, 64 Bit | 26 M Pixels/sec |
| Fills, 1280 x 1024 | 400 M Pixels/sec |
| Text, 9 x 13 | 240 K Chars/sec |

## Monitor Resolutions

The GLZ6 supports Intergraph’s InterVue 27 and standard multi-sync monitors.  Table 4-4 defines the supported monitors and the appropriate resolution with the maximum refresh rate available. 16.8 million colors are supported at each of the stated resolutions.

Table 4-4.  Supported Monitors

| Monitor | Resolution | Maximum Refresh Rate | Stereo Supported |
| --- | --- | --- | --- |
| InterVue-27 | 640 x 480 | 186 Hz |  |
|  | 800 x 600 | 153 Hz |  |
|  | 1024 x 768 | 122 Hz |  |
|  | 1280 x 960 | 100 Hz |  |
|  | 1280 x 1024 | 94 Hz | Yes |
|  | 1600 x 1200 | 81 Hz | Yes |
|  | 1600 x 1280 | 76 Hz | Yes |
| Multi-sync | 640 x 480 | 85 Hz | Yes |
|  | 800 x 600 | 85 Hz | Yes |
|  | 1024 x 768 | 85 Hz | Yes |
|  | 1152 x 864 | 85 Hz |  |
|  | 1280 x 960 | 75 Hz |  |
|  | 1280 x 1024 | 75 Hz | Yes |
|  | 1600 x 1200 | 64 Hz | Yes |
|  | 1600 x 1280 | 64 Hz | Yes |

For further information about monitor resolutions, stereo, and the GL display driver, refer to the README.TXT file on the diskette containing the GL display driver software.