Performance Discussion

Copyright

Copyright 1997 by VideoLogic Limited. All rights reserved. No part of this publication may be copied or distributed, transmitted, transcribed, stored in a retrieval system, or translated into any human or computer language, in any form or by any means, electronic, mechanical, magnetic, manual or otherwise, or disclosed to third parties without the express written permission of VideoLogic Limited.

The trademarks and any copyright or other intellectual property rights of whatever nature that subsist or may subsist in the Software Package (including but not limited to all programs, compilation of command words and other syntax contained therein) are and shall remain the property of VideoLogic Limited absolutely.

Disclaimer

VideoLogic Limited makes no representation or warranties with respect to the content of this document and specifically disclaims any implied warranties of merchantability or fitness for any particular purpose. Further, VideoLogic Limited reserves the right to revise this publication and to make changes in it from time to time without obligation of VideoLogic Limited to notify any person or organisation of such revision or changes.

Trademarks

IBM, PC and OS/2 are registered trademarks of International Business Machines, Inc. Microsoft is a registered trademark, Windows, Direct3D, DirectDraw, Microsoft Write and WordPad are trademarks of Microsoft Corporation. VESA is a trademark of the Video Electronics Standards Committee. AutoCAD, 3D Studio, MicroStation, Cinepak, Indeo and Video 1 are all trademarks of their respective companies. VideoLogic, the VideoLogic logo and PowerVR are trademarks of VideoLogic Limited. All other product names are trademarks of their respective companies.

# Contents

1. Technical Background

1.1. Introduction

1.2. Polygon Throughput/Fill Rate Inter-relationship

2. Facts About Conventional 3D and PowerVR Fill Rates

3. Benchmarking Considerations

3.1. D3Dtest

3.2. D3D General

3.3. Forward and backward facing polygons

3.4. Quads

3.5. MipMapping

# 1	Introduction

The objective of this document is to provide some background on the technical issues regarding 3D benchmarking and how PowerVR differs from other solutions. In addition, the document provides some issues to watch out for if performing benchmarking or developing benchmark programs.

It is suggested that the reader also studies the D3D & SGL  Application notes.

# 2	Technical Background

## 3	Introduction

There are two key parameters that provide some insight into the performance of a 3D system:

- triangle/polygon throughput

- fill rate.

However, these 2 parameters cannot be viewed independently. It is critical to understand the significance and interrelationship of these parameters. In addition it is important that the  dependency of these parameters on other key factors, particularly polygon size, depth complexity and polygon shape are understood.

These issues are first looked at in the context of conventional 3D systems i.e. those using z-buffers and non deferred texturing approaches. Later in this note the issues are extended to PowerVR style approaches.

## 4	Polygon Throughput/Fill Rate Inter-relationship

A solution with a high number for one or the other of these two parameter is not ideal. In fact a system with medium performance for both triangle throughput and fill rate is more desirable than one where one of the parameters has a very high value and the other a low value. Also to make the matter more complicated different games and applications will have vastly differing demands on these parameters.

A good illustration is software only systems (e.g. D3D with no hardware assist) where the basic triangle throughput is very high indeed. In fact, software only triangle throughput (set-up) in D3D is higher than the corresponding rate for any D3D hardware accelerator available today. However triangle set-up throughput does not equates to triangle drawing throughput. Software only solutions can draw very small  triangles  (1 to 4 pixels) fast but they get into real trouble when the triangles get bigger or where a few very large background (sky and ground) triangles have to be drawn as well. This is where fill rate (the ability to actually draw pixels and “fill” triangles) becomes at significant factor.

The best way to see the significance of this is by way of an example - taking the case of a “fast” system that delivers 300K triangles per second.  At 30 FPS, this triangle throughput equates to 10K triangles per frame. If the system can do an average of 10-pixel triangles then its performance equates to 10 x 10K = 100 K drawn pixels. The problem is that a 640x480 (800x600) screen alone, ignoring the depth complexity, is 307K (480K) pixels and 10K pixels of 10 pixels only cover one third (one fifth) of the screen!

Real life scenarios are more complex than this as depth complexity ( defined as the average number of times, per frame, a pixel on the screen is updated) needs to be taken into consideration.  With the early PC 3D games, where no real 3D hardware acceleration was available, the games tended to minimise depth complexity by specific choice of the 3D world (e.g.  underground tunnels) or by polygon pre-sorting. On the other hand arcade and game consoles, because of the available 3D hardware acceleration, have been offering 3D games with much less restricted 3D world (racing and fighting games) and therefore a much higher depth complexity. The early 3D PC games had a depth complexity between 1.3 and 1.8 while many of the modern action 3D games (e.g. arcade type games) tend to have depth complexities in the range of 2 to 4. Also, effects such as translucency effectively add to the depth complexity of a scene and demand even more fill rate. Going back to the above example and considering the case of an advanced game where the camera is constantly moving and has a (typical) depth complexity of 3, the screen has to be, on average, redrawn three times. So for a screen resolution of 640x480 (800x600), the average triangle size for the above example needs to be (3x640x480)/10000 = 92 pixels (in the case of 800x600 resolution this will be 3x800x600/10000= 144 pixels). Relating these to the minimum required fill rate, we get fill rate = 10000x92x30 = 27.7 Million Pixels/s for 640x480 screen and an even higher fill rate of 10000x144x30 = 43.2 Million Pixels/s for the 800x600 resolution case.

i.e. Required Fill Rate = Width x Height x FPS x Average Depth Complexity.

In the extreme case of 1027x768, which will become important in the future, the average triangle size  for the above example will be 3x1024x768/10000=236 pixels and the required fill rate will be 70.8 Million Pixels/s.

Looking at the same example at 60 FPS, the number of triangles per frame will  be 300K/60 = 5K, leading to the triangle sizes of 184-pixels (for 640x480 resolution) and 288-pixels (for 800x600 resolution). The associated minimum fill rate will be 5000x184x60 = 55.4 Million Pixels/s for 640x480 resolution and 5000x288x60 = 86.4 Million Pixels/s for the case of 800x600 resolution.

# 5	Facts About Conventional 3D and PowerVR Fill Rates

For conventional 3D systems the Fill Rate and Triangle Throughput are related by a simple equation:-

(Triangle Throughput)x(Triangle Size) = Fill Rate = Pixel Processing Rate

Note that Fill Rate and Pixel Processing Rate are the same in this case. This is important as the situation is different for PowerVR due to deferred texturing approach.

Generally speaking for very small triangles the limiting factor in performance is triangle set-up. The triangle set-up  depends on the extent of hardware set-up assist, CPU performance and software driver speed.

On the other hand the Fill rate is solely dependent on the hardware rendering pipeline and in particular is often constrained by the texturing engine and associated memory fetches (Pixel Processing Rate). For example for bilinear texturing 4 texels have to be fetched to produce one output pixel. For this reason the performance of the texture memory system is a key factor in determining the overall pixel Fill Rate.  To give you a better feel for the issues  and the limits governing conventional 3D systems consider a typical fast memory system clocked at 66MHz. As texturing involves a high degree of random accesses to the memory it is very realistic to assume that  at least 10% of accesses  will involve memory page breaks. Such page misses often take 10 time longer than a “normal”  page hit. This means that the average performance of the memory system will be around 33MHz. With that in mind a conventional system that uses a very dedicated texture memory system and fetches four texels concurrently can deliver up to 33Mpixels/s fill rate. However if the 3D system can not access a wide enough (e.g. 64 bit) and dedicated texture memory every cycle, to fetch texels, then the fill rate will drop correspondingly. For example, if the texture memory is shared with other functional blocks such as z-buffer or frame buffer, the achievable fill rate could easily drop to well below 10  or even 5 Mpixels/s (and it does in many 2d/3d integrated chips).

So as far as conventional systems are concerned a  today’s well designed  and high performance system with “dedicated” DRAM for its texture memory could achieve in the neighbourhood of 30 Mpixel fill rate. Using this rational estimate in the above equation, assuming 100(200)-pixel triangles, results in a maximum achievable triangle  rate of 300K (150K). And if the texture memory is shared, these figures could be substantially less.

For a PowerVR system the basic relationship between Fill Rate and Triangle Throughput is very different. This is because PowerVR employs a deferred texturing approach where only visible triangles have any demand on texturing pipeline. This means that depth complexity also plays a part in this relationship. Specifically the relationship can be expressed (in a simplified form) as:-

(Triangle Throughput)x(Triangle Size)=d x (Pixel Processing Rate)=Effective Fill Rate

where d is the depth complexity.

Note that in the PowerVR case, as the depth is fully sorted prior to any texturing, the Pixel Processing Rate is not wasted for triangles that are later covered by other ones. This is  why the “Effective” Fill Rate (i.e. as used in conventional systems) can achieve a much higher value. A very simple case to illustrate this difference is one where a number of large polygons are sitting in front of each other in z order. In the case of a conventional z-buffered system all these polygons are drawn (on the top of each other) and the Pixel Processing Resources of the hardware are used(wasted). While in the case of PowerVR the polygons are sorted first in hardware and only the visible front polygon is actually drawn.

Again for smaller triangles the limiting factor for Triangle Throughput performance will be triangle set up. The triangle set-up  depends on the extent of hardware assist, CPU performance and software driver speed.

The maximum achievable Fill Rate (assuming  a similar memory system as that of a conventional system) is increased in proportion to depth complexity (i.e. 99 MPixels/s for the example above assuming a depth complexity of 3). This is the fundamental advantage (in addition to cost minimisation) of PowerVR architecture over conventional systems.

The point to bear in mind, however, is that to achieve this huge Fill Rate either the Triangle Throughput (via faster CPU, faster drivers or more hardware set up) or Triangle size (or both) need to be scaled up accordingly.

# 6	Benchmarking Considerations

## 7	D3Dtest - Comparing Like with like

When comparing the result to 3DFX you should really use the “No Update” option. The reason being that the current D3Dtest is a windowed application; The current 3DFX solution cannot support windowed 3D uses a separate screen with the image in the middle without any of 2D windows. This allows the more efficient frame buffer switching (Flipping) to be implemented, where renders occur in a double buffered fashion the buffers switch or flipped after each render. Solutions that  support the windowed case (as for PowerVR PCX1) do actually blit the rendered image from an off screen buffer to on screen (in PowerVR’s case the 2D controller does this blit). This blit is only happening because of the windowed nature of the application and is not representative of a real game which will be full screen. In such a case PowerVR can use double buffered full screen images and simply switch between them rather than using a 2D blit by the 2D controller. NOTE:- even when you enable “No Update” mode,  although you do not see a rendered image, PowerVR is indeed rendering the image over PCI and into the 2D controller frame buffer. You should see an  increase in triangle rate and fill rate when you use “No Update”. The reason is that the fill rate test renders a large screen area and the 2D blit costs more.

## 8	D3D General

When running D3D make sure the display mode is set to 60Hz and 640x480x16bit, then you are comparing like with like. Note:-  for full screen apps, D3D locks the render rate to refresh rate. So with 60Hz refresh the steps are 60, 30, 20, 15, (60Hz is 16.6 ms and all these rates have a frame time which is multiple of this). And with a 75Hz refresh (13.3 ms) the steps are 75, 37.5, 25, 18.7 etc and for a 100Hz refresh the steps are 100, 50, 33, 25, etc. So if you get anything other than 60 or 30 fps then you are NOT running at 60 Hz refresh rate. Imagine how much of the 2D controller bandwidth is taken up by screen updates when the refresh rate is 100Hz rather than 60. So watch out.

The 2D card will impact the performance (DRAM bandwidth) which gets worse as the resolution or refresh rate is increased. For best result use a graphics card with a higher performance video memory (VRAM, MDRAM).

## 9	Forward and backward facing polygons

Many benchmarks and all real games have backward and forward facing polygons. For each frame the transformations has to be done on ALL polygons and then depending on the 3D hardware backward facing polygons may be processed differently. Some 3D drivers eliminate the backward facing triangles while some send both forward facing and backward facing triangles to the hardware.  The reason for this could be that, depending on the hardware set up and the rendering pipeline, it may be more effective to do so than have the driver do the maths and the decision making. For PowerVR PCX1 the driver  eliminates the backward facing polygons. This is done very early in the maths and as such they cost very little. For these reasons it is often important that any Benchmarking effort also measures the overhead associated with backward facing triangles as it will not be the same in all cases, and such triangles do exist in real applications. Obviously tests that use both forward and backward facing triangles get a very different answer compared to those that only use forward facing triangles.

## 10	Quads

Some hardware solutions implement quads. PowerVR can also implement quads  (and in fact polygons with larger number of sides) so if a benchmark allows coplanar triangles to be combined or uses models with quads (each replace two triangles!) much higher performance can be achieved as the associated set-up is reduced and in our case fill rate is almost free.